| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0142r2 |
| --- | --- |

| Title: | SCE5: Buffer Size Estimation of SCE5.2.x |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Corp. / Sony Electronics Inc. |  |  |

_____________________________

# Abstract

On the usage of motion data buffer for SHVC is being studied under SCE5.2.x.

It is proposed by SCE5.2.1 that temporal motion prediction be omitted at the enhancement layer to reduce the required buffer size, as the collocated base layer motion information takes part of TMVP in the enhancement layer.

It is proposed by SCE5.2.2 to postpone motion data compression after encoding/decoding of the enhancement layer, or 2-stage motion data compression for improving coding efficiency.

At the 12th JCT-VC meeting in Geneva, it was suggested to study on the buffer size requirements for SCE5.2.x proposals.

This contribution provides information on the buffer sizes required for each of SCE5.2.x proposals. Trade-off of buffer sizes and coding efficiency is also studied.

# Introduction

Current SHM contains both temporal motion vector and co-located base-layer motion vector as the candidates of Merge. It is proposed by SCE5.2.1 that temporal motion prediction be omitted at the enhancement layer if collocated base-layer motion information is available, to reduce the required buffer size.

Minimum inter-PU size is 4x8 or 8x4 in HEVC version 1, but for temporal motion vector prediction motion information is stored by 16x16-unit to reduce buffer size. In current SHM same decimation ratio is applied for co-located BL motion information. It is proposed by SCE5.2.2 to postpone motion data compression after encoding/decoding of the enhancement layer, or 2-stage motion data compression for improving coding efficiency.

At the 12th JCT-VC meeting in Geneva, it is suggested to study on the buffer size requirements for SCE5.2.x proposals.

# Buffer Size Estimation

In this contribution coding efficiency and required buffer sizes of the following methods are compared:

- SCE5.2.1: Disabling TMVP

- SCE5.2.2 (2:1) : Half BL MV buffer compression

- SCE5.2.2 (1:1) : No BL MV buffer compression

- SCE5.2.1 + SCE5.2.2 (2:1)

- SCE5.2.1 + SCE5.2.2 (1:1)

For each of the motion information, the following data needs to be stored:

- Mvx (16bits)

- Mvy (16bits)

- Refidx (4bits)

So totally 16+16+4 = 36bits will be necessary. If byte-alignment is considered, 40bits will be necessary.

Minimum inter PU size specified in HEVC version 1 is either 4x8 or 8x4. With our implementation motion data for each of 4x4 units is prepared, and in the case that PU size is 4x8, same motion data are stored for 2 consecutive blocks in vertical direction, and in the case that PU size is 8x4, same motion data are stored for 2 consecutive blocks in horizontal direction.

Assuming that

- W: EL Width

- H: EL Height

- R: Scalability Ratio (2, 1.5 or 1)

- D: Decimation Ratio of colblmv (4, 2 or 1)

- N: Number of DPB

Then (bi-directional case)

- EL_pixel_buf_size = W * H * 8[bit] * 1.5 = 12* W*H

- BL_mv_buf_size= (W/R) * (H/R) * (1/ 4D * 1/ 4D) * 40 [bit] * 2 = (5 / (D2R2) ) *W*H

- EL_mv_buf_size= W * H * (1/16 * 1/16) * 40 [bit] * 2 * N = 5/16 *N*W*H

Note that only L0 and L1 motion data of the current frame need to be stored for the base layer, whereas L0 and L1 data for all reference frames need to be stored for the enhancement layer as it may be used for encoding/decoding of the future frames within the same layer.

Assuming that N=6 (bi-directional case)

- EL_mv_buf_size / EL_pixel_buf_size = (5/16) / 12 * 6 = 15.6%

- BL_mv_buf_size / EL_pixel_buf_size = (5 / D2R2) / 12 = 5 / (12*D2*R2)

=0.65% (R=2; D=4)

=1.16% (R=1.5; D=4)

=2.6% (R=1; D=4)

=2.6% (R=2; D=2)

=4.6% (R=1.5; D=2)

=10.4% (R=1; D=2)

=10.4% (R=2; D=1)

=18.5% (R=1.5; D=1)

=41.7% (R=1; D=1)

# Comparisons of Coding Efficiency and Buffer Size

Relations of required buffer sizes and coding efficiency are shown in Fig 3.1 (RA_2x), Fig 3.2 (RA_1.5x), Fig 3.3 (RA_SNR), Fig 3.4 (LD_2x), Fig 3.5 (LD_1.5x) and Fig 3.6 (LD_SNR).

In each of the figures, horizontal axis is the increase/decrease of the buffer size compared to SHM (relative to the EL_pixel_buf_size). Positive value means increase and negative means decrease. Vertical axis is the coding efficiencies of each proposal. Positive means loss and negative means loss.

The method of SCE5.2.2 (1:1) brings around 1% gain for the RA/LD_2x cases, but causes increase in buffer size by around 40% compared to EL_pixel_buffer_size with the RA_SNR case. Similar gain can be obtained with SCE5:2:2 (2:1) with much less increase in buffer size.

The method of SCE5.2.1 can reduce buffer size by 15.6% but causes loss in coding efficiency by 0.8 to 0.9% for RA/LD_2x cases. This loss can be compensated by combining with SCE5.2.2 (2:1) or SCE5.2.2 (1:1). Coding efficiency of both methods are similar while the latter requires more buffer size.

Therefore methods of SCE5.2.2 (2:1) or combination of SCE5.2.1+SCE5.2.2 (2:1) provide good trade-off between coding efficiency and implementation cost.

Fig 3.1 RA_2x

Fig 3.2 RA_1.5x

Fig 3.3 RA_SNR

Fig 3.4 LD_2x

Fig 3.5 LD_1.5x

Fig 3.6 LD_SNR

Assuming, for example, that the resolution of the enhancement layer is 3840x2160 pixels and the bit-depth is 8, increase (positive values) or decrease (negative values) of buffer sizes with each of the proposals [in the unit of bytes] are shown in Table 3.1:

Table 3.1

|  | RA_2x | RA_1.5x | RA_SNR | LD_2x | LD_1.5x | LD_SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 5.2.2 (2:1) | 242611 | 427991 | 970445 | 121306 | 213996 | 485222 |
| 5.2.2 (1:1) | 1213056 | 2157373 | 4864666 | 606528 | 1078687 | 2432333 |
| 5.2.1 | -1940890 | -1940890 | -1940890 | -970445 | -970445 | -970445 |
| 5.2.1+5.2.2(2:1) | -1735603 | -1512899 | -970445 | -867802 | -756449 | -485222 |
| 5.2.1+5.2.2(1:1) | -727834 | 216484 | 2923776 | -363917 | 108242 | 1461888 |

# Additional Remarks

At the 12th JCT-VC meeting in Geneva, concern was raised that required buffer size of the 2-stage motion data compression is less than the postponing of motion data compression but it requires additional operations for compression.

According to a software engineer in Sony overhead of this operation can be ignored, as “motion data compression” operation is just decimation of motion data.

# Conclusion

In this contribution trade-off between buffer size and coding efficiency on the methods related to motion data compression for SHVC is studied. It is shown that the methods of SCE5.2.2 (2:1) or SCE5.2.1+SCE5.2.2 (2:1) provide good trade-off between coding efficiency and implementation cost. 2-stage motion data compression requires additional operation, but this overhead can be ignored, as the process of motion data compression actually is just decimation.

It is recommended to consider adoption of SCE5.2.2 (2:1) or combination of SCE5.2.1+SCE5.2.2 (2:1) into SHVC working draft.

# References

[1] “Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, January 2013

[2] “SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP)”, JCTVC-M0283, April 2013

[3] “SCE5: Result of SCE5.2.2”, JCTVC-M0141, April 2013

[4] “SCE5: Summary report of core experiment on inter-layer syntax prediction using HEVC base layer”, JCTVC-M0025, April 2013

# Patent rights declaration(s)

Sony Corp. / Sony Electronics Inc. does not have any current or pending patent rights relating to the technology described in this contribution.