| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0143r1 |
| --- | --- |

| Title: | Non-SCE3: Quantized GRP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In scalable coding, if the performance of prediction in the base layer is poor, it tends to be also poor in the enhancement layer. To improve coding efficiency of such area residual prediction is an effective tool.

Generalized Residual Prediction (GRP) has been studied in SCE3. GRP can effectively reduce the redundancy between base-layer and enhancement-layer while the complexity of interpolation process and the memory band increased. A proposal on single-loop scalability JCTVC-L0154 also contains prediction with residue from the base layer.

When the input pixel is in 8-bit depth, the residue becomes 9-bit depth. Taking byte-alignment into account, 16-bit depth would be needed to store residual, which causes increase in buffer size.

This document proposes quantized residual prediction to reduce the required buffer size.

# Introduction / Problem Statement / …

In scalable coding, if the performance of prediction in the base layer is poor, it tends to be also poor in the enhancement layer. To improve coding efficiency of such area residual prediction is an effective tool.

Generalized Residual Prediction (GRP) has been studied in SCE3. A proposal on single-loop scalability JCTVC-L0154 also contains prediction with residue from the base layer.

When the input pixel is in 8-bit depth, the residue becomes 9-bit depth. Taking byte-alignment into account, 16-bit depth would be needed to store residual, which causes increase in buffer size.

With HW, encoding/decoding and de-blocking may be implemented based on LCU-level or sub-LCU-level, but with SW it may be based on picture-level. Similarly encoding/decoding of the base layer is followed by the one of the enhancement layer at picture-level. It means that residue signal of the base layer need to be stored at picture-level, which causes increase of the required buffer size.

# Proposed Method

To solve the problem stated in the previous section this document proposes quantized residual prediction: when the residue is stored >>1 operation is conducted, and <<1 operation is conducted when it is extracted from the buffer if the input signal is in 8-bit depth.

# Simulation

## Simulation Condition

SHM-1.0 is used as the anchor. On top of the anchor software the proponent of SCE3.3.6 implemented their proposed method and provided to the author. The author implemented quantized GRP on top of SCE3.3.6 software and its performance in coding efficiency has been compared with the anchor.

The following settings are from the software of JCTVC-L0265:

#define SVC_ENABLE_BEMCP		1

#define SVC_ENABLE_BEMCP_2MODES	(1 && SVC_ENABLE_BEMCP)

#define SIMPLE_MC_FOR_GRILP		1

Additionally the following is the setting related to the proposed method:

#define QUANT_GRP			1

Class A and B sequences have been tested with {RA, LP}_{2x,1.5x,SNR} conditions.

## Simulation Result

Summary of the result is shown in Table 3.2.1. Complete result is shown in JCTVC-M0143.xls respectively. To make comparison result of SCE3.3.6 is shown in Table 3.2.2 [6].

The author would like to thank Sharp for crosschecking [4].

As reference the result of SCE3.3.6 is shown in Table 3.2.2.

Compared with Table 3.2.2, loss of the proposed method is 0.3%, 0.5%, 0.2%, 0.3%, 0.5% and 0.1% with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR cases respectively. Compared with the anchor the proposed method brings gain by -1.8%, -2.8%, -1.6%, -2.2%, -3.2% and -2.1% with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR cases respectively.

Table 3.2.1

Table 3.2.2

# Conclusion

In scalable coding, if the performance of prediction in the base layer is poor, it tends to be also poor in the enhancement layer. To improve coding efficiency of such area residual prediction is an effective tool. However, when the input pixel is in 8-bit depth, the residue becomes 9-bit depth. Taking byte-alignment into account, 16-bit depth would be needed to store residual. So residual prediction will cause increase in buffer size.

To solve this problem this document proposes quantized residual prediction: when the residue is stored >>1 operation is conducted, and <<1 operation is conducted when it is extracted from the buffer if the input signal is in 8-bit depth.

The proposed method has been tested on top of SCE3.3.6. The result shows that compared with SCE3.3.6, loss of the proposed method is 0.3%, 0.5%, 0.2%, 0.3%, 0.5% and 0.1% with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR cases respectively, and compared with SHM-1.0 the proposed method brings gain by -1.8%, -2.8%, -1.6%, -2.2%, -3.2% and -2.1% with RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR cases respectively.

It is recommended that the proposed method be studied under CE.

# References

[1] “Description of Core Experiment SCE 3: Combined Inter and Inter-Layer Prediction in SHVC”, JCTVC-L1103, January 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-L1009, January 2013

[3] “On interpolation filter for Generalized Residual Prediction”, JCTVC-L0265, January 2013

[4] “Cross-check on Non-SCE3: Quantized GRP (JCTVC-M0143)”, JCTVC-M0078, April 2013

[5] “Single-loop SNR scalability using Binary Residual Refinement Coding”, JCTVC-L0154, January 2013

[6] “SCE3: Results of test 3.6 on Generalized Residual Prediction with shorter-tap MC filter”, JCTVC-M0073, April 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).