| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0144r3 |
| --- | --- |

| Title: | Non-SCE5: Combined Tests of SCE5.1.5 and SCE5.2.* |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

SCE5.1.5 proposes to add the bottom-right position of the collocated base-layer motion in addition to the center for motion vector prediction with base layer to improve coding efficiency.

On the usage of motion data buffer for SHVC is being studied under SCE5.2.x.

It is proposed by SCE5.2.1 that temporal motion prediction be omitted at the enhancement layer to reduce the required buffer size, as the collocated base layer motion information takes part of TMVP in the enhancement layer. It is proposed by SCE5.2.2 to postpone motion data compression after encoding/decoding of the enhancement layer, or 2-stage motion data compression for improving coding efficiency.

With SCE5.2.1 loss in coding efficiency would be observed, but it is expected that having an additional candidate like SCE5.1.5 can compensate this loss.

If base layer motion data are compressed not by 4:1 but 2:1 or 1:1, it is more probable that the motion data of the bottom-right position differs from the one of the center position at the base layer. Therefore it is expected that more improvement in coding efficiency can be obtained.

This document examines such interactions between SCE5.1.5 and SCE5.2.*.

# Introduction / Problem Statement / …

SCE5.1.5 proposes to add the bottom-right position of the collocated base-layer motion in addition to the center for motion vector prediction with base layer to improve coding efficiency.

In the current SHM motion data is compressed by 4:1 at the enhancement layer. It is proposed by SCE5.2.2 that it be compressed by 2:1 or 1:1 to improve coding efficiency. With SCE5.2.2 it is more probable that the motion data of the bottom-right position differs from the one of the center position at the base layer. Therefore it is expected that more improvement in coding efficiency can be obtained.

It is proposed by SCE5.2.1 that temporal motion prediction be omitted at the enhancement layer to reduce the required buffer size, as the collocated base layer motion information takes part of TMVP in the enhancement layer. Loss in coding efficiency will be observed only with this proposal, but it is expected that having an additional candidate like SCE5.1.5 can compensate this loss.

Fig 1.1

# Simulation

## Simulation Condition

SHM-1.0 is used as the anchor. On top of the anchor software the proponent of SCE5.1.5 implemented the proposed method and provided to the author for the purpose of crosschecking. Additionally the method proposed by SCE5.2.1 and SCE5.2.2 have been implemented by the author.

The following combinations have been tested:

- SCE515 + ELnoTemp (SCE521)

- SCE515 + SCE522 (No BL MV Compression)

- SCE515 + SCE521 (ELnoTemp) + SCE522 (No BL MV Compression)

- SCE515 + SCE522 (Half BL MV Compression)

- SCE515 + SCE521 (ELnoTemp) + SCE522 (Half BL MV Compression)

The following is the macro in “Typedef.h” implemented by the proponent of SCE5.1.5:

#define IL_MRG_BL_H                      		1

#define IL_MRG_ONLY_CHECK_LEFT_AND_ABOVE 	1

The followings are macro settings in “Typedef.h” implemented by the author:

SCE515 + ELnoTemp (SCE521):

#define EL_noTemp			1

#define SCE522_MOTION_COMP		0

SCE515 + SCE522 (No BL MV Compression)

#define EL_noTemp			0

#define SCE522_MOTION_COMP		1

#define HALF_COMP			0

SCE515 + SCE521 (ELnoTemp) + SCE522 (No BL MV Compression)

#define EL_noTemp			1

#define SCE522_MOTION_COMP		1

#define HALF_COMP			0

SCE515 + SCE522 (Half BL MV Compression)

#define EL_noTemp			0

#define SCE522_MOTION_COMP		1

#define HALF_COMP			1

SCE515 + SCE521 (ELnoTemp) + SCE522 (Half BL MV Compression)

#define EL_noTemp			1

#define SCE522_MOTION_COMP		1

#define HALF_COMP			1

Class A and B sequences have been tested with {RA, LP}_{2x,1.5x,SNR} conditions.

## Simulation Result

Summary of the results of the each test are shown in Table 2.2.1 [SCE515 + ELnoTemp(SCE521)], Table 2.2.2 [SCE515 + SCE522(No BL MV Compression)], Table 2.2.3 [SCE515 + SCE521(ELnoTemp) + SCE522(No BL MV Compression)], Table 2.2.4 [SCE515 + SCE522(Half BL MV Compression)] and Table 2.2.5 [SCE515 + SCE521(ELnoTemp) + SCE522(Half BL MV Compression)] respectively.

Complete results are shown in JCTVC-M0144_ELnoTemp.xls, JCTVC-M0144_MvComp00.xls, JCTVC-M0144_MvComp00_ELnoTemp.xls, JCTVC-M0144_MvComp05.xls and JCTVC-M0144_MvComp05_ELnoTemp.xls, respectively.

As references results with SCE5.1.5 and SCE5.2.1 are shown in Table 2.2.6 and 2.2.7 respectively.

The author would like to thank MediaTek for crosschecking [3].

As reported by [4] loss by 0.8%, 0.3%, 0.4%, 0.9%, 0.6% and 0.5% is observed only with SCE5.2.1 for  RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. As shown in Table 2.2.1, this loss can be compensated by combining with SCE5.1.5.

As reported by [5] gain by -0.6%, -0.2%, -0.6%, -0.4%, -0.2% and -0.5% is obtained with SCE5.1.5 for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. Additional gain is obtained by combining with SCE5.2.2 proposals: by combining with 2:1 compression, gain by -1.3%, -0.6%, -0.9%, -1.0%, -0.5% and -0.9% is obtained for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively; by combining with 1:1 compression, gain by -1.5%, -0.7%, -1.0%, -1.2%, -0.5% and -1.0% is obtained for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. Gain obtained with the former method is close to the one with the latter method while the latter method requires more buffer size [6].

By combining SCE5.1.5 and SCE5.2.1 and SCE5.2.2 (2:1) gain by -0.9%, -0.4%, -0.7%, -0.5%, -0.1% and -0.7% is obtained for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively.

Table 2.2.1 SCE515 + ELnoTemp (SCE521)

Table 2.2.2 SCE515 + SCE522 (No BL MV Compression)

Table 2.2.3 SCE515 + SCE521 (ELnoTemp) + SCE522 (No BL MV Compression)

Table 2.2.4 SCE515 + SCE522 (Half BL MV Compression)

Table 2.2.5 SCE515 + SCE521 (ELnoTemp) + SCE522 (Half BL MV Compression)

Table 2.2.6 SCE5.1.5

Table 2.2.7 SCE5.2.1

# Conclusion

In this contribution results of combined tests of SCE5.1.5 and 5.2.* have been reported.

Loss by 0.8%, 0.3%, 0.4%, 0.9%, 0.6% and 0.5% is observed only with SCE5.2.1 for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. This loss can be compensated by combining with SCE5.1.5.

Gain by -0.6%, -0.2%, -0.6%, -0.4%, -0.2% and -0.5% is obtained with SCE5.1.5 for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. Additional gain is obtained by combining with SCE5.2.2 proposals: by combining with 2:1 compression, gain by -1.3%, -0.6%, -0.9%, -1.0%, -0.5% and -0.9% is obtained for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively; by combining with 1:1 compression, gain by -1.5%, -0.7%, -1.0%, -1.2%, -0.5% and -1.0% is obtained for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively. Gain obtained with the former method is close to the one with the latter method while the latter method requires more buffer size.

By combining SCE5.1.5 and SCE5.2.1 and SCE5.2.2 (2:1) gain by -0.9%, -0.4%, -0.7%, -0.5%, -0.1% and -0.7% is obtained for RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR conditions respectively.

It is recommended that the methods proposed by SCE5.1.5, SCE5.2.1 and SCE5.2.2 (2:1 compression for EL) be considered for adoption.

# References

[1] “Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, January 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-L1009, January 2013

[3] “Non-SCE5: Crosscheck of JCTVC-M0144 on combined tests of SCE5.1.5 and SCE5.2.*”, JCTVC-M0393, April 2013

[4] “SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP)”, JCTVC-M0283, April 2013

[5] “SCE5: Crosscheck of SCE5.1.5”, JCTVC-M0149, April 2013

[6] “SCE5: Buffer Size Estimation of SCE5.2.x”, JCTVC-M0142, April 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).