| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0145r1 |
| --- | --- |

| Title: | SCE3: Crosscheck of SCE3.3.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides crosscheck result of SCE3.3.6.

# Introduction / Problem Statement / …

Generalized Residual Prediction (GRP) has been studied in SCE3. GRP can effectively reduce the redundancy between base-layer and enhancement-layer while the complexity of interpolation process and the memory band increased.

To solve this problem it is proposed by SCE3.3.6 that shorter-tap filter be applied for interpolation of the residue.

# Simulation

## Simulation Condition

SHM-1.0 is used as the anchor. On top of this anchor software the proposed method has been implemented by the proponent. Implementation has been successfully verified by the author. The proposed method is to apply shorter-tap filtering for the upsampling of the residue, and to make comparison test with longer-tap has also been conducted.

Macro settings in “Typedef.h” are as follows:

Shorter-tap filter:

#define SVC_ENABLE_BEMCP		1

#define SVC_ENABLE_BEMCP_2MODES	(1 && SVC_ENABLE_BEMCP)

#define SIMPLE_MC_FOR_GRILP		1

Longer-tap filter:

#define SVC_ENABLE_BEMCP		1

#define SVC_ENABLE_BEMCP_2MODES	(1 && SVC_ENABLE_BEMCP)

#define SIMPLE_MC_FOR_GRILP		0

Class A and B sequences have been tested with {RA, LP}_{2x,1.5x,SNR} conditions.

## Simulation Result

Summary of the result is shown in Table 2.2.1 (Short) and Table 2.2.2 (Long) respectively. Complete result is shown in JCTVC-M0145_short.xls and JCTVC-M0145_long.xls respectively.

With shorter-tap filter gain by -2.1%, -3.3%, -1.8%, -2.5% -3.7% and -2.2% is obtained with each of RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR case. With longer-tap filter gain by -1.7%, -2.4%, -1.5%, -2.4% -3.6% and -2.7% is obtained with each of RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR case.

The result matched the one provided by the proponent.

Table 2.2.1 (Short):

Table 2.2.2 (Long):

The results on complexity assessment provided by the proponent have also been verified. Crosscheck results are shown in JCTVC-M0145_Comp_PSLICE.xls, JCTVC-M0145_Comp_BSLICE.xls and JCTVC-M0145_Comp_long.xls.

# Conclusion

This document reports crosscheck result of SCE3.3.6. With shorter-tap filter gain by -2.1%, -3.3%, -1.8%, -2.5% -3.7% and -2.2% is obtained with each of RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR case. With longer-tap filter gain by -1.7%, -2.4%, -1.5%, -2.4% -3.6% and -2.7% is obtained with each of RA_2x, RA_1.5x, RA_SNR, LD_2x, LD_1.5x and LD_SNR case. The result matched the one provided by the proponent. The results on complexity assessment provided by the proponent have also been verified.

It is recommended to consider for adoption of shorter-tap filter interpolation.

# References

[1] “Description of Core Experiment SCE 3: Combined Inter and Inter-Layer Prediction in SHVC”, JCTVC-L1103, January 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-L1009, January 2013

[3] “On interpolation filter for Generalized Residual Prediction”, JCTVC-L0265, January 2013

[4] “SCE3: Results of test 3.6 on Generalized Residual Prediction with shorter-tap MC filter”, JCTVC-M0073, April 2013

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.