| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0147 |
| --- | --- |

| Title: | SCE5: Crosscheck of SCE5.1.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides crosscheck result of SCE5.1.4.

# Introduction / Problem Statement / …

Motion vector coding with base layer motion data has been investigated under SCE5.

SCE5.1.4 contains the following proposals:

- A motion vector is derived from that of collocated and neighboring blocks with some refinement process which is similar to the SVC manner.

- The predicted motion vector is added to merge lists as a third candidate.

# Simulation

## Simulation Condition

SHM-1.0 is used as the anchor. On top of this anchor software the proposed methods have been implemented by the proponent, and have been successfully verified by the author. However, the author found that “Test 2” setting does not work with SNR scalability.

The followings are macro settings in “Typedef.h”:

Test 1:

#define KDDI_NOCOMPRESSMV	0

#define KDDI_MERGE3RD		1

#define KDDI_SVCMV		0

Test 2:

#define KDDI_NOCOMPRESSMV	0

#define KDDI_MERGE3RD		1

#define KDDI_SVCMV		1

Class A and B sequences have been tested with {RA, LP}_{2x,1.5x} conditions.

## Simulation Result

Summary of the results of Test 1 and 2 are shown in Table 2.2.1 and 2.2.2, respectively. Complete results are shown in JCTVC-M0147_test1.xls and JCTVC-M0147_test2.xls respectively.

The results match the one provided by the proponent.

With Test 1 gain is obtained by 0.0%, 0.2%, -0.2% and -0.1% for RA_2x, RA_1.5x, LD_2x and LD_1.5x respectively. With Test 2 gain is obtained by -0.1%, 0.3%, -0.2% and 0.0% for RA_2x, RA_1.5x, LD_2x and LD_1.5x respectively.

Table 2.2.1 Test 1

Table 2.2.2 Test 2

# Conclusion

This document provides crosscheck result of SCE5.1.4. With Test 1 gain is obtained by 0.0%, 0.2%, -0.2% and -0.1% for RA_2x, RA_1.5x, LD_2x and LD_1.5x respectively. With Test 2 gain is obtained by -0.1%, 0.3%, -0.2% and 0.0% for RA_2x, RA_1.5x, LD_2x and LD_1.5x respectively. The results match the one provided by the proponent.

It is recommended that the proposed method be discussed among other SCE5 proposals.

# References

[1] “Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-L1105, January 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-L1009, January 2013

[3] “TE5: Results of test 5.2.8 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer”, JCTVC-L0376, January 2013

[4] “SCE5: Results of test 5.1.4 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer”, JCTVC-M0413, April 2013

# Patent rights declaration(s)

Sony does not have any current or pending patent rights relating to the technology described in this contribution.