| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0148 |
| --- | --- |

| Title: | Signalling inter-layer reference picture positions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | yongjin9.cho@samsung.com b.d.choi@samsung.com m.w.park@samsung.com jinyoung79.lee@samsung.com jw0413.yoon@samsung.com hc.wey@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes new syntax elements in a slice header extension that signals positions of inter-layer reference pictures.

# Introduction

By default, inter-layer reference pictures are placed in the last entry of reference picture lists. However, it is demonstrated by experiments that putting inter-layer reference pictures in different positions has influence on the coding performance [2]. Hence, it is desirable to allow flexibility of positions of inter-layer reference pictures. Such flexibility can be achieved by explicitly signalling positions of inter-layer reference pictures. Even though the ref_pic_lists_modification() syntax element can be used for the purpose, it is not very efficient because all reordered entries in reference picture lists have to be signalled.

# Proposal

## Syntax changes in the slice segment header

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if (layer_id>0) { |  |
| num_l0_inter_layer_ref_pic_pos | ue(v) |
| if ( num_l0_inter_layer_ref_pic_pos > 0) |  |
| for ( i=0; i<num_l0_inter_layer_ref_pic; i++ ) |  |
| l0_inter_layer_ref_pic_pos[i] | ue(v) |
| if( slice_type = = B ) { |  |
| num_l1_inter_layer_ref_pic_pos | ue(v) |
| if ( num_l1_inter_layer_ref_pic_pos > 0) |  |
| for ( i=0; i<num_l1_inter_layer_ref_pic; i++ ) |  |
| l1_inter_layer_ref_pic_pos[i] | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| … |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

Table 1. Modification of the slice segment header

## Slice segment header semantics

num_l0_inter_layer_ref_pic_pos specifies the number of signaled inter-layer reference picture positions for list0 reference picture list. When num_l0_inter_layer_ref_pic_pos equals 0, all inter-layer reference pictures are placed in the default position. The maximum value of num_l0_inter_layer_ref_pic_pos is the number of direct reference layers of the current layer, i.e., NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ].

l0_inter_layer_ref_pic_pos[i] specifies the position of the i-th inter-layer reference picture. l0_inter_layer_ref_pic_pos[0] signals the initial position of the inter-layer reference pictures, and l0_inter_layer_ref_pic_pos[i] (i>0) signals the difference from l0_inter_layer_ref_pic_pos[i-1]. When num_l0_inter_layer_ref_pic_pos is less than NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ], l0_inter_layer_ref_pic_pos[num_l0_inter_layer_ref_pic_pos-1] specifies the position of all remaining inter-layer reference pictures.

num_l1_inter_layer_ref_pic_pos specifies the number of signaled inter-layer reference picture positions for list0 reference picture list. When num_l1_inter_layer_ref_pic_pos equals 0, all inter-layer reference pictures are placed in the default position. The maximum value of num_l1_inter_layer_ref_pic_pos is the number of direct reference layers of the current layer, i.e., NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ].

l1_inter_layer_ref_pic_pos[i] specifies the position of the i-th inter-layer reference picture. l1_inter_layer_ref_pic_pos[0] signals the initial position of the inter-layer reference pictures, and l1_inter_layer_ref_pic_pos[i] (i>0) signals the difference from l1_inter_layer_ref_pic_pos[i-1]. When num_l1_inter_layer_ref_pic_pos is less than NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ], l1_inter_layer_ref_pic_pos[num_l0_inter_layer_ref_pic_pos-1] specifies the position of all remaining inter-layer reference pictures.

## Decoding process

Decoding process for reference picture lists construction is explained in the highlighted text.

G.8.3.4  Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
ilrIdx = 0
ilrPos = l0_inter_layer_ref_pic_pos[ilrIdx]
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++ ) {
		if ( rIdx == ilrPos ) {
   		if ( ilrIdx >= num_l0_inter_layer_ref_pic_pos-1) {
           for ( ; ilrIdx < NumDirectRefLayers[ LayerIdInVps[nuh_layer_id] ]; rIdx++, ilrIdx++ )
              RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
   		}
   		else {
           RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
           ilrIdx++
           ilrPos = ilrPos + l0_inter_layer_ref_pic_pos[ ilrIdx ]
   		}
   	}
   	else {
   		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
   		i++
   	}
   }
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++ ) {
		if ( rIdx == ilrPos ) {
			if ( ilrIdx >= num_l0_inter_layer_ref_pic_pos-1) {
           for ( ; ilrIdx < NumDirectRefLayers[ LayerIdInVps[nuh_layer_id] ]; rIdx++, ilrIdx++ )
              RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
   		}
   		else {
           RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
           ilrIdx++
           ilrPos = ilrPos + l0_inter_layer_ref_pic_pos[ ilrIdx ]
   		}
   	}
   	else {
   		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
   		i++
   	}
   }
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++ ) {
		if ( rIdx == ilrPos ) {
   		if ( ilrIdx >= num_l0_inter_layer_ref_pic_pos-1) {
           for ( ; ilrIdx < NumDirectRefLayers[ LayerIdInVps[nuh_layer_id] ]; rIdx++, ilrIdx++ )
              RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
   		}
   		else {
           RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
           ilrIdx++
           ilrPos = ilrPos + l0_inter_layer_ref_pic_pos[ ilrIdx ]
   		}
   	}
   	else {
   		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr [ i ]
   		i++
   	}
   }
	for( i = ilrIdx; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 
}

The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
		RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
ilrIdx = 0
ilrPos = l1_inter_layer_ref_pic_pos[ilrIdx]
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++ ) {
		if ( rIdx == ilrPos ) {
			if ( ilrIdx >= num_l1_inter_layer_ref_pic_pos-1) {
           for ( ; ilrIdx < NumDirectRefLayers[ LayerIdInVps[nuh_layer_id] ]; rIdx++, ilrIdx++ )
              RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
   		}
   		else {
           RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
           ilrIdx++
           ilrPos = ilrPos + l1_inter_layer_ref_pic_pos[ ilrIdx ]
   		}
   	}
   	else {
   		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
   		i++
   	}
   }
   for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++ ) {
		if ( rIdx == ilrPos ) {
   		if ( ilrIdx >= num_l1_inter_layer_ref_pic_pos-1) {
           for ( ; ilrIdx < NumDirectRefLayers[ LayerIdInVps[nuh_layer_id] ]; rIdx++, ilrIdx++ )
              RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
   		}
   		else {
           RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
           ilrIdx++
           ilrPos = ilrPos + l1_inter_layer_ref_pic_pos[ ilrIdx ]
   		}
   	}
   	else {
   		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
   		i++
   	}
   }
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++ ) {
		if ( rIdx == ilrPos ) {
   		if ( ilrIdx >= num_l1_inter_layer_ref_pic_pos-1) {
           for ( ; ilrIdx < NumDirectRefLayers[ LayerIdInVps[nuh_layer_id] ]; rIdx++, ilrIdx++ )
              RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
   		}
   		else {
           RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ ilrIdx ]
           ilrIdx++
           ilrPos = ilrPos + l1_inter_layer_ref_pic_pos[ ilrIdx ]
   		}
   	}
   	else {
   		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr [ i ]
   		i++
   	}
   }
	for( i = ilrIdx; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ] 
}

When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)	
	RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :RefPicListTemp1[ rIdx ]

# References

- L. Zhang et. al., “Proposed text for JCT3V-B0046 based on MV-HEVC working draft,” Document JCT3V-B0046, the 2nd JCT3V meeting, Shanghai, CN, Oct. 2012

- H. Lee et. al., “TE 2: Result of test 3.2.2 on inter-layer reference picture placement,” Document JCTVC-L0060, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).