| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0150 |
| --- | --- |

| Title: | Inter-layer reference picture set initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | yongjin9.cho@samsung.com b.d.choi@samsung.com m.w.park@samsung.com jinyoung79.lee@samsung.com jw0413.yoon@samsung.com hc.wey@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes to have inter-layer reference picture lists be initialized in the order of closeness to the current picture.

# Introduction

Tables 1 and 2 show the currently defined inter-layer reference picture set marking process and derivation process of related variables (NumDirectRefLayers[i] and RefLayerId[i][j]). As can be observed from Table 2, inter-layer reference pictures are put in an ascending order (from lower to higher layer) by the current marking process. But, this is against the convention of putting closer reference pictures to a current picture in the head of reference picture lists. For this reason, we propose the ordering of inter-layer reference pictures be changed to a descending order (from closer to farther layers).

G.8.1.2  Marking process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ currLayerId ] ]; i++ ) {
	rlPic = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ currLayerId ][ i ] ]
	RefPicSetInterLayer[ i ] = rsPic as derived in the clause G.8.1.4 with rlPic given as input
	rsPic is marked as "used for long-term reference"
}

Table 1. Marking process for inter-layer reference picture set (G.8.1.2 [1])

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = 0, NumDirectRefLayers[ i ] = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

Table 2. Derivation process of NumDirectReftLayers[i] and RefLayerId[i][j] (F.7.4.3.1.1 [2])

# Proposal

We propose to initialize a reference picture set in such a way that closer inter-layer reference pictures to the current layer are listed before farther inter-layer reference pictures in each reference picture list. This follows the convention of placing closer reference pictures in reference picture lists in the version 1 HEVC and legacy H.264 specifications. The proposed change to F.7.4.3.1.1 is indicated in Table 3, where the index ‘j’ is put in a descending order from ‘i-1’ to 0.

The variables NumDirectRefLayers[ i ] and RefLayerId[ i ][ j ] are derived as follows:

for( i = 1; i <= vps_max_layers_minus1; i++ )
	for( j = i-1, NumDirectRefLayers[ i ] = 0; j >=0; j-- )
		if( direct_dependency_flag[ i ][ j ]  = =  1 )
			RefLayerId[ i ][ NumDirectRefLayers[ i ]++ ] = layer_id_in_nuh[ j ]

Table 3. Proposed change in the derivation of RefLayerId[i][j]

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).