| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0151r2 |
| --- | --- |

| Title: | AHG9: Inter-layer Prediction Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Tzu-Der Chuang, No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com +886-3-5670766 ext. 25555 peter.chuang@mediatek.com |
| Source: | Sony Corp., MediaTek Inc. |  |  |

_____________________________

# Abstract

In SHVC draft text (JCTVC-L1008), dependency between layers at sequence-level is specified in vps_extension(). Inter-layer prediction signalling at picture-level or slice-level would be necessary for the purpose of providing trade-off between coding efficiency and complexity or error resiliency. For example, a certain implementation will be based on multi-loop scalability while other will be based on single-loop scalability, depending on memory bandwidth capabilities.

It is proposed by JCTVC-K0175, K0264, L0258, L0278 and L0449 that inter-layer prediction signalling be specified in terms of temporal hierarchy. Single-loop scalability as proposed by JCTVC-L0111 and L0154 can be regarded as special case of this signalling. Additionally it is proposed by JCTVC-L0258 that inter-layer prediction signaling be specified at the slice header.

In addition, it is proposed by JCTVC-L0071 and L0278 that inter-layer texture prediction and inter-layer syntax prediction separately. This can also be applied for SVC-style single-loop scalability as proposed by JCTVC-L0154. Motion vector information is the only syntax element for inter-layer prediction in current SHM (JCTVC-L1008). However prediction of other syntax elements like intra mode (JCTVC-L1101), residue (JCTVC-L1103), or slice syntax (JCTVC-L0231) is now being studied. Single-loop scalability as proposed by JCTVC-L0154 also contains such inter-layer syntax prediction. Therefore separation of indications of inter-layer texture prediction and inter-layer syntax prediction will be useful.

This document contains proposed syntax for inter-layer texture and syntax prediction signalling.

# Introduction / Problem Statement / …

In SHVC draft text (JCTVC-L1008), dependency between layers is specified in vps_extension(). It is a sequence-level specification. However enabling/disabling of inter-layer prediction specification at picture-level or slice-level will help providing trade-off between coding efficiency and complexity, or error resiliency. For example, a certain implementation will be based on multi-loop scalability while other will be based on single-loop scalability, depending on memory bandwidth capabilities.

Moreover, separation of signalling of on/off with inter-layer texture and syntax prediction will also be helpful for efficiency-complexity trade-off, as inter-layer texture prediction requires more storage size than syntax prediction.

The followings are related contributions in the past JCTVC meetings:

- Proposals for signaling inter-layer prediction associated with temporal hierarchy: JCTVC-K0175, JCTVC-K0264, JCTVC-L0258, JCTVC-L0278 and JCTVC-L0449

  - Single-loop scalability can be regarded as the special case of these proposals: JCTVC-L0111 and JCTVC-L0154

  - Additionally JCTVC-L0258 proposed to signal enabling/disabling of inter-layer prediction at the slice header.

- Separation of signaling inter-layer texture and syntax prediction: JCTVC-L0071, JCTVC-L0278

  - Motion vector coding is the only syntax element for inter-layer syntax prediction: JCTVC-L1008

  - Inter-layer syntax predictions of other syntax elements have been proposed as follow:

    - Intra-mode coding: JCTVC-L1101

    - Residue signal: JCTVC-L1103

    - Slice header syntax: JCTVC-L0231

    - Single-loop scalability (JCTVC-L0154) also contains inter-layer syntax prediction

# Proposal

## Inter-layer Texture Prediction Indication

It is proposed by JCTVC-K0175, K0264, L0278 and L0449 that inter-layer prediction signalling be specified in terms of temporal hierarchy. Single-loop scalability as proposed by JCTVC-L0111 and L0154 can be regarded as special case of this signalling.

It is proposed by JCTVC-L0258 that inter-layer prediction signaling be specified at the slice header.

This document proposes that inter-layer texture prediction be specified as follows, like RPS in HEVC version 1:

- Default setting be specified at GOP-level, associated with temporal hierarchy, and

- it can be override at the slice header

The proposed syntax is shown in Table 2.1.1 and 2.1.2.

Table 2.1.1

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| for( j = 0; j <i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| for( i = 0; i<= vps_max_layers_minus1; i++ ) { |  |
| vps_max_sublayer_for_inter_layer_texture_prediction[ i ] | u(3) |
| } |  |
| } |  |

vps_max_sublayer_for_inter_layer_texture_prediction specifies max_sublayer where inter-layer texture prediction is enabled. If the temporalId of the current picture is smaller than or equal to vps_max_sublayer_for_inter_layer_texture_prediction, inter-layer texture prediction is enabled. Otherwise inter-layer texture prediction is disabled. When not specified its value is inferred to be equal to vps_max_sub_layers_minus1.

Table 2.1.2

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( layer_id >0 && tepmpralId < vps_max_sublayer_for_inter_layer_texture_prediction[layer_id]-1 ) |  |
| slice_inter_layer_texture_prediction_flag | u(1) |
| … |  |
| } |  |

slice_inter_layer_texture_prediction_flag is equal to 1 indicates that inter-layer texture prediction is enabled for the current slice. slice_inter_texture_prediction_flag is equal to 0 indicates that inter-layer is disabled for the current slice. When not specified its value is inferred to be equal to 1 if temporalId < vps_max_sublayer_for_inter_layer_texture_prediction[layer_id] and 0 otherwise.

## Inter-layer Syntax Prediction Indication

Motion vector information is the only syntax element for inter-layer prediction in current SHM (JCTVC-L1008). In this case it is proposed to add syntax elements “sps_colblmv_mvp_enabled_flag” and “slice_colbl_mvp_enabled_flag” as shown in Table 2.2.1 and 2.2.2.

It is proposed by JCTVC-L0073 and L0221 that tmvp is disabled at the enhancement layer, if co-located base-layer motion information is available. This can be realised by setting temporal_mvp_enabled_flag as 0 and colbl_mvp_enabled_flag as 1.

Table 2.2.1:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( layer_id > 0) |  |
| sps_colbl_mvp_enabled_flag | u(1) |
| ... | u(1) |
| } |  |

sps_colbl_mvp_enabled_flag equals to 1 specifies that slice_colbl_mvp_enabled_flag is present in the slice headers of non-IDR pictures in the CVS. sps_colbl_mvp_enabled_flag equals to 0 specifies that slice_colbl_mvp_flag is not present in slice headers and that collocated base layer motion vector predictors are not used in the CVS.

Table 2.2.2:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( layer_id > 0 && sps_colblmv_mvp_enabled_flag ) |  |
| slice_colbl_mvp_enabled_flag | u(1) |
| ... |  |
| } |  |

slice_colbl_mvp_enabled_flag specifies whether collocated base layer motion vector prediction can be used for inter prediction. If slice_colbl_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no collocated base layer motion vector predictor is used in decoding of the current picture. Otherwise (slice_colbl_mvp_enable_flag is equal to 1), collocated base layer motion vector predictor may be used in decoding of the current picture. When not present the value of slice_colbl_mvp_enabled_flag is inferred to be equal to 0.

However inter-layer predictions of other syntax elements have been proposed as follows:

-	Intra-mode coding: JCTVC-L1101

-	Residue signal: JCTVC-L1103

-	Slice header syntax: JCTVC-L0231

A proposal on single-loop scalability, JCTVC-L0154, also contains inter-layer syntax prediction as intra-mode coding, motion vector coding and base-layer residuals.

If these inter-layer syntax predictions are adopted, it will be useful to signal enabling/disabling of inter-layer syntax prediction as follows, proposed by JCTVC-L0071:

Table 2.2.3:

| if (layer_id > 0) |  |
| --- | --- |

Table 2.2.3:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ){ |  |
| if (layer_id > 0) |  |
| pps_inter_layer_syntax_prediction_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_inter_layer_syntax_prediction_flag is equal to 1 indicates that inter-layer syntax prediction is enabled for the current picture. pps_inter_layer_syntax_prediction_flag is equal to 0 indicates that inter-layer syntax prediction is disabled for the current picture. When not present its value is inferred to be equal to 0.

Table 2.2.4:

| if (layer_id > 0 && pps_inter_layer_syntax_pred_flag) |  |
| --- | --- |

Table 2.2.4:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (layer_id > 0 && pps_inter_layer_syntax_pred_flag) |  |
| slice_header_inter_layer_syntax_prediction_flag | u(1) |
| … |  |
| } |  |

slice_header_inter_layer_syntax_prediction_flag is equal to 1 indicates that inter-layer syntax prediction is enabled for the current slice. slice_header_inter_layer_syntax_prediction_flag is equal to 0 indicates that inter-layer syntax prediction is disabled for the current slice. When not present its value is inferred to be equal to pps_inter_layer_syntax_prediction_flag.

Or it is possible that inter-layer syntax prediction be specified associated with temporal hierarchy and be override at the slice header, similarity with the syntax proposed in the previous section (for inter-layer texture prediction).

# Conclusion

In this contribution syntax for signalling inter-layer texture and syntax prediction is proposed as:

- Indication of inter-layer texture prediction and syntax prediction be signalled separately;

- Inter-layer texture (and syntax) prediction be signalled in terms of temporal hierarchy at VPS and can be override at the slice header;

It is recommended that the proposed changes in syntax elements be considered for adoption with other related proposals.

MediaTek and Sony will volunteer integrating the proposed syntax into SHM.

# References

[1] “On inter-layer prediction enabling/disabling for HEVC scalable extensions”, JCTVC-K0175, October 2012

[2] “Hierarchical inter-layer prediction in multi-loop scalable extension of HEVC”, JCTVC-K0264, October 2012

[3] “AHG9: Inter-layer prediction flag and inter-layer syntax prediction flag”, JCTVC-L0071, January 2013

[4] “Non-TE5: On the effectiveness of temporal and base layer co-located motion vector prediction candidates”, JCTVC-L0073, January 2013

[5] “Lightweight single-loop scalability with SHVC”, JCTVC-L0111, January 2013

[6] “Single-loop SNR scalability using Binary Residual Refinement Coding”, JCTVC-L0154, January 2013

[7] “Non-TE5: Memory reduction for MV data of EL in SHVC”, JCTVC-L0221, January 2013

[8] “Inter-layer slice header syntax element prediction in SHVC”, JCTVC-L0231, January 2013

[9] “Inter layer prediction indication flags and pictures management”, JCTVC-L0258, January 2013

[10] “SHVC: On Inter-layer Prediction”, JCTVC-L0278, January 2013

[11] “Design considered for signaling inter-layer prediction indication”, JCTVC-L0449, January 2013

[12] “SHVC Test Model 1 (SHM 1)”, JCTVC-L1007, January 2013

[13] “SHVC Draft Text 1”, JCTVC-L1008, January 2013

[14] “Common SHM test conditions and software reference configurations”, JCTVC-L1009, January 2013

[15] “Core Experiment 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, January 2013

[16] “Description of Core Experiment SCE 3: Combined Inter and Inter-Layer Prediction in SHVC”, JCTVC-L1103, January 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).