| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0152 |
| --- | --- |

| Title: | Slice level inter-layer prediction signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | yongjin9.cho@samsung.com b.d.choi@samsung.com m.w.park@samsung.com jinyoung79.lee@samsung.com jw0413.yoon@samsung.com hc.wey@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes a method to enable a finer control of inter-layer prediction.

# Introduction

Current straw-man design is shown in table 1, where inter-layer prediction flagging is limited to a temporal sub-layer. To allow a finer control, we need a picture and/or a slice level signalling of inter-layer prediction flag.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| for( j = 0; j <i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| for( i = 0; i<= vps_max_layers_minus1; i++ ) { |  |
| max_sublayer_for_ilp_plus1[ i ] | u(3) |
| } |  |
| } |  |

max_sublayer_for_ilp_plus1[ i ] equal to 0 indicates that non-RAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_sublayer_for_ilp_plus1[ i ] greater than 0 indicates that pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and with TemporalId greater than max_sublayer_for_ilp_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_sublayer_for_ilp_plus1[ i ] is unspecified.

Table 1 Straw-man syntax and semantics for inter-layer prediction signalling

# Proposal

In the slice segment header design of the HEVC version 1, a number of flags are reserved for future use. We propose to use one of the reserved flag in the slice segment header as an inter-layer prediction flag to enable a finer signalling of inter-layer prediction flag.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| for( i = 0; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| … |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

Table 2. Reserved flags in the slice segment header

Because the slice_reserved_flag is not specified for any use currently, we propose to use one of the reserved flags, e.g., slice_reserved_flag[0], as a no inter-layer prediction flag. Then, a picture level inter-layer prediction control can be enabled with the following modification in semantics.

G.7.4.7.1 General slice segment header semantics

The specifications of subclause F.7.4.7.1 apply with the following modifications.

slice_reserved_flag[ i ] with i equal to 0 indicates whether the coded slice is used as reference for inter-layer prediction, when max_sublayer_for_ilp_plus1[j] – 1 is greater than or equal to TemporalId for the j-th layer that contains the coded slice. slice_reserved_flag[0] equal to 1 signals that the coded slice is not used as reference for inter-layer prediction (no inter-layer prediction). slice_reserved_flag[0] shall be equal to 1 for sub-layers of j-th layer whose TemporalId is greater than max_sublayer_for_ilp_plus1[j] – 1. When max_sublayer_for_ilp_plus1[j] is equal to 0, slice_reserved_flag[0] shall be equal to 1 for non-RAP pictures. When not present, slice_reserved_flag[0] is inferred to be equal to 1. Slice_reserved_flag[i] with i greater than 0 is reserved for future use by ITU-T | ISO/IEC.

# References

- M. Hanuksela et. al., “Design considered for signalling inter-layer prediction indication,” Document JCTVC-L0449, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).