| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0153 |
| --- | --- |

| Title: | Extension parameter set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi | Email: | yongjin9.cho@samsung.com b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document introduces an extension parameter set (EPS) that carries new syntax elements in the unified extension design.

# Introduction

In the version 1 HEVC specification, three parameter sets are defined to carry coding parameters at video, sequence, and picture levels. With the standardization activity of unified HEVC extensions, we need a new parameter set that carry parameters related to extension coding tools.

# Proposal

## Extension parameter set RBSP syntax

| ext_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| eps_ext_parameter_set_id | ue(v) |
| eps_seq_parameter_set_id | ue(v) |
| eps_cp_present_flag | u(1) |
| if(eps_cp_present_flag) { |  |
| eps_cp_precision | ue(v) |
| eps_max_view_id | u(6) |
| for ( i = 0; i <eps_max_view_id; i++ ) { |  |
| eps_cp_scale[ i ] | se(v) |
| eps_cp_off[ i ] | se(v) |
| eps_cp_inv_scale_plus_scale[ i ] | se(v) |
| eps_cp_inv_off_plus_off[ i ] | se(v) |
| } |  |
| } |  |
| eps_dlt_flag | u(1) |
| if ( eps_dlt_present_flag ) { |  |
| // syntaxes on depth lookup table |  |
| } |  |
| eps_adapt_upsample_present_flag | u(1) |
| if ( eps_adapt_upsample_present_flag ) |  |
| for ( i=0 ; i < 8 ; i ++ ) |  |
| eps_auf_coeff[ i ] | se(v) |
| eps_cross_color_filter_enable_flag | u(1) |
| if (eps_cross_color_filter_enable_flag ) { |  |
| eps_cross_color_filter_cb_enhance_ flag | u(1) |
| if (eps_cross_color_filter_cb_enhance_ flag) { |  |
| for ( i=0 ; i < 12 ; i ++ ) |  |
| eps_cb_cross_color_filter_coeff_plus8 [i] | u(4) |
| } |  |
| eps_cross_color_filter_cr_enhance_ flag | u(1) |
| if ( eps_cross_color_filter_cr_enhance_ flag ) { |  |
| for ( i=0 ; i < 12 ; i ++ ) |  |
| eps_cr_cross_color_filter_coeff_plus8 [i] | u(4) |
| } |  |
| } |  |
| } |  |

## Extension parameter set RBSP semantics

eps_ext_parameter_set_id identifies the EPS for reference by other syntax elements.

eps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the active EPS. The value of eps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

eps_cp_present_flag specifies whether the syntax elements on camera paremters are present.

eps_cp_precision specifies the precision of eps_cp_scale[ i ], eps_cp_off[ i ], eps_cp_inv_scale_plus_scale[ i ], and eps_cp_inv_off_plus_off[ i ]. The value of eps_cp_precision shall be in the range of 0 to 5, inclusive.

eps_max_view_id specifies the number of layers which have eps_cp_scale[ i ], eps_cp_off[ i ], eps_cp_inv_scale_plus_scale[ i ], and eps_cp_inv_off_plus_off[ i ].

eps_cp_scale[ i ], eps_cp_off[ i ], eps_cp_inv_scale_plus_scale[ i ], and eps_cp_inv_off_plus_off[ i ] specify conversion parameters for converting a depth value to a disparity value.

eps_dlt_flag specifies whether the syntax elements on depth lookup table are present.

eps_adapt_upsample_present _flag equal to 1 specifies the presence of the syntax element auf_coeff[i]. eps_adapt_upsample_enable_flag equal to 0 specifies that default upsampling filter shall be used.

eps_auf_coeff[i] specify the value of the i-th adaptive upsampling filter coefficients.

eps_cross_color_filter_enable_flag equal to 1 specifies the presence of cross-color enhancement filter parameters.

eps_cross_color_cb_enhance_flag equal to 1 specifies the presence of the syntax element eps_cb_cross_color_filter_coeff_plus8[i]. eps_cross_color_filter_enable_flag to 0 specifies that Cb enhancement is disabled.

eps_cb_cross_color_filter_coeff_plus8 [i] specifies the values of Cb enhancement filter coefficient plus 8. The coefficients take values from -8 to 7.

eps_cross_color_cr_enhance_flag equal to 1 specifies the presence of the syntax element eps_cr_cross_color_filter_coeff_plus8[i]. cr_enhance_enabled_flag equal to 0 specifies that Cr enhancement is disabled.

eps_cr_cross_color_filter_coeff_plus8 [i] specifies the values of Cr enhancement filter coefficient plus 8. The coefficients take values from -8 to 7.

# References

- E. Alshina et. al., “Non SCE4: Simplified design of cross-color inter-layer filter,” Document JCTVC-M0089, the 13th JCTVC meeting, Incheon, KR, Apr. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).