| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0154 |
| --- | --- |

| Title: | Non-SCE3 Combination of Merge and GRP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjing Zhu Haitao Yang Huawei Industrial Base, Bantian Longgang District, Shenzhen China 518129. | Email: | wenjing.zhu@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes a combination of Generalized Residual Prediction (GRP) mode and merge mode. In the proposed modification, GRP is only allowed if the motion of a 2Nx2N PU is derived with merge mode. Up to three GRP modes as well as normal temporal prediction mode can be selected for such a PU. It is reported the BD-rate reduction of the proposed method are -1.7%/-2.6%/-2.1% for RA 2X/1.5X/SNR, -3.5%/-5.0%/-4.4% for LDP 2X/1.5X/SNR, and -3.5%/-5.1%/-3.8% for LDB 2X/1.5X/SNR.

# Introduction

GRP is proposed in [1] [2] [3], the motivation is to improve the coding efficiency with second order prediction. The prediction signal combines three components, EL_ref, BL_ref and BL_col.

The final prediction signal is as follow (uni-directional case):

EL_Pred = w1*EL_ref + w2*BL_ref + w3*BL_col, subject to w1+w2+w3 = 1

Many contributions propose different GRP modes, their results show that utilizing of GRP indeed has much gain. This document proposes a simplification of GRP and provides test results of Combination of Merge and GRP.

# Technical details

## Selection of GRP Mode

To study performance of different GRP modes, we design a simulation. In the simulation, every PU can choose his own GRP weight (w1, w2 and w3) according to SATD, the simulation provides 231 kinds of weight. The weight combination detail is that dynamic range of w1 and w2 is [-1, 1], step of changing w1 and w2 is 0.1, w3 is obtained through equation w1+w2+w3 = 1, and the absolute value of w3 is less than or equal to 1. If w1 =1, w2 = 0 and w3 = 0, GRP become normal temporal prediction, so normal temporal prediction is treated as one GRP mode. The mode index is not written to bitstream, so encoder and decoder are mismatch.

Run the simulation in LDP case, and then collect GRP weight of every PU, calculate every GRP weight’s frequency, the result is that Res-0(normal temporal prediction), Res-1, Res-0.5 and Diff0.5 are top 4 most frequent GRP modes chosen.

In this contribution, Res-1, Res-0.5 and Diff0.5 are the GRP modes utilized. When utilizing GRP, the mode index should be written into bitstream, the mode index assignment is as below.

Mode Index Assignment

| GRP mode idx | Mode | Prediction siganl |
| --- | --- | --- |
| 0 | Res-0 (normal temporal prediction) | EL_pred = EL_ref |
| 1 | Res-0.5 | EL_pred = EL_ref + 0.5 (BL_col – BL_ref) |
| 2 | Diff-0.5 | EL_pred = BL_col + 0.5 (EL_ref – BL_ref) |
| 3 | Res-1 | EL_pred = EL_ref + (BL_col – BL_ref) |

## Combination of merge and GRP

In merge mode, the current PU use spatial or temporal neighbour PU’s motion information, so the residual tends to be larger than inter PU. Doing GRP on merge PU tends to give more PSNR and bit-rate benefits.

Based on the guess above, we design a simulation. To avoid too high memory bandwidth burden, just do GRP on 2Nx2N PU and every PU have four modes to choose, Res-0, Res-1, Res-0.5 and Diff-0.5. In the simulation, collect the number of 2Nx2N merge PU, the number of 2Nx2N GRP PU and the number of merge PU utilizing GRP.

Run the simulation in LDP case, results shows that more than about 56% of 2Nx2N merge PU utilize GRP and 95% of 2Nx2N GRP PU is merge PU.

Based on the theoretical and statistical analysis, the contribution proposes just do GRP on 2Nx2N merge PU.

## Fast Encoding Algorithm

This contribution’s fast encoding Algorithm is the same as [1]. The detail is that if left, top, and top-right CUs are all larger than the current CU, no GCP mode will be evaluated (i.e. only uses normal inter prediction).

# Test Results

## TestSet1 (FastEnc OFF)

## TestSet2 (FastEnc ON)

# Proposed Syntax Table

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if ( layerId > 0 && ( cu_skip_flag[ x0 ][ y0 ] ¦ ¦ ( merge_flag[ x0 ][ y0 ] && PartMode = = 2Nx2N ) ) |  |
| GRP_Weight_flag |  |
| } |  |

# Conclusions

This contribution proposed to constrain the usage of GRP to 2Nx2N merge PU. It is reported the BD-rate reduction of the proposed method are -1.7%/-2.6%/-2.1% for RA 2X/1.5X/SNR, -3.5%/-5.0%/-4.4% for LDP 2X/1.5X/SNR, and -3.5%/-5.1%/-3.8% for LDB 2X/1.5X/SNR.

# References

- T.-D. Chuang, S. Liu, M. Guo, P. Lai, Y.-W. Huang, S. Lei, “Non-TE3: Adaptive predictor compensation with generalized residual prediction”, JCTVC-L0074, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0078, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “Non-TE3: Extension of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0190, Geneva, Switzerland, 14–23 Jan. 2013.

- E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-L1009, Geneva, Switzerland, 14–23 Jan. 2013.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).