| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0154 |
| --- | --- |

| Title: | Non-SCE3 Combination of Merge and GRP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjing Zhu Haitao Yang Huawei Industrial Base, Bantian Longgang District, Shenzhen China 518129. | Email: | wenjing.zhu@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes a combination of Generalized Residual Prediction (GRP) mode and merge mode. In the proposed modification, GRP is only allowed if the motion of a 2Nx2N PU is derived with merge mode. Up to three GRP modes as well as normal temporal prediction mode can be selected for such a PU. It is reported the BD-rate reduction of the proposed method are -1.7%/-2.6%/-2.1% for RA 2X/1.5X/SNR, -3.5%/-5.0%/-4.4% for LDP 2X/1.5X/SNR, and -3.5%/-5.1%/-3.8% for LDB 2X/1.5X/SNR.

# Introduction

GRP is proposed in [1] [2] [3], the motivation is to improve the coding efficiency with second order prediction. The prediction signal combines three components, EL_ref, BL_ref and BL_col. Many contributions propose different GRP modes, their results show that utilizing of GRP indeed has much gain. But what’s the best GRP modes? To answer this question, firstly, interpret GRP as multi-hupothesis prediction, The final prediction signal is as follow (uni-directional case):

EL_Pred = w1*EL_ref + w2*BL_ref + w3*BL_col, subject to w1+w2+w3 = 1

Secondly, find the best weighting factor combination (w1, w2, w3).

Following the steps, we design a simulation, in the simulation, every PU can choose a set of weighting factors (w1, w2, w3) according to SATD, and this simulation provides 231 sets. The weighting factor combination detail is that dynamic range of w1 and w2 is [-1, 1], step of changing w1 and w2 is 0.1, w3 is obtained through equation w1+w2+w3 = 1, and the absolute value of w3 is less than or equal to 1. If w1 =1, w2 = 0 and w3 = 0, GRP becomes normal temporal prediction, so normal temporal prediction is treated as one GRP mode.

Every PU has its own best GRP weighting factor, so combining all the PU’s best GRP weighting factor information can give distribution of GRP weighting factor. The design is to show distribution of the three weighting factor, so the selection of weighting factors are not signalled.

Run the simulation in LDP case, and then collect GRP weight of every PU, calculate every GRP weighting factor’s frequency, the result is that (1, 0, 0), (1, -1, 1), (1, -0.5, 0.5), (0.5, -0.5, 1)are top 4 most frequent GRP weighting factor chosen. The detail is in [6]. Table 1 shows the correspondence between GRP weighting factor and GRP modes proposed.

Table 1 the top 4 sets of weighting factor and corresponding GRP modes

| (w1, w2, w3) | Prediction signal | Inter prediction mode |
| --- | --- | --- |
| (1, 0, 0) | EL_pred = EL_ref | Res-0 |
| (1, -1, 1) | EL_pred = EL_ref + (BL_col – BL_ref) | Res-1 |
| (1, -0.5, 0.5) | EL_pred = EL_ref + 0.5 (BL_col – BL_ref) | Res-0.5 |
| (0.5, -0.5, 1) | EL_pred = BL_col + 0.5 (EL_ref – BL_ref) | Diff-0.5 |

Among the 4 sets of GRP weighting factor, (1, 0, 0) is the normal temporal prediction, remaining 3 weights were proposed as GRP modes before, Table 2 shows the correspondence between GRP weighting factor and GRP mode proposed.

# Motivation

In current design of HEVC, for PU utilizing inter prediction, there is a special mode call merge mode. In merge mode, to save side information, the current PU use spatial or temporal neighbour PUs’ motion information.

If a PU is merge PU, because it inherits motion information from other PU, from the perspective of prediction accuracy, it may not be good, so the residual tends to be larger than ordinary inter PU. Purpose of utilizing GRP is to get more accurate prediction so the residual can be minimized. According to the analysis above, it is natural to combine merge and GRP together, to achieve more accurate prediction and less head information in the same time. It is also believed that the using of GRP can increase the number of blocks using merge, since the combination with GRP make it more preferable.

Based on the guess above, to find the relationship between merge and GRP, we design a simulation. To avoid too high memory bandwidth burden, do GRP only on 2Nx2N PU and every PU has four modes to choose, Res-0, Res-1, Res-0.5 and Diff-0.5, the selection of GRP mode is written into bitrate, PU chooses the best GRP mode according to RDO. In the simulation, collect the number of 2Nx2N PU with only merge, the number of 2Nx2N PU with only GRP, and the number of 2Nx2N PU with both merge and GRP. The ratio of GRP PU in merge PU and the ratio of merge PU in GRP PU are shown in Table 3 and Table 4.

Run the simulation in LDP case, collect the needed data, the statistical data is as below.

Table 3 Statistical Relationship between Merge And GRP (Spatial case)

| Spatial Ratio | Conditional probability | BL_QP/EL_QP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (22,22) | (26,26) | (30,30) | (34,34) | (22,24) | (26,28) | (30,32) | (34,36) |
| 1.5 | MRGInGRP | 0.9291 | 0.9382 | 0.9468 | 0.9593 | 0.9256 | 0.94 | 0.958 | 0.9737 |
|  | GRPInMerge | 0.7074 | 0.6309 | 0.5969 | 0.5713 | 0.7518 | 0.688 | 0.6616 | 0.6509 |
| 2.0 | MRGInGRP | 0.9063 | 0.9236 | 0.9414 | 0.9559 | 0.9143 | 0.9348 | 0.9532 | 0.9635 |
|  | GRPInMRG | 0.5117 | 0.4176 | 0.3841 | 0.339 | 0.5465 | 0.4921 | 0.4738 | 0.4193 |

Table 4 Statistical Relationship between Merge And GRP (SNR case)

| Spatial Ratio | Conditional probability | BL_QP/EL_QP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (26,20) | (30,24) | (34,28) | (38,32) | (26,22) | (30,26) | (34,30) | (38,34) |
| 1.0 | MRGInGRP | 0.9475 | 0.953 | 0.9538 | 0.9558 | 0.9589 | 0.9611 | 0.9628 | 0.9669 |
|  | GRPInMRG | 0.7573 | 0.6381 | 0.5354 | 0.4495 | 0.7738 | 0.7495 | 0.6453 | 0.5565 |

Results shows that more than about 56% of 2Nx2N merge PU utilizes GRP and 95% of 2Nx2N GRP PU is merge PU. The simulation shows a strong relationship between merge and GRP.

# Technical details

## Combination of merge and GRP

From the statistical data, we know that almost all the GRP happens on merge PU, so we propose doing GRP only on merge PU, to achieve the trade-off between the compression performance and the computational complexity, the proposed method is only applied to the EL 2Nx2N PUs.

In this technical proposal, For 2Nx2N PU with merge, Normal temporal prediction and GRP can be chosen according to RDO, for other PU, only do normal temporal prediction.

## Selection of GRP Mode

From statistical analysis about GRP weight, this proposal chooses Res-1, Res-0.5 and Diff-0.5 as the three GRP modes utilized. For 2Nx2N PU with merge, the three GRP modes and normal temporal prediction can be selected from the perspective of RDO, for other PU, only do normal temporal prediction.

## Signal of GRP Modes

For 2Nx2N with merge, write the mode index into bitstream. Table 5 shows the mode index assignment. The index is codes as TU code, every bin has a context model, and three context models are needed for mode index.

For the statistical data above, we know that about 45% of 2Nx2N PU with merge chooses normal temporal prediction, so Res-0 should have the shortest TU code, so mode index of Res-0 is 0.

Table 5 Mode Index Assignment

| Inter prediction mode | Prediction |
| --- | --- |
| 0 | Res-0 (normal temporal prediction) |
| 1 | Res-0.5 |
| 2 | Diff-0.5 |
| 3 | Res-1 |

## Proposed Syntax table

This proposal changes the syntax table, the detail is as below.

Table 6 Proposed Syntax Table

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| … |  |
| } |  |
| if ( layerId > 0 && ( cu_skip_flag[ x0 ][ y0 ] ¦ ¦ ( merge_flag[ x0 ][ y0 ] && PartMode = = 2Nx2N ) ) |  |
| inter_pred_mode | ae(v) |
| } |  |

# Test Results

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y |  | U | V | Y | U | V |  |  |
| Class A | -1.7% | -4.2% | -4.3% |  |  |  |  | -1.5% | -4.0% | -4.3% |  |  |
| Class B | -1.7% | -2.4% | -2.6% | -2.6% |  | -4.7% | -5.1% | -2.3% | -3.1% | -3.0% |  |  |
| Overall (Test vs Ref) | -1.7% | -2.9% | -3.1% | -2.6% |  | -4.7% | -5.1% | -2.1% | -3.4% | -3.4% |  |  |
| Overall (Test vs single layer) | 17.2% | 28.1% | 28.9% | 13.4% |  | 22.6% | 24.4% | 12.4% | 24.5% | 28.4% |  |  |
| EL only (Test vs Ref) | -3.2% | -4.4% | -4.5% | -6.4% |  | -8.5% | -8.9% | -3.3% | -4.7% | -4.7% |  |  |
| Enc Time[%] | 120.6% |  |  | 113.3% |  |  |  | 114.9% |  |  |  |  |
| Dec Time[%] | 105.1% |  |  | 107.7% |  |  |  | 106.3% |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  | Matched |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |  |  |
|  | Y | U | V | Y |  | U | V | Y | U |  | V |  |
| Class A | -3.1% | -4.1% | -3.4% |  |  |  |  | -3.4% | -4.1% | -3.5% |  |  |
| Class B | -3.7% | -1.9% | -1.2% | -5.0% |  | -4.5% | -3.5% | -4.9% | -4.4% | -4.1% |  |  |
| Overall (Test vs Ref) | -3.5% | -2.5% | -1.8% | -5.0% |  | -4.5% | -3.5% | -4.4% | -4.3% | -3.9% |  |  |
| Overall (Test vs single layer) | 21.6% | 32.9% | 35.5% | 16.3% |  | 27.2% | 31.5% | 16.9% | 28.5% | 33.5% |  |  |
| EL only (Test vs Ref) | -5.8% | -4.8% | -4.0% | -10.8% |  | -10.0% | -8.9% | -6.7% | -6.6% | -6.2% |  |  |
| Enc Time[%] | 121.4% |  |  | 111.9% |  |  |  | 116.5% |  |  |  |  |
| Dec Time[%] | 104.5% |  |  | 105.9% |  |  |  | 105.9% |  |  |  |  |
| BL Match | Matched |  |  |  | Matched |  |  | Matched |  |  |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |  | LD-B HEVC SNR |  |  |  |  |
|  | Y | U | V | Y |  | U | V | Y | U | V |  |  |
| Class A | -3.1% | -4.9% | -4.7% |  |  |  |  | -3.0% | -4.9% | -4.6% |  |  |
| Class B | -3.6% | -3.2% | -3.1% | -5.1% |  | -6.7% | -6.4% | -4.1% | -4.0% | -3.5% |  |  |
| Overall (Test vs Ref) | -3.5% | -3.7% | -3.5% | -5.1% |  | -6.7% | -6.4% | -3.8% | -4.3% | -3.8% |  |  |
| Overall (Test vs single layer) | 23.7% | 32.1% | 33.6% | 18.2% |  | 24.0% | 26.5% | 19.1% | 27.8% | 32.0% |  |  |
| EL only (Test vs Ref) | -6.0% | -6.2% | -5.9% | -12.1% |  | -13.4% | -13.1% | -6.1% | -6.6% | -6.1% |  |  |
| Enc Time[%] | 115.7% |  |  | 109.1% |  |  |  | 111.9% |  |  |  |  |
| Dec Time[%] | 108.6% |  |  | 111.8% |  |  |  | 109.1% |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  | Matched |  |  |  |  |

# Conclusions

This contribution proposed to constrain the usage of GRP to 2Nx2N merge PU. It is reported the BD-rate reduction of the proposed method are -1.7%/-2.6%/-2.1% for RA 2X/1.5X/SNR, -3.5%/-5.0%/-4.4% for LDP 2X/1.5X/SNR, and -3.5%/-5.1%/-3.8% for LDB 2X/1.5X/SNR.

# References

- T.-D. Chuang, S. Liu, M. Guo, P. Lai, Y.-W. Huang, S. Lei, “Non-TE3: Adaptive predictor compensation with generalized residual prediction”, JCTVC-L0074, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0078, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “Non-TE3: Extension of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0190, Geneva, Switzerland, 14–23 Jan. 2013.

- E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-L1009, Geneva, Switzerland, 14–23 Jan. 2013.

- W.	Zhu, O. Au, W.	Dai, X.	Zhang, H. Zhang, “A study of Generalized Residual Prediction”, JCTVC-M0157, Incheon, Korea, 18-26 April. 2012.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).