| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0156_r1 |
| --- | --- |

| Title: | AHG5: Simplification of chroma intra prediction process for YUV4:2:2 coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Shun-ichi Sekiguchi Tokumichi Murakami 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, a simplified scheme of YUV4:2:2 intra chroma prediction is proposed. The proposed scheme simplifies the current WD process for YUV4:2:2 chroma intra prediction by introducing a mode index conversion table for intra DM mode. The coding performance of the proposed scheme is comparable to that of the current HEVC Range Extension anchor.

# Introduction

In the current HEVC Range Extensions draft [1], the specification of intra angular prediction has been changed from HEVC version1 for YUV4:2:2 coding. This modification requires additional process in intra angular prediction only for YUV4:2:2 case. In order to make changes minimum relative to version1 specification, we propose to introduce a conversion table of intra mode index, which does not require any changes to intra prediction processing from version1.

# Proposed scheme

The current specification has Intra_DM mode, where both luma and chroma components utilize the same intra prediction mode. In the case that chroma format of input source is YUV4:2:2, as shown in Fig.1, the conversion of prediction angle is needed in order to predict using the same prediction angle for both luma and chroma components in YUV4:4:4 sampling domain.

Fig. 1. The problem of DM mode for YUV4:2:2 coding

Fig.2 illustrates the relationship on prediction direction between luma and chroma components with explicit equation form. For implementation of this chroma prediction, the current Range Extensions draft calculates new prediction angle that fulfills the relationship in Fig.2. For codec design considering compatibility with version 1 specification, it is preferable that this kind of new operation could be avoided as much as possible.

Fig. 2. Relationship on prediction direction between luma and chroma components

From this perspective, we propose a simpler solution to resolve this YUV4:2:2 specific issue, which converts the index of intra chroma prediction mode from luma mode index using a new conversion table. Table 1 shows the proposed conversion table of intra prediction mode for Intra_DM mode in YUV4:2:2. This table converts intra prediction mode indicated by Intra_DM mode, into relevant angular prediction mode that indicates the nearest approximated prediction direction vc shown in Fig.2. By using this table for Intra_DM mode in YUV4:2:2 coding, appropriate prediction angle conversion can be achieved without deriving new prediction angle.

Table 1. The conversion table of intra prediction mode for Intra_DM mode

| IntraPredModeY | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | 0 | 1 | 2 | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| IntraPredModeY | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| IntraPredModeC | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 26 | 26 | 26 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |  |

# Simulations & Results

The proposed scheme has been implemented on top of HM10.0_RExt2.0, and performance evaluation has been conducted using the common test configurations for HEVC Range Extensions[2] on intra only configurations. Table 2 shows the results of the coding performance of the proposed scheme compared with HEVC Range Extensions anchor. The results demonstrate that the introduction of the proposed scheme does not affect coding efficiency, while normative changes to chroma intra prediction process from version1 can be reduced.

Table 2. BD bitrate savings of the proposed scheme

|  | All Intra Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.1% | 0.1% | 0.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.1% | 0.1% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | -0.2% | -0.3% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | All Intra High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.1% | 0.1% | 0.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.1% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | -0.1% | -0.2% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | All Intra Super-High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | -0.1% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |

# Additional tests on combination with JCTVC-M0127

In JCTVC-M0127, some prediction angle conversion schemes are proposed in the same fashion as our proposal. These schemes also introduce the conversion table to refine chroma prediction direction for YUV4:2:2 case. As shown in Table 3, there are three proposed tables in JCTVC-M0127. In this section, we investigate three combined schemes that replace the conversion table in Table 1 with one of new ones in Table 3. The combined schemes have been implemented on top of HM10.0_RExt2.0, and coding simulations have been performed under the common test conditions for HEVC Range Extensions[2]. The test results are shown in Table 4. Given these results, it has been found that the coding performance of the combined schemes is comparable to that of three proposals in JCTVC-M0127.

Table 3. The conversion table proposed in JCTVC-M0127

(a) Proposal 1

| IntraPredModeY | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | 0 | 1 | 2 | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| IntraPredModeY | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| IntraPredModeC | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |  |

(b) Proposal 2

| IntraPredModeY | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 18 | 18 | 18 |
| IntraPredModeY | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| IntraPredModeC | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |  |

(c) Proposal 3

| IntraPredModeY | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 21 | 21 |
| IntraPredModeY | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| IntraPredModeC | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 30 |  |

Table 4. BD bitrate savings of the combined schemes

(a) Combination of Proposal 1

|  | All Intra Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | -0.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | -0.1% | -0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | -0.2% | -0.3% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | All Intra High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | -0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | -0.2% | -0.2% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | -0.1% | -0.1% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | All Intra Super-High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | -0.1% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

(b) Combination of Proposal 2

|  | All Intra Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | -0.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.1% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.2% | 0.1% | 0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.1% | 0.0% |
| Overall | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | All Intra High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.0% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.1% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | All Intra Super-High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.1% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | 0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.0% | 0.1% | 0.0% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

(c) Combination of Proposal 3

|  | All Intra Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | -0.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.1% | 0.0% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 0.1% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | All Intra High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 0.1% | 0.1% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | -0.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.1% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | All Intra Super-High-tier |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.0% | 0.0% | 0.0% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusions

This contribution has proposed a simplification of chroma intra prediction process for YUV4:2:2 coding case, by introducing a mode index conversion table for Intra_DM mode. Coding performance of the proposed scheme has been found to be comparable with that of the Range Extensions anchor. We recommend adoption of the proposed scheme into next version of HEVC Range Extensions draft.

# References

- David Flynn, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 2 (for PDAM)”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L1005, 12th Meeting: Geneva, CH, January, 2013.

- David Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L1006, 12th Meeting: Geneva, CH, January, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative descriptions to be reflected to WD text

A description of the proposed scheme is shown as follows.

- 8.4.3	Derivation process for chroma intra prediction mode

This process is only invoked when ChromaArrayType is not equal to 0.

Input to this process is a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb ][ yCb ] as specified in Table 8-1.

Table 8-1 – Specification of IntraPredModeC

| intra_chroma_pred_mode [ xCb ][ yCb ] | ChromaArrayType | IntraPredModeY[ xCb ][ yCb ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 26 | 10 | 1 | X ( 0 <= X <= 34 ) |
| 0 |  | 34 | 0 | 0 | 0 | 0 |
| 1 |  | 26 | 34 | 26 | 26 | 26 |
| 2 |  | 10 | 10 | 34 | 10 | 10 |
| 3 |  | 1 | 1 | 1 | 34 | 1 |
| 4 | Other than 2 | 0 | 26 | 10 | 1 | X |
| 4 | 2 | chrIdx[0] | chrIdx[26] | chrIdx[10] | chrIdx[1] | chrIdx[X] |

Table 8-3 – Specification of chrIdx[lumaIdx]

| lumaIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| chrIdx | 0 | 1 | 2 | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| lumaIdx | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |
| chrIdx | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 26 | 26 | 26 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |  |

...

- 8.4.4.2.6	Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Figure 8-1 illustrates the total 33 intra angles and Table 8-2 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Figure 8-1 – Intra prediction angle definition (informative)

Table 8-2 – Specification of intraPredAngle

| predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-3 further specifies the mapping table between predModeIntra and the inverse angle parameter invAngle.

Table 8-3 – Specification of invAngle

| predModeIntra | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntra | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The variables intraPredAngleAdj and invAngleAdj are derived as follows:

–	When ChromaArrayType is equal to 2 and cIdx is not equal to 0, the following applies:

- If intraPredMode is greater than 17, intraPredAngleAdj is set equal to intraPredAngle  >>  1) and invAngleAdj is set equal to invAngle  <<  1,

- Otherwise (intraPredMode is less than 18), the variables intraPredAngleAdj and invAngleAdj are derived as follows:

  - If Abs( intraPredAngle ) is greater than 16, intraPredAngleAdj is set equal to Sign( intraPredAngle ) * 32 and invAngleAdj is set equal to −256,

  - Otherwise (Abs( intraPredAngle ) is less than 17), intraPredAngleAdj is  set equal to intraPredAngle  <<  1 and when nTbS * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle  >>  1.

–	Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), intraPredAngleAdj is set equal to intraPredAngle and when nT * intraPredAngle  >>  5 is less than −1, invAngleAdj is set equal to invAngle.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nTbS	(8-1)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngleAdj + 128 )  >>  8 ) ],
	with x = −1..( nTbS * intraPredAngleAdj )  >>  5	(8-2)

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS + 1..2 * nTbS	(8-3)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngleAdj )  >>  5	(8-4)

iFact = ( ( y + 1 ) * intraPredAngleAdj ) & 31	(8-5)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-6)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-7)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nTbS is less than 32, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-8)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nTbS	(8-9)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-10)

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS + 1..2 * nTbS	(8-11)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-12)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-13)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-14)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-15)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nTbS is less than 32, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-16)