| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0157 |
| --- | --- |

| Title: | Study of Generalized Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wenjing Zhu Oscar Au Wei Dai Xingyu Zhang Hong Zhang | Email: | wzhuaa@ust.hk weidai@ust.hk hzhangad@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

Generalized Residual Prediction is a predictive coding tool which utilizes collocated up-sampled BL signal (BL_col) and BL up-sampled temporal prediction signal (BL_ref) referenced using the same EL motion information. Except first order prediction of EL temporal prediction signal (EL_ref), BL_col and BL_ref provide second order prediction. In this contribution, different kinds of GRP weight are studied and gain of different GRP modes and different combinations of GRP mode are provided.

# Introduction

Many contributions study the performance of GRP, it indeed gives much gain, In SCE3, GRP is studied, in GRP mode, the final EL prediction signal combine 3 signal, EL_ref, BL_ref and BL_col. In a general form, the EL prediction signal can be written as follow (in uni-directional case):

EL_Pred = w1*EL_ref + w2*BL_ref + w3*BL_col, subject to w1+w2+w3 = 1

This document provides performance of different GRP mode and different combination of GRP mode.

# Technical details

To study performance of different GRP modes, we design a simulation. In the simulation, every PU can choose his own GRP weight (w1, w2 and w3) according to SATD.

Fix dynamic range of w1, w2 and w3 to [-1, 1], let the step size of w1 and w2 be 0.1, w1 and w2 change independently, after w1 and w2 are decided, w3 can be obtained easily through equation w1+w2+w3 = 1.

In the simulation, w1 and w2 have 21 possible values, but some values are illegal because the absolute value of w3 will be larger than 1. After occluding the illegal case, the simulation provides 231 kinds of weight. If w1 = 1, w2 =0, w3 = 0, it is normal temporal prediction, so normal temporal prediction is treated as one GRP mode.

For every PU, try all kinds of weight, compare the SATD, and then decide the best GRP weight. The GRP mode index is not written into bitstream, so encoder and decoder are mismatch.

Run the simulation in LDP case, collect GRP weight of every PU, and then calculate every GRP weight’s frequency, the result shows that Res-0(normal temporal prediction), Res-1, Res-0.5 and Diff0.5 are top 4 most frequent kinds of GRP mode chosen.

After obtaining the best GRP modes, test every GRP modes and every kind of combination. In these tests, GRP only happens on 2Nx2N PU, each PU selects the GRP mode according to RD performance, mode index is also not written into bitstream, so signalling bits of mode index don’t have any impact on RD performance, only the selection only takes account to accuracy of prediction, the most accurate GRP mode will be chosen. The performance without signalling bits is the upper bound gain we can get with these GRP modes.

# Test Results

Performance of Res-1 with respect to SHM 1.0

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.8% | -4.7% | -4.6% |  |  |  | -3.3% | -4.2% | -4.4% |
| Class B | -4.0% | -2.4% | -2.4% | -4.8% | -4.7% | -4.6% | -3.4% | -3.4% | -4.0% |
| Overall (EL+BL) | -3.9% | -3.1% | -3.0% | -4.8% | -4.7% | -4.6% | -2.9% | -3.8% | -4.2% |
| Overall (EL) | -6.2% | -5.3% | -5.2% | -9.3% | -9.0% | -8.8% | -5.3% | -5.6% | -6.1% |
| Enc Time[%] | 107.0% |  |  | 103.3% |  |  | 107.6% |  |  |
| Dec Time[%] | 88.5% |  |  | 102.3% |  |  | 100.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Performance of Res-0.5 with respect to SHM 1.0

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -4.8% | -4.4% | -3.8% |  |  |  | -4.6% | -4.3% | -4.2 |
| Class B | -4.6% | -2.2% | -1.3% | -4.9% | -3.4% | -2.1% | -4.9% | -4.4% | -4.4% |
| Overall (EL+BL) | -4.6% | -2.8% | -2.0% | -4.9% | -3.4% | -2.1% | -4.8% | -4.4% | -4.3% |
| Overall (EL) | -4.1% | -5.4% | -5.5% | -7.7% | -6.0% | -4.8% | -6.9% | -6.4% | -6.4% |
| Enc Time[%] | 110.1% |  |  | 109.0% |  |  | 109.3% |  |  |
| Dec Time[%] | 87.9% |  |  | 98.6% |  |  | 98.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Performance of Diff-0.5 with respect to SHM 1.0

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -4.5% | -3.5% | -3.2% |  |  |  | -5.0% | -4.4% | -4.0% |
| Class B | -5.0% | -0.6% | -0.0% | -6.1% | -2.8% | -2.6% | -5.7% | -2.8% | -3.1% |
| Overall (EL+BL) | -4.9% | -1.4% | -0.9% | -6.1% | -2.8% | -2.6% | -5.5% | -3.3% | -3.4% |
| Overall (EL) | -8.0% | -4.4% | -3.8% | -12.8% | -9.0% | -8.6% | -8.9% | -6.5% | -6.5% |
| Enc Time[%] | 117.4% |  |  | 103.3% |  |  | 106.6% |  |  |
| Dec Time[%] | 102.5% |  |  | 109.8% |  |  | 103.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

More test results are in the excel attached with this document.

# Patent rights declaration(s)

Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).