| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0157 |
| --- | --- |

| Title: | Study of Generalized Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wenjing Zhu Oscar Au Wei Dai Xingyu Zhang Hong Zhang | Email: | wzhuaa@ust.hk weidai@ust.hk hzhangad@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

Generalized Residual Prediction is a predictive coding tool which utilizes collocated up-sampled BL signal (BL_col) and BL up-sampled temporal prediction signal (BL_ref) referenced using the same EL motion information. Except first order prediction of EL temporal prediction signal (EL_ref), BL_col and BL_ref provide second order prediction. In this contribution, different kinds of GRP weight are studied and gain of different GRP modes and different combinations of GRP mode are provided.

# Introduction

Many contributions study the performance of GRP, it indeed gives much gain, In SCE3, GRP is studied, when utilizing GRP mode, the final EL prediction signal combine 3 signal, EL_ref, BL_ref and BL_col. In a general form, the EL prediction signal can be written as follow (in uni-directional case):

EL_Pred = w1*EL_ref + w2*BL_ref + w3*BL_col, subject to w1+w2+w3 = 1

This document provides performance of different GRP modes and different combination of GRP modes.

# Technical details

To study performance of different GRP modes, we design a simulation. In the simulation, every PU can choose a set of GRP weighting factor (w1, w2 and w3) according to SATD.

Fix dynamic range of w1, w2 and w3 to [-1, 1], let the step size of w1 and w2 be 0.1, w1 and w2 can change independently, after w1 and w2 are decided, w3 can be obtained easily through equation w1+w2+w3 = 1.

In the simulation, w1 and w2 have 21 possible values, but some values are illegal because the absolute value of w3 will be larger than 1. After occluding the illegal case, the simulation provides 231 sets of weighting factor. If w1 = 1, w2 =0, w3 = 0, it is normal temporal prediction, so normal temporal prediction is treated as one set of GRP weighting factor.

For every PU, try all sets of weighting factor, compare the SATD, and then decide the best GRP weighting factor. The target of this simulation is to show distribution of the weight factor, and then find the most frequent GRP weighting factor, so GRP mode index is not written into bitstream, encoder and decoder are mismatch.

Run the simulation in LDP case, collect GRP weighting factor of every PU, and then calculate every GRP weighting factor’s frequency, the result is that (1, 0, 0), (1, -1, 1), (1, -0.5, 0.5), (0.5, -0.5, 1) are top 4 most frequent GRP weighting factor chosen. The four sets of GRP weighting factor represent different ways to modify prediction signal of enhancement layer. In SCE3, the GRP modes proposed correspond to different sets of weighting factor, the four sets of GRP weighting factor have seen proposed as different GRP modes. Table 1 shows the correspondence between the top 4 sets of GRP weighting factor and GRP modes proposed.

Table 1 the top 4 sets of weighting factor and corresponding GRP modes

| (w1, w2, w3) | Prediction signal | Inter prediction mode |
| --- | --- | --- |
| (1, 0, 0) | EL_pred = EL_ref | Res-0 |
| (1, -1, 1) | EL_pred = EL_ref + (BL_col – BL_ref) | Res-1 |
| (1, -0.5, 0.5) | EL_pred = EL_ref + 0.5 (BL_col – BL_ref) | Res-0.5 |
| (0.5, -0.5, 1) | EL_pred = BL_col + 0.5 (EL_ref – BL_ref) | Diff-0.5 |

After obtaining the best GRP modes, test every GRP modes and every kind of combination. In these tests, GRP only happens on 2Nx2N PU, each PU selects the GRP mode according to RD performance, mode index is also not written into bitstream, so signalling bits of mode index don’t have any impact on RD performance, the selection only takes account of accuracy of prediction, the most accurate GRP mode will be chosen. The performance without signalling bits is the upper bound gain we can get with these GRP modes.

Additionally, in this simulation, there is also a set of weighting factor (-1, 1, 1) which be chosen frequently, the reason may be that the false high-frequency component of collocated up-sampled BL signal caused by aliasing should be subtracted, so in performance comparison test, this set of weighting factor called DiffM1 is also tested.

# Test Results

Performance of Res-1 with respect to SHM 1.0

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.8% | -4.7% | -4.6% |  |  |  | -3.3% | -4.2% | -4.4% |
| Class B | -4.0% | -2.4% | -2.4% | -4.8% | -4.7% | -4.6% | -3.4% | -3.4% | -4.0% |
| Overall (EL+BL) | -3.9% | -3.1% | -3.0% | -4.8% | -4.7% | -4.6% | -2.9% | -3.8% | -4.2% |
| Overall (EL) | -6.2% | -5.3% | -5.2% | -9.3% | -9.0% | -8.8% | -5.3% | -5.6% | -6.1% |
| Enc Time[%] | 107.0% |  |  | 103.3% |  |  | 107.6% |  |  |
| Dec Time[%] | 88.5% |  |  | 102.3% |  |  | 100.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Performance of Res-0.5 with respect to SHM 1.0

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -4.8% | -4.4% | -3.8% |  |  |  | -4.6% | -4.3% | -4.2 |
| Class B | -4.6% | -2.2% | -1.3% | -4.9% | -3.4% | -2.1% | -4.9% | -4.4% | -4.4% |
| Overall (EL+BL) | -4.6% | -2.8% | -2.0% | -4.9% | -3.4% | -2.1% | -4.8% | -4.4% | -4.3% |
| Overall (EL) | -4.1% | -5.4% | -5.5% | -7.7% | -6.0% | -4.8% | -6.9% | -6.4% | -6.4% |
| Enc Time[%] | 110.1% |  |  | 109.0% |  |  | 109.3% |  |  |
| Dec Time[%] | 87.9% |  |  | 98.6% |  |  | 98.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Performance of Diff-0.5 with respect to SHM 1.0

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -4.5% | -3.5% | -3.2% |  |  |  | -5.0% | -4.4% | -4.0% |
| Class B | -5.0% | -0.6% | -0.0% | -6.1% | -2.8% | -2.6% | -5.7% | -2.8% | -3.1% |
| Overall (EL+BL) | -4.9% | -1.4% | -0.9% | -6.1% | -2.8% | -2.6% | -5.5% | -3.3% | -3.4% |
| Overall (EL) | -8.0% | -4.4% | -3.8% | -12.8% | -9.0% | -8.6% | -8.9% | -6.5% | -6.5% |
| Enc Time[%] | 117.4% |  |  | 103.3% |  |  | 106.6% |  |  |
| Dec Time[%] | 102.5% |  |  | 109.8% |  |  | 103.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

More test results are in the excel attached with this document.

# Patent rights declaration(s)

Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).