| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0158 |
| --- | --- |

| Title: | SCE1-1.3 Differential coding of enhancement layer intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Done Bugdayci, Kemal Ugur Visiokatu 3, Tampere, Finland | Email: | ext-done.bugdayci@nokia.com; kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents an algorithm for the scalable extension of High Efﬁciency Video Coding (HEVC) standard, where the enhancement layer intra mode is coded using base layer intra mode information. We propose to replace the MPM list based coding of intra prediction mode with a differential coding approach. The difference between the intra prediction modes of base and enhancement layer blocks is taken as the value to be coded which ranges from -34 to +34. Various syntax elements are defined to code this difference. In order to avoid parsing dependency on base layer, Mode Dependent Coefficient Scanning (MDCS) is disabled and scanning pattern is set to be diagonal always. The tests show overall 0.2% luma gain in 2x scalability and 0.2% luma gain in 1.5x scalability in All Intra configuration of the test conditions.

# Introduction

In scalable extension of HEVC, coding efficiency of enhancement layer(s) can be improved by utilizing base layer intra prediction mode information during coding of enhancement layer intra mode due to the high correlation between the layers. This proposal aims to exploit this correlation by coding only the difference of the base and enhancement layer intra prediction modes. The statistics suggests that the most probable value of the difference is 0 followed by ±1 and ±2. The rest of the possible values have much smaller probability which may be approximated as equally distributed.

# Algorithm Description

Intra prediction mode information of enhancement layer is coded with respect to base layer. The difference between the intra prediction modes of base and enhancement layer blocks is taken as the value to be coded which ranges from -34 to +34. The difference is represented with a sign and a magnitude component and various syntax elements are deﬁned as follows in order to efﬁciently code this value:

- A binary valued syntax element indicating whether the difference is 0 or not is deﬁned as luma_intra_pred_equal_base_ﬂag.

- If luma_intra_pred_equal_base_ﬂag is equal to 0, a binary valued syntax element indicating whether the absolute value of the difference is equal to 1 or not is deﬁned as luma_intra_pred_diff_abs_equal1_ﬂag.

- If luma_intra_pred_diff_abs_equal1_ﬂag is equal to 0, a binary valued syntax element indicating whether the absolute value of the difference is equal to 2 or not is deﬁned as luma_intra_pred_diff_abs_equal2_ﬂag.

- A binary valued syntax element indicating the sign of the difference is deﬁned as luma_intra pred_diff_sign_ﬂag. Note that this syntax element is only present if luma_intra_pred_equal_base_ﬂag takes a value of 0.

- If luma_intra_pred_diff_abs_equal2_ﬂag is not equal to 1, the absolute value of the difference minus 3 (which corresponds to the remaining 32 different values ranging from 2 to 34) is coded using luma_intra_pred_diff_abs_remaining.

Syntax element luma_intra_pred_diff_abs_remaining is binarized using Fixed-Length (FL) binarization scheme of CABAC. Only the syntax element luma_intra_pred_equal_base_ﬂag is coded with context, whereas all the remaining syntax elements are coded in bypass mode. Table 1 indicates the syntax changes related to the proposed algorithm. Note that, during the implementation of the above algorithm in the provided software, flag values were taken in reverse (e.g. if the first bin is 0 then luma intra prediction mode is equal to base luma intra prediction mode, etc.) which makes no difference in the performance.

Table 1 Syntax changes related to the proposed algorithm

} else if( PredMode = = MODE_INTRA ) {

if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize )

pcm_flag

if( pcm_flag ) {

num_subsequent_pcm

NumPCMBlock = num_subsequent_pcm + 1

while( !byte_aligned( ) )

pcm_alignment_zero_bit

pcm_sample( x0, y0, log2CbSize )

} else {

prev_intra_luma_pred_flag[ x0 ][ y0 ]

if( prev_intra_luma_pred_flag[ x0 ][ y0 ] )

mpm_idx[ x0 ][ y0 ]

else

rem_intra_luma_pred_mode[ x0 ][ y0 ]

luma_intra_pred_equal_base_flag[x0 + i][y0 + j ]

if( !luma_intra_pred_equal_base_flag [ x0 + i ][ y0+ j ] ) {

luma_intra_pred_diff_abs_equal1 _flag[ x0 + i ][ y0 + j]

if( luma_intra_pred_diff_abs_level_equal1_flag [ x0 + i ][ y0+ j ] ) {

luma_intra_pred_diff_abs_equal2_flag[ x0 + i ][ y0 + j ]

}

luma_intra_pred_diff_sign_flag[ x0 + i ][ y0 + j ]

if( luma_intra_pred_diff_abs_level_equal2_flag [ x0 + i ][ y0+ j ] ) {

luma_intra_pred_diff_abs_remaining[ x0 + i ][ y0 + j ]

}

}

intra_chroma_pred_mode[ x0 ][ y0 ]

}

} else { /* MODE_INTER */

# Simulation Results

Test Sequences, bitrates and coding conditions used in the simulations are in conformance with JCTVC-L1009 and JCTVC-L1101. Anchor software is SHM1.0. Below, complexity and RD performance results are provided for the case when MDCS is disabled. Additional information for the case with MDCS is enabled can be found in the excel sheets.

Table 2 All Intra test results with accurate timing information when MDCS is disabled

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.0% |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.2% | 13.5% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.4% | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% |
| Enc Time[%] | 101.5% |  |  | 100.5% |  |  |
| Dec Time[%] | 98.9% |  |  | 100.5% |  |  |

# Conclusion

This contribution presents a method for coding the intra mode of the enhancement layer using the base layer intra mode information. The proposed method takes the difference between base and enhancement layer intra modes and codes the difference, instead of using the MPM approach. As MPM process is not utilized, intra modes of enhancement layer need not to be stored. In addition, the MPM derivation process is not used, further reducing the complexity.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).