| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0164 |
| --- | --- |

| Title: | Layer identifier extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

To support more than 64 layers, conforming to the current HEVC spec., we propose the layer identifier extension solution. In NAL unit headers, three bits are allocated for nuh_temporal_id_plus1. Forcing all NAL units of all layers into having the same TemporalId in an access unit, the nuh_temporal_id_plus1of non-base layers can be interpreted as extra bits of nuh_layer_id to represent additional layers. The TemporalID of layers with non-zero nuh_layer_id is inferred from the TemporalID of the base layer. Approximately 500 layers can be represented by the proposed solution.

# Introduction

In the HEVC and its extension specification texts, the bit length of the nuh_layer_id, is fixed to 6 bits. Thus, the maximum number of layers is 64. However, in super multi-view applications or other high-end devices, more than 64 layers may be needed. To support more than 64 layers, conforming to the current HEVC spec., we propose the layer identifier extension solution. In NAL unit headers, three bits are allocated for nuh_temporal_id_plus1. Forcing all layers into having the same TemporalId in an access unit, the nuh_temporal_id_plus1of non-base layers can be interpreted for other purposes instead of TemporalID. Simply, the nuh_temporal_id_plus1 can be interpreted as extra bits of nuh_layer_id. It extends the bit length of the generic layer identifier to 9 bits. Then, about 500 layers can be represented.

To enable this, a layer_id_extension_flag, is proposed. layer_id_extension_flag equal to 1 indicates that TemporalIDs of all layers have the same value. The TemporalID of layers with non-zero nuh_layer_id is inferred from the TemporalID of the base layer. Then, nuh_temporal_id_plus1 is used as extra bits of nuh_layer_id, as given by the proposed text.

In other words, the nuh_temporal_id_plus1 specifies generic sub-layer identifiers, where the sub-layer indicates not only temporal layer but also other scalability layers.

# Proposed specification text

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| layer_id_extension_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i < MaxNumLayer; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) { |  |
| if(layer_id_extension_flag) |  |
| layer_id_in_nuh [ i ] | u(9) |
| else |  |
| layer_id_in_nuh[ i ] | u(6) |
| } |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= ExtMaxLayerId; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| for( i = 1; i < MaxNumLayers; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

layer_id_extension_flag equal to 1 indicates that the TemporalID of layers with non-zero nuh_layer_id is equal to the TemporalID of the base layer of which nuh_layer_id is equal to 0.

Note: if layer_id_extension_flag is equal to 1, nuh_temporal_id_plus1 is used as extra bits of nuh_layer_id.

Where ExtLayerId indicates the extended generic layer identifier, the variable ExtLayerId is specified as follows:

if(layer_id_extension_flag && nuh_layer_id > 0)

- ExtLayerId = (nuh_layer_id-1)*(1<<3 -1) + nuh_temporal_id_plus1

else

ExtLayerId = nuh_layer_id

The variable MaxNumLayers is specified as follows:

if(layer_id_extension_flag)

MaxNumLayers = (vps_max_layers_minus1) *(1<<3 -1) + 1;

else

MaxNumLayers = vps_max_layers_minus1 + 1;

The variable ExtMaxLayerId is specified as follows:

if(layer_id_extension_flag)

ExtMaxLayerId = vps_max_layer_id *(1<<3 -1)

else

- ExtMaxLayerId = vps_max_layer_id

The variable TemporalId is specified as follows:

if(nuh_layer_id == 0)

- TemporalId = nuh_temporal_id_plus1 − 1

else

TemporalId = nuh_temporal_id_plus1 of the base layer NAL unit of which nuh_layer_id is equal to 0 in the same access unit – 1

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).