| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0165 |
| --- | --- |

| Title: | On parameter set prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In order to remove duplicated syntax signalling between sequence parameter sets (SPSs) in multi-layered extension of HEVC, three approaches are proposed; SPS-to-SPS prediction approach, VPS-to-SPS prediction approach, and a hybrid design enabling both approaches operationally. The proposed solution for SPS-to-SPS prediction is aligned to JCTVC-L0137 with minor changes, and the proposed solution for VPS-to-SPS prediction signals the common parts of SPS in a VPS once and each SPS refers to the VPS for parameter prediction. We assert that containing common syntaxes of SPSs of multi-layers in VPS extension is beneficial to inform the structure of coded video sequences for session negotiation and fast player and decoder set-up, as well as parameter set compression.

# Introduction

The proposal is a flexible design to remove duplicated syntax signalling between sequence parameter sets (SPSs) in multi-layered extension of HEVC. Since many SPS syntax elements may have identical values across SPSs, signalling those parameters independently is redundant.

In the last Geneva meeting, two proposals were related to this issue: JCTVC-L0137 proposed a solution of inter-SPS prediction, while JCTVC-L0132 proposed a hybrid solution which combines an inter-SPS prediction with VPS-to-SPS prediction.

In this proposal, as a follow-up proposal of JCTVC-L0132, partially agreeing to the design criteria of video video parameter set (VPS) and SPS in JCTVC-L0137, we assert that containing common syntaxes of SPSs of multi-layers in VPS extension is beneficial to inform the structure of coded video sequences for session negotiation and content notification, as well as parameter set compression.

Commonly, SPS contains the parameters related to sub-layer ordering, video format, coding tools, and reference picture set (RPS). Among them, the syntaxes related to sub-layer ordering is signalled in VPS in HEVC v1, expecting the parameters will be used for session negotiation. Also, experts agreed that parameters related to video format in SPS are helpful for session negotiation and expected to be signalled in VPS extension. RPS and other coding tool related parameters can be utilized for system set-up. Once those common parts of SPS via VPS are delivered, the systems receiving bit streams can easily set up the player and decoder, rather than parsing all layers’ SPS. Optionally signalling the common parts of SPS would be desirable.

In this contribution, three approaches are proposed; SPS-to-SPS prediction approach, VPS-to-SPS prediction approach, and a hybrid design enabling both approaches operationally. The details of the proposed solution are presented in the following section.

# Proposed parameter set prediction approach

In this contribution, two flags, vps_sps_common_syntax_present_flag and vps_sps_interlayer_prediction_enable_flag, are proposed to indicate whether the common syntaxes of SPSs in multi-layers are signalled only in VPS once, and whether the syntaxes of a specific SPS is predicted from a directly referred SPS, respectively.

If vps_sps_common_syntax_present_flag is equal to 1, the common syntaxes of sequence paremter sets of enhancement layers are signalled once in a video parameter set. The common SPS syntaxes of enhancement layers are prediced from the video paramter set.

If vps_sps_interlayer_prediction_enable_flag is equal to 1, the common syntaxes of the sequence parameter sets of enhancement layers can be predicted from the sequence parameter sets of the directly referred layers.

Otherwise, the common syntaxes of SPS are signalled in each SPS independently without prediction.

When vps_sps_interlayer_prediction_enable_flag is equal to 1, each SPS of a layer can be predicted from a directly referred layer, which is specified by direct_dependency_flag. If multiple layers are directly referred layers with vps_sps_interlayer_prediction_enable_flag equal to 1, which layer of them is used for the SPS prediction for the current SPS is signaled in the SPS by indexes.

If vps_sps_common_syntax_present_flag is equal to 1, the common syntaxes of SPS for video format, coding tool, and reference picture sets are signaled in VPS. Those syntaxes can be used for session negotiation or video contents information delivery, as well as the reference by SPSs of multi-layers.

The common syntaxes of SPS are categorized as four groups; 1. Video format related parameters, 2. Sub-layer ordering related parameters, 3. Coding tool related parameters, 4. Reference picture set related parameters.

Three flags indicate whether those groups are signaled in VPS or not, respectively. In VPS syntax of HEVC, the sub-layer ordering related parameters are already signaled. Thus, there is no need to additionally signal the sub-layer ordering related parameters in VPS extension. vps_video_format_info_present_flag indicates whether video format related parameters are signalled in VPS. vps_coding_tool_info_present_flag indicates whether coding tools related parameters are signalled in VPS. vps_reference_picture_set_info_present_flag indicates whether reference picture set related parameters are signalled in VPS. If the three flags are not present, the values of three flags are inferred as equal to 0 as default.

In each SPS, sps_parameter_prediction_flag is signaled to indicate whether the common parameters of SPS are signaled in the current SPS, or predicted from the associated VPS or other directly referred SPSs.

sps_video_format_info_ref_layer_idx specifies which SPS or VPS is used for the prediction of video format related parameters. If sps_video_format_info_ref_layer_idx is equal to 0, the video format related parameters are signaled in the current SPS without syntax prediction. If sps_video_format_info_ref_layer_idx is equal to 1, the video format related parameters are predicted from the associated VPS, without signaling the video format related syntax in the current SPS. Otherwise, sps_video_format_info_ref_layer_idx indicates which SPS is used as the reference for the video format related syntax prediction. The video format related parameters in the active SPS of the (sps_video_format_info_ref_layer_idx – 2) th directly referred layer are referred for the syntax prediction of the current SPS.

sps_sub_layer_ordering_info_ref_layer_idx specifies which SPS or VPS is used for the prediction of sub-layer ordering related parameters. If sps_sub_layer_ordering_info_ref_layer_idx is equal to 0, the sub-layer ordering related parameters are signaled in the current SPS without syntax prediction. If sps_sub_layer_ordering_info_ref_layer_idx is equal to 1, the sub-layer ordering related parameters are predicted from the associated VPS, without signaling the sub-layer ordering related syntax in the current SPS. Otherwise, sps_sub_layer_ordering_info_ref_layer_idx indicates which SPS is used as the reference for the sub-layer ordering related syntax prediction. The sub-layer ordering related parameters in the active SPS of the (sps_sub_layer_ordering_info_ref_layer_idx – 2) th directly referred layer are referred for the syntax prediction of the current SPS.

sps_coding_tool_info_ref_layer_idx specifies which SPS or VPS is used for the prediction of coding tool related parameters. If sps_coding_tool_info_ref_layer_idx is equal to 0, the coding tool related parameters are signaled in the current SPS without syntax prediction. If sps_coding_tool_info_ref_layer_idx is equal to 1, the coding tool related parameters are predicted from the associated VPS, without signaling the coding tool related syntax in the current SPS. Otherwise, sps_coding_tool_info_ref_layer_idx indicates which SPS is used as the reference for the coding tool related syntax prediction. The coding tool related parameters in the active SPS of the (sps_coding_tool_info_ref_layer_idx – 2) th directly referred layer are referred for the syntax prediction of the current SPS.

sps_reference_picture_set_info_ref_layer_idx specifies which SPS or VPS is used for the prediction of reference picture set related parameters. If sps_ reference_picture_set _info_ref_layer_idx is equal to 0, the reference picture set related parameters are signaled in the current SPS without syntax prediction. If sps_reference_picture_set_info_ref_layer_idx is equal to 1, the reference picture set related parameters are predicted from the associated VPS, without signaling the reference picture set related syntax in the current SPS. Otherwise, sps_reference_picture_set _info_ref_layer_idx indicates which SPS is used as the reference for the reference picture set related syntax prediction. The reference picture set related parameters in the active SPS of the (sps_ reference_picture_set _info_ref_layer_idx – 2) th directly referred layer are referred for the syntax prediction of the current SPS.

# Specification text

In the proposed specification text, the modified parts are highlighted by Yellow for VPS-to-SPS, Green for SPS-to-SPS, and Sky for common or hybrid.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( lsIdx = 1; lsIdx <= vps_num_layer_sets_minus1; lsIdx ++ ) { |  |
| vps_profile_present_flag[ lsIdx ] | u(1) |
| if( !vps_profile_present_flag[ lsIdx ] ) |  |
| profile_layer_set_ref_minus1[ lsIdx ] | ue(v) |
| profile_tier_level( vps_profile_present_flag[ lsIdx ], vps_max_sub_layers_minus1) |  |
| } |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| vps_sps_interlayer_prediction_enable_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if( direct_dependency_flag[ i ][ j ] && vps_sps_interlayer_prediction_enable_flag) |  |
| parameter_set_prediction_flag[ i ][ j ] | u(1) |
| } |  |
| vps_sps_common_syntax_present_flag | u(1) |
| if(vps_sps_common_syntax_present_flag) { |  |
| vps_video_format_info_present_flag | u(1) |
| vps_coding_tool_info_present_flag | u(1) |
| vps_reference_picture_set_info_present_flag | u(1) |
| if(vps_video_format_info_present_flag) { |  |
| if(vps_chroma_format_idc = = 3 ) |  |
| vps_separate_colour_plane_flag | u(1) |
| vps_pic_width_in_luma_samples | ue(v) |
| vps_pic_height_in_luma_samples | ue(v) |
| vps_conformance_window_flag | u(1) |
| if(vps_conformance_window_flag ) { |  |
| vps_conf_win_left_offset | ue(v) |
| vps_conf_win_right_offset | ue(v) |
| vps_conf_win_top_offset | ue(v) |
| vps_conf_win_bottom_offset | ue(v) |
| } |  |
| vps_bit_depth_luma_minus8 | ue(v) |
| vps_bit_depth_chroma_minus8 | ue(v) |
| } |  |
| if(vps_coding_tool_info_present_flag) { |  |
| vps_log2_min_luma_coding_block_size_minus3 | ue(v) |
| vps_log2_diff_max_min_luma_coding_block_size | ue(v) |
| vps_log2_min_transform_block_size_minus2 | ue(v) |
| vps_log2_diff_max_min_transform_block_size | ue(v) |
| vps_max_transform_hierarchy_depth_inter | ue(v) |
| vps_max_transform_hierarchy_depth_intra | ue(v) |
| vps_scaling_list_enable_flag | u(1) |
| if(vps_scaling_list_enable_flag ) { |  |
| vps_scaling_list_data_present_flag | u(1) |
| if( vps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| vps_amp_enabled_flag | u(1) |
| vps_sample_adaptive_offset_enabled_flag | u(1) |
| vps_pcm_enabled_flag | u(1) |
| if(vps_pcm_enabled_flag ) { |  |
| vps_pcm_sample_bit_depth_luma_minus1 | u(4) |
| vps_pcm_sample_bit_depth_chroma_minus1 | u(4) |
| vps_log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| vps_log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| vps_pcm_loop_filter_disable_flag | u(1) |
| } |  |
| vps_temporal_mvp_enable_flag | u(1) |
| vps_strong_intra_smoothing_enable_flag | u(1) |
| } |  |
| if(vps_reference_picture_set_info_flag) { |  |
| vps_num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < vps_num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| vps_long_term_ref_pics_present_flag | u(1) |
| if(vps_ long_term_ref_pics_present_flag ) { |  |
| vps_num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < vps_num_long_term_ref_pics_sps; i++ ) { |  |
| vps_lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| vps_used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| log2_min_luma_coding_block_size_minus3 |  |
| log2_diff_max_min_luma_coding_block_size |  |
| log2_min_transform_block_size_minus2 |  |
| log2_diff_max_min_transform_block_size |  |
| max_transform_hierarchy_depth_inter |  |
| max_transform_hierarchy_depth_intra |  |
| scaling_list_enable_flag |  |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag |  |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag |  |
| sample_adaptive_offset_enabled_flag |  |
| pcm_enabled_flag |  |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 |  |
| pcm_sample_bit_depth_chroma_minus1 |  |
| log2_min_pcm_luma_coding_block_size_minus3 |  |
| log2_diff_max_min_pcm_luma_coding_block_size |  |
| pcm_loop_filter_disable_flag |  |
| } |  |
| } |  |
| } |  |

vps_sps_interlayer_prediction_enable_flag equal to 1 indicates the common syntaxes of the sequence parameter sets of enhancement layers can be predicted from the sequence parameter sets of the directly referred layers. vps_sps_interlayer_prediction_enable_flag equal to 0 specifies the common syntaxes of SPSs are signalled in each SPS independently without inter-layer prediction.

parameter_set_prediction_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a reference layer for the parameter set prediction of the layer with index i. parameter_set_prediction_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a reference layer for the parameter set prediction of the layer with index i. When parameter_set_prediction_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

vps_sps_common_syntax_present_flag equal to 1 specifies the common syntaxes of sequence paremter sets of enhancement layers, of which layer_id is not equal to 0, are signalled once in a video parameter set. vps_sps_common_syntax_present_flag equal to 0 specifies the common syntaxes of sequence parameter sets are signalled in each SPS independently.

vps_video_format_info_present_flag equal to 1 indicates that the syntaxes related to video format are signalled in the current VPS. If vps_video_format_info_present_flag is not present, it is inferred to be equal to 0.

vps_coding_tool_info_present_flag equal to 1 indicates that the syntaxes related to coding tools are signalled in the current VPS. If vps_coding_tool_info_present_flag is not present, it is inferred to be equal to 0.

vps_reference_picture_set_info_present_flag equal to 1 indicates that the syntaxes related to reference picture set are signalled in the current VPS. If vps_reference_picture_set_info_present_flag is not present, it is inferred to be equal to 0.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 ) { |  |
| sps_parameter_prediction_flag | u(1) |
| if(sps_parameter_prediction_flag) { |  |
| sps_video_format_info_ref_layer_idx | ue(v) |
| sps_ sub_layer_ordering_info_ref_layer_idx | ue(v) |
| sps_coding_tool_info_ref_layer_idx | ue(v) |
| sps_reference_picture_set_info_ref_layer_idx | ue(v) |
| } |  |
| } |  |
| if(sps_video_format_info_ref_layer_idx = = 0) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( (sps_sub_layer_ordering_info_present_flag && !sps_ sub_layer_ordering_info_ref_layer_idx) ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| if(sps_coding_tool_info_ref_layer_idx = = 0) { |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| } |  |
| if(sps_reference_picture_set_info_ref_layer_idx = = 0) { |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if(sps_coding_tool_info_ref_layer_idx = = 0 ) { |  |
| sps_temporal_mvp_enable_flag | u(1) |
| strong_intra_smoothing_enable_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_parameter_prediction_flag equal to 1 indicates that the common parameters of SPS are not signaled, and those parameters are predicted from the associated VPS or other SPSs. sps_parameter_prediction_flag equal to 0 indicates that the common parameters of SPS are signaled in the current SPS.

sps_video_format_info_ref_layer_idx equal to 0 specifies that the video format related parameters are signaled in the current SPS without parameter prediction. sps_video_format_info_ref_layer_idx equal to 1 specifies that the video format related parameters are predicted from the associated VPS, without signaling the video format related syntax in the current SPS. Otherwise, sps_video_format_info_ref_layer_idx indicates that the parameters related to video format are inferred from the parameters related to video format in the active SPS of the (sps_video_format_info_ref_layer_idx – 2) th directly referred layer.

sps_sub_layer_ordering_info_ref_layer_idx equal to 0 specifies that the sub-layer ordering related parameters are signaled in the current SPS without parameter prediction. sps_sub_layer_ordering_info_ref_layer_idx equal to 1 specifies that the sub-layer ordering related parameters are predicted from the associated VPS, without signaling the sub-layer ordering related syntax in the current SPS. Otherwise, sps_sub_layer_ordering_info_ref_layer_idx indicates that the parameters related to sub-layer ordering are inferred from the parameters related to sub-layer ordering in the active SPS of the (sps_sub_layer_ordering_info_ref_layer_idx – 2) th directly referred layer.

sps_coding_tool_info_ref_layer_idx equal to 0 specifies that the coding tool related parameters are signaled in the current SPS without parameter prediction. sps_coding_tool_info_ref_layer_idx equal to 1 specifies that the coding tool related parameters are predicted from the associated VPS, without signaling the coding tool related syntax in the current SPS. Otherwise, sps_coding_tool_info_ref_layer_idx indicates that the parameters related to coding tools are inferred from the parameters related to coding tools in the active SPS of the (sps_sub_layer_ordering_info_ref_layer_idx – 2) th directly referred layer.

sps_reference_picture_set_info_ref_layer_idx equal to 0 specifies that the reference picture set related parameters are signaled in the current SPS without parameter prediction. sps_reference_picture_set_info_ref_layer_idx equal to 1 specifies that the reference picture set related parameters are predicted from the associated VPS, without signaling the reference picture set related syntax in the current SPS. Otherwise, sps_reference_picture_set_info_ref_layer_idx indicates that the parameters related to reference picture set are inferred from the parameters related to reference picture set in the active SPS of the (sps_reference_picture_set_info_ref_layer_idx – 2) th directly referred layer.

Note that, if only VPS-to-SPS prediction is adopted, sps_video_format_info_ref_layer_idx, sps_sub_layer_ordering_info_ref_layer_idx, sps_coding_tool_info_ref_layer_idx, and sps_reference_picture_set_info_ref_layer_idx can be replaced with sps_video_format_info_present_flag, sps_sub_layer_ordering_info_present_flag, sps_coding_tool_info_present_flag, and sps_reference_picture_set_info_present_flag, respectively.

sps_video_format_info_present_flag, sps_sub_layer_ordering_info_present_flag, sps_coding_tool_info_present_flag, and sps_reference_picture_set_info_present_flag indicate whether the corresponding syntax elements are signaled in the SPS, or prediction from the associated VPS, respectively.

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).