| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0166r2 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Draft 10 with AVC for UHD-1 material |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Name(s) Pierre Andrivon Philippe Salmon Philippe Bordes Technicolor 975avenue des champs blancs – CS17616 35576 Cesson-Sévigné Cedex, France Marco Arena Paola Sunna Centro Ricerche e Innovazione Tecnologica RAI, corso Giambone 68, 10135 Torino, Italy | Tel: Email: Tel: Email: | +33 2 99 27 38 07 pierre.andrivon@technicolor.com +39 011 8103189 marco.arena@rai.it |
| Source: | Technicolor, RAI |  |  |

_____________________________

# Abstract

This contribution relates performances comparison between H.264/AVC (JM18.4) and HEVC Draft 10 (HM10.0) considering 8-bit and 10-bit bit-depth UHD 3840x2160 resolution (a.k.a. 2160p or UHD-1 or Quad Full HD) material. Test material coding with different nature is assessed and both the Main Profile and the new consumer-oriented Main 10 Profile, recently adopted in HEVC Draft 9, are considered. Concerning UHD-1 test material, this contribution states that HEVC Main and Main 10 Profiles (HM10.0) outperforms AVC High and equivalent High 10 Profiles (JM18.4 with HM-like configuration files) in objective measure (BD-rate) of around 25% for All Intra, 45 % for Random Access, 45 % for Low Delay B on average. Besides it is stated that Main 10 profile outperforms Main profile for 10-bit content coding of around 4% luma for AI, 5% luma for RA, 5% luma for LB on average.

# Introduction

The consumer-oriented Main 10 Profile (M10P) [1] aiming at leveraging both Ultra HD (UHD) and 1080p premium services has been adopted in the text specification draft 9 of HEVC during Shanghai meeting ([2]). In order to confirm and complete performance trends exposed in [3], [4], [7], [8]  this document proposes to benchmark 2160p UHD resolution (3840x2160) with 8 and 10-bit bit-depth. Color space is fixed to sRGB/Rec.709 chromaticity coordinates.

Considering this UHD resolution, both Main Profile (8-bit 4:2:0 chroma subsampling, Annex A.3.2 of [2]) and basic Main 10 Profile (up to 10-bit per component with same constrains as expressed by the Main Profile, Annex A.3.3 of [2]) for HM10.0 (HEVC) are tested comparatively with equivalent settings for High and High 10 (that was not designed as a consumer profile) profiles for JM18.4 (H.264/AVC).

# Experiments

## Assessed test protocol

Following experiments have been conducted: 8 and 10-bit 2160p test material coding with AVC High and equivalent to HEVC Main 10 profile, “High 10 Profile” (JM18.4 with provided HM-like configuration files), comparatively to HEVC (HM10.0) Main and Main 10 (with all channel set to 10-bit) Profiles.

Five sequences are selected (Cf. Table 1 hereafter). These five brand-new released EBU UHD sequences from the UHD-1 test set (DPX 2160p 10-bit R’G’B’ Rec.709). All sequences are converted to either 8-bit or 10-bit 4:2:0 Y’CbCr Rec.709 [6] format for ingestion in both tested video codecs, namely JM18.4 with provided HM-like configuration files and HM10.0 with All Intra (AI), Random Access (RA), Low Delay B (LB) GoP types ([9]).

The Y′CbCr signal was derived from the corresponding sRGB color space as follows:

- the input R’G’B’ values were normalized to a floating range between 0 and 1

- the resulting values were converted to Y’PbPr color space

- the final Y’CbCr was derived scaling the Y’PbPr signal according to the headroom and the footroom of the Rec. 709 (at 10 or 8 bits)

Tests are performed on 250 frames (5 seconds), as according to tests made by Technicolor, BD-rate performances trend is not influenced by the encoded number of pictures for the selected sequences.

This test material set may be interesting as it presents different content characteristics namely in terms of texture, color, motion and above all represent a future-proof digital acquisition mean (Sony F65 digital camera [10]).

For the 10-bit encodings the source codes have been run on a cluster grid with the following characteristics:

- OS: Linux (RedHat 5.5)

- CPU: 2 x Intel(R) Xeon(R) E5680 @ 3.33GHz, 6 cores per blade.

- RAM: 96 GB per 6 cores.

Results were cross-checked on a Windows grid.

For the 8 bit encodings the JM source codes have been run on a cluster grid with the following characteristics:

- OS: Windows 7 Professional

- CPU: 2 x Intel(R) Xeon(R) X2650 @2.67 GHz (2 processors)

- RAM: 16 GB

While the HM source codes have been run on a cluster grid with the following characteristics:

- OS: Linux Ubuntu 12.04 lts

- CPU: 64x AMD Opteron(tm) Processor 6282 SE (1.4 GHz)

- RAM: 132 GB

| Sequence name | Characteristics | Snapshot |
| --- | --- | --- |
| Lupo_candlelight | From EBU UHD-1 public test set Sony F65 capture Rec.709 DPX 10-bit Low light Complex Motion |  |
| rain_fruits | From EBU UHD-1 public test set Sony F65 capture Rec.709 DPX 10-bit Hugely textured Complex illumination |  |
| fountain_lady | From EBU UHD-1 test set Sony F65 capture Rec.709 DPX 10-bit Colorful Complex motion |  |
| Lupo_confetti | From EBU UHD-1 test set Sony F65 capture Rec.709 DPX 10-bit Very complex motion with sparse objects |  |
| waterfall_pan | From EBU UHD-1 test set Sony F65 capture Rec.709 DPX 10-bit Hugely textured Pan |  |

Table 1 – The five sequences of UHD 3840x2160 resolution  used as test material

## Objective results

# Objective results are presented in terms of BD-rate PSNR-based objective measure for Y’, Cb, Cr. Log files and configuration files were compared between HM10.0 and JM18.4 and it was verified that QP derived along each GoP are the same as well as the GoP structures.

### HEVC Main Profile vs AVC High Profile (8-bit)

|  | Y | U | V |
| --- | --- | --- | --- |
| AI | -26.5% | -22.7% | -30.5% |
| RA | -47.3% | -34.8% | -41.3% |
| LB | -48.7% | -33.2% | -41.5% |

Table 2 AVC HP (JM18.4) vs HEVC MP (HM10.0) BD-rate for 2160p sequences

According to Table 2, AI gains of HM10.0 over JM18.4 are around 25 % for AI, 45% for both RA and LB configurations.

### HEVC Main 10 Profile vs AVC High Profile (10-bit)

|  | Y | U | V |
| --- | --- | --- | --- |
| AI | -25.5% | -20.8% | -26.9% |
| RA | -46.8% | -23.6% | -31.1% |
| LB | -50.8% | -28.5% | -37.3% |

Table 3 AVC HP (JM18.4) 10-bit vs HEVC M10P (HM10.0) BD-rate for 2160p sequences

Results presented here above are consistent with those presented in Table 2.

### HEVC Main and Main 10 Profiles comparison considerations

For the Main versus Main 10 profiles comparison, we consider the hereunder assessment scheme:

Figure 1: Main vs Main10 profiles objective assessment

For Main profile coding, a rounding offset is added to the 10-bit

source and it is right shifted two times prior to coding. Then bitstream is decoded and a 2 null bits left shift is performed on the reconstructed 8-bit content. The resulting 10-bit content is compared with 10-bit source with PSNR. Thus, Main profile tests PSNR are recomputed with 10-bit PSNR, the reference being the original full 10-bit material. For Main 10 profile coding, same numbers as presented in part 2.2.2 are used.

|  | Y | U | V |
| --- | --- | --- | --- |
| AI | -3.6% | -8.5% | -10.8% |
| RA | -4.9% | -14.4% | -18.2% |
| LB | -5.4% | -16.4% | -19.6% |

Table 4 HEVC MP vs M10P (HM10.0) BD-rate for 2160p sequences

Considering HEVC, results tend to show that for the considered versatile test material and test conditions, Main 10 profile outperforms Main profile BD-rate performances for 10-bit source of around 4% luma for AI, 5% luma for RA, 5% luma for LB in terms of objective fidelity to the 10-bit source content.

# Conclusion

This contribution reportedly shows that considering 3840x2160 (QFHD) material, HEVC (HM10.0) Main and Main 10 profiles coding outperforms H.264/AVC (JM18.4) equivalent profiles coding in objective measure (BD-rate) of around 25 % for All Intra, 45 % for Random Access, 45% for Low Delay B on average. Besides it is stated that Main 10 profile outperforms Main profile for 10-bit content coding of around 4% luma for AI, 5% luma for RA, 5% luma for LB on average. It is also remarked that double check has been performed between contributors.

# References

- A. Duenas et al, JCTVC-K0109, “On a 10-bit consumer-oriented profile in High Efficiency Video Coding (HEVC)”, Shanghai, CN, Oct. 2012.

- B. Bross et al, JCTVC-L1003, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent)”, Geneva, CH, Jan. 2013.

- P. Andrivon et al, JCTVC-I0108, “High Bit Depth Considerations in HEVC”, Geneva, CH, Apr. 2012.

- P. Andrivon et al, JCTVC-J0079, “AHG12: On beyond 8 bit-depth support in HEVC”, Stockholm, SE, July 2012.

- Rec. ITU-R BT.2020-0, “Parameter values for the ultra-high definition television systems for production and international programme exchange”, Aug. 2012.

- Rec. ITU-R BT.709-5, “Parameter values for the HDTV standards for production and international programme exchange”, Apr. 2002.

- B. Li et al, JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics”, Geneva, CH, Jan. 2013.

- P. Sunna, M. Arena, JCTVC-K0110, “Comparison of Compression Performance of HEVC with AVC at different resolutions”, Shanghai, CN, Oct. 2012.

- K. McCann et al, JCTVC-K1002, “High Efficiency Video Coding (HEVC) Test Model 9 (HM 9) Encoder Description”, Shanghai, CN, Oct. 2012.

- Sony F65, http://pro.sony.com/bbsc/ssr/show-highend/resource.solutions.bbsccms-assets-show-highend-F65.shtml

# Patent rights declaration(s)

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution.

RAI does not have any current or pending patent rights relating to the technology described in this contribution.