| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0167 |
| --- | --- |

| Title: | Parameter set and other related issues |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

For HEVC extensions, e.g. MV-HEVC, SHVC, and 3D-HEVC, the parameter set properties and cross-layer alignments are proposed, with the new definitions, group of layer and independent group of layer. A group of layer is simply a group of layer within a bitstream. A complete group of layers can be defined as a new term, namely independent group of layer, where any layer in an independent group of layer has a dependency from another layer in the same independent group of layer, while a layer in an independent group of layer has no dependency from any layer out of the independent group of layer. The parameter sharing or prediction across layers may be supported in both MV-HEVC and SHVC. However, the parameter sharing or prediction across independent group of layers are not desirable, because any parsing or decoding dependency across independent group of layers may interfere a bitstream extraction and a separated management of simulcast streams. Moreover, IRAP or POC alignments within an independent group of layer will give more flexibility than alignment for a whole layers. The text on the activation of VPS will be also proposed to be aligned with the proposed definitions.

Note that this contribution is originally proposed for MV-HEVC and 3D-HEVC, but the same concepts and definitions can be also adopted for SHVC.

# Proposed definitions

For HEVC extensions, e.g. MV-HEVC, SHVC, and 3D-HEVC, new concepts, group of layer and independent group of layer, are defined. Based on these concepts, coded video sequence is newly defined.

## Background/Motivation

In SHVC or MV-HEVC, a bitstream consists of one or more coded video sequences (CVSs), and each CVS may have one or more NAL units with nuh_layer_id equal to 0 or greater than 0. Each layer may or may not have a dependency from other layers. All layers can be grouped into several group of layers. A layer belonging to each group of layer may or may not have a dependency from a layer belonging to another group of layers. Then, a complete group of layers can be defined as a new term, namely independent group of layer, where any layer in an independent group of layer has a dependency from another layer in the same independent group of layer, while a layer in an independent group of layer has no dependency from any layer out of the independent group of layer. In other words, a layer may be inter-layer predicted from only the layers belonging to the same independent group of layer. A layer cannot refer to a layer belonging to another independent group of layer for inter-layer prediction.

With the new concept of independent group of layer, the CVS can be re-defined, as all access units in a coded video sequence belong to an independent group of layer. The definition of independent group of layer and the redefinition of CVS is needed for setting up the properties of parameter sets and other alignment or restriction across layers for SHVC and MV-HEVC.

## Proposed specification text

group of layer (GOL): a group of layers within a bitstream. independent group of layer (IGOL): a group of layer, where any layer in an independent group of layer has a dependency from another layer in the same independent group of layer except a layer, while a layer in an independent group of layer has no dependency from any layer out of the independent group of layer. coded video sequence (CVS): A sequence of access units belonging to an independent group of layer, which consists, in decoding order, of an IRAP access unit with NoRaslOutputFlag equal to 1, followed by zero or more access units that are not IRAP access units with NoRaslOutputFlag equal to 1, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit with NoRaslOutputFlag equal to 1.

# Proposed parameter set properties and others

For HEVC extensions, e.g. MV-HEVC, SHVC, and 3D-HEVC, the parameter set properties and cross-layer alignments are proposed, with the proposed definitions.

## Background/Motivation

The parameter sharing or prediction across layers may be supported in both MV-HEVC and SHVC. However, the parameter sharing or prediction across independent group of layers are not desirable, because any parsing or decoding dependency across independent group of layers may interfere a bitstream extraction and a separated management of simulcast streams. Moreover, IRAP pictures or POC alignments within an independent group of layer will give more flexibility than alignment for a whole layers. In fact, there is no strong requirement on alignments of random access points, POC values, and the start of CVS for simulcast bitstreams, which have no dependency each other. At last, the text on the activation of VPS will be also discussed.

## Proposed specification text and description

### Parameter/Parameter set properties

- Parameter set sharing across layers

Any slice, parameter set, or SEI NAL unit in a layer belonging to an independent group of layer can refer to any SPS or PPS in another layer belonging to the same independent group of layer.

Any slice, parameter set, or SEI NAL unit in a layer belonging to an independent group of layer cannot refer to any SPS or PPS in another layer not belonging to the same independent group of layer.

Any parameter cannot be shared or predicted across independent group of layers.

### Alignment across layers

Alignment within an independent group of layer –

- IDR alignment within an independent group of layer:

When a nal_unit_type of a NAL unit belonging to an independent group of layer in an access unit is equal to IDR_W_RADL or IDR_N_LP, a nal_unit_type of any NAL unit belonging to the same independent group of layer shall be equal to IDR_W_RADL or IDR_N_LP.

- POC alignment within an independent group of layer:

PicOrderCntVal for all layers belonging to an independent group of layer in an access unit shall be the same.

Alignment across group of layers -

- IRAP alignment across independent group of layers:

No restriction on an IRAP picture alignment across independent group of layers is present.

- POC alignment across independent group of layers:

PicOrderCntVal for a layer belonging to an independent group of layer may or may not be equal to a PicOrderCntVal value of a layer belonging to another independent group of layer in an access unit.

### VPS activation

- A VPS is activated at the beginning of a CVS for an independent group of layer. An active VPS for a CVS of an independent group of layer may or may not be the same with an active VPS for a CVS of another independent group of layer.

- Proposed specification text:

- A VPS RBSP includes parameters that can be referred to by one or more SPS RBSPs belonging to an independent group of layer or one or more SEI NAL units containing an active parameter sets SEI message belonging to an independent group of layer. Each VPS RBSP is initially considered not active at the start of the operation of the decoding process. At most one VPS RBSP is considered active for an independent group of layer at any given moment during the operation of the decoding process, and the activation of any particular VPS RBSP for an independent group of layer results in the deactivation of the previously-active VPS RBSP for the independent group of layer (if any).

- When a VPS RBSP (with a particular value of vps_video_parameter_set_id) is not already active for an independent group of layer and it is referred to by activation of an SPS RBSP belonging to an independent group of layer (in which sps_video_parameter_set_id is equal to the vps_video_parameter_set_id value), or is referred to by an SEI NAL unit containing an active parameter sets SEI message belonging to an independent group of layer (in which active_video_parameter_set_id is equal to the vps_video_parameter_set_id value), it is activated for an independent group of layer. This VPS RBSP is called the active VPS RBSP for an independent group of layer until it is deactivated by the activation of another VPS RBSP for an independent group of layer. A VPS RBSP, with that particular value of vps_video_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means. An activated VPS RBSP shall remain active for the entire CVS.

- NOTE 1 – Because an IRAP access unit with NoRaslOutputFlag equal to 1 begins a new CVS and an activated VPS RBSP must remain active for the entire CVS, a VPS RBSP can only be activated by an active parameter sets SEI message when the active parameter sets SEI message is part of an IRAP access unit with NoRaslOutputFlag equal to 1.

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).