| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0168 |
| --- | --- |

| Title: | On random access point for HEVC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

On top of the agreements on random access point, more details are proposed.

On IRAP pictures:

- The nuh_layer_id values of BLA NAL units can be greater than 0.

- BLA pictures with nuh_layer_id greater than 0 can be inter-layer predicted while inter prediction is disallowed.

- BLA NAL units need not be aligned across layers.

- An IRAP NAL unit of each layer with NoRaslOutputFlag equal to 1 may activate an SPS independently for the corresponding layer.

- Any IRAP picture with with NoRaslOutputFlag equal to 1 and nuh_layer_id greater than 0 does not reset PicOrderCntMsb equal to 0.

On RASL/RADL pictures:

- RASL/RADL NAL units with nuh_layer_id greater than 0 can be used for the same purpose as a base layer.

- RASL pictures with nuh_layer_id greater than 0 associated with IRAP pictures with NoRaslOutputFlag equal to 1 are not output, because may not be decodable.

- All pictures in upper layers referring to RASL pictures associated with IRAP pictures with NoRaslOutputFlag equal to 1 for inter-layer prediction are also handled as RASL pictures.

On splicing and bitstream extraction operation:

- At a splicing point, the first NAL units of layers with nuh_layer_id greater than the maximum nuh_layer_id value in the previous NAL units may be IDR or BLA. If those NAL units are not IDR or BLA, all following NAL units in decoding order are handled as RASL pictures until an IRAP picture appear.

- At an up-layer switching point, the first NAL units of layers with nuh_layer_id greater than the maximum nuh_layer_id value in the previous NAL units shall be IDR, BLA, or CRA with NoRaslOutputFlag equal to 1.

On indication of IRAP picture types in an access unit:

- Using pic_type in access unit delimiter, an information on IRAP picture types of NAL units in an access unit is signalled to assist slicing or bitstream extraction for layer switching.

# Introduction

In the last Geneva meeting, several decisions on random access point pictures for SHVC and MV-HEVC were made based on the contribution, JCTVC-L0039, JCT3V-C0081, and JCT3V-C0085. In this contribution, more details on top of the previous adoptions are suggested.

# On IRAP pictures in enhancement layers

- The nuh_layer_id values of BLA NAL units can be greater than 0.

- In enhancement layers, BLA is used for indicating a splicing point or a layer switching point.

- When a bitstream is spliced, if the NAL unit type of a base layer is changed from CRA to BLA, the BLA NAL units of enhancement layers may be changed to CRA NAL units when present.

- When a bitstream is up-layer switched, the CRA NAL units of the associated upper layers are changed to BLA NAL units.

- BLA pictures with nuh_layer_id greater than 0 can be inter-layer predicted while inter prediction is disallowed

- BLA NAL units need not be aligned across layers, similarly to CRA.

- A BLA NAL unit type can be used for all VCL NAL units with a particular value of nuh_layer_id while another NAL unit type can be used for all VCL NAL units with another particular value of nuh_layer_id in the same access unit.

- Proposed specification text:

F.7.4.2.2 NAL unit header semantics When the nal_unit_type value nalUnitTypeA is equal to IDR_W_DLP or IDR_N_LP BLA_W_LP, BLA_W_DLP or BLA_N_LP for a coded picture with a particular PicOrderCntVal value and within a particular CVS, the nal_unit_type value shall be equal to nalUnitTypeA for all VCL NAL units of all coded pictures with the same particular PicOrderCntVal value and within the same particular CVS.

- An IRAP NAL unit of each layer with NoRaslOutputFlag equal to 1 may activate an SPS independently for the corresponding layer.

- Even though an IRAP NAL access unit with NoRaslOutputFlag equal to 1 and nuh_layer_id greater than 0 does not begin a new CVS, it may activate an SPS for the corresponding layer.

- Proposed specification text:

F.7.4.2.4.2 Order of VPS, SPS and PPS RBSPs and their activation When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a nuh_layer_id value and it is referred to by activation of a PPS RBSP for that nuh_layer_id value (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), it is activated for that nuh_layer_id value. This SPS RBSP is called the active layer SPS RBSP for that nuh_layer_id value until it is deactivated by the activation of another SPS RBSP for the same layer. An SPS RBSP, with that particular value of sps_seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means. An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from an IRAP picture with NoRaslOutputFlag equal to 1 having that nuh_layer_id value, inclusive, until either the IRAP picture with that nuh_layer_id value and NoRaslOutputFlag equal to 1, exclusive, or the end of the CVS, whichever is earlier. The nuh_layer_id value the NAL unit containing the SPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same SPS RBSP may be the active layer SPS for more than one nuh_layer_id value.

- Any IRAP picture with with NoRaslOutputFlag equal to 1 and nuh_layer_id greater than 0 does not reset PicOrderCntMsb equal to 0.

- Proposed specification text:

### 8.3.1 Decoding process for picture order count If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and nuh_layer_id equal to 0, PicOrderCntMsb is set equal to 0. ### F. 8.3.1 Decoding process for picture order count PicOrderCntVal for a coded picture with nuh_layer_id greater than 0 is set equal to PicOrderCntVal of a coded picture with nuh_layer_id equal to 0.

# On RASL/RADL pictures in enhancement layers

- The nuh_layer_id values of RASL/RADL NAL unit can be greater than 0.

- RASL pictures with nuh_layer_id greater than 0 associated with IRAP pictures with NoRaslOutputFlag equal to 1 are not output, because may not be decodable.

- All pictures in upper layers referring to RASL pictures associated with IRAP pictures with NoRaslOutputFlag equal to 1 for inter-layer prediction are also handled as RASL pictures.

# On splicing and bitstream extraction

- At a splicing point, the first NAL units of layers with nuh_layer_id greater than the maximum nuh_layer_id value in the previous NAL units may be IDR or BLA. If those NAL units are not IDR or BLA, all following NAL units in decoding order are handled as RASL pictures until an IRAP picture appear.

- When a bitstream is spliced, CRA NAL units in all layers at a layer switching point are changed to BLA NAL units.

- At an up-layer switching point, the first NAL units of layers with nuh_layer_id greater than the maximum nuh_layer_id value in the previous NAL units shall be IDR, BLA, or CRA with NoRaslOutputFlag equal to 1.

- When a bitstream is extracted with layers upper than the previous layers, CRA NAL units of the associated up-layers at a layer switching point are changed to BLA NAL units, or their HandleCraAsBlaFlags are set equal to 1.

# Signalling random access point types

In order to assist splicing operations or bitstream extraction with layer switching, the random access point picture types of NAL units in an access unit is signalled in an access unit delimiter (AUD). The pic_type in AUD indicates the slice_type values for all slices of the coded picture. However, only three types are assigned for I, P, and B picture types. Using the remaining five types, the IRAP picture types of the coded pictures in an IRAP access unit can be indicated. This signalling is helpful for splicing operation or layer switching operation, because a useful information on IRAP picture types for splicing or layer switching can be obtained by parsing only AUD before checking the NAL unit types of all NAL units in the access unit. This syntax is still informative, without affecting the decoding process.

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| rbsp_trailing_bits( ) |  |
| } |  |

| pic_type | slice_type values that may be present in the coded picture |
| --- | --- |
| 0 | I |
| 1 | P, I |
| 2 | B, P, I |

| pic_type | IRAP picture types of NAL units in the access unit |
| --- | --- |
| 3 | IDR in all layers |
| 4 | CRA in all layers |
| 5 | CRA in the base layer |
| 6 | BLA in all layers |
| 7 | BLA in the base layer |

pic_type indicates that the slice_type values for all slices of the coded picture are members of the set listed in Table 7-2 for the given value of pic_type. When the access unit is an IRAP access unit or may contain one or more NAL units of which nal_unit_type is in the range of 16 to 23, pic_type indicates which kinds of IRAP pictures are contained in the access unit.

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).