| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0170 |
| --- | --- |

| Title: | Temporal layer management for HEVC extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Jaewon Yoon Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Several suggestions on temporal layers in enhancement layers are provided., .

On TSA/STSA pictures:

- The nuh_layer_id values of TSA/STSA NAL units can be greater than 0.

- TSA/STSA NAL units need not be aligned across layers.

On TemporalID in enhancement layers

- Pictures with a smaller TemporalId in enhancement layers cannot refer to pictures with a greater TemporalId in referenced layers.

On temporal rate and bitstream extraction

- Allowing different temporal rates for each layer.

- Each layer can be extracted with a different temporal rate.

- A flag signalled in VPS is proposed, which indicates whether temporal rates and TSA/STSA are synchronized across layers.

A flag signalled in VPS is proposed, which indicates whether temporal rates and TSA/STSA are synchronized across layers.

# Introduction

In the last Geneva meeting, several decisions on random access point pictures for SHVC and MV-HEVC were made based on the contribution, JCTVC-L0039, JCT3V-C0081, and JCT3V-C0085. In this contribution, details on temporal sub-layer access pictures and temporal rate are suggested.

# On TSA/STSA pictures in enhancement layers

- The nuh_layer_id values of TSA/STSA NAL units can be greater than 0.

- In enhancement layers, TSA/STSA is used for indicating a sub-layer (temporal layer) access point, same as a base layer.

- TSA/STSA NAL units need not be aligned across layers.

- Each layer can be extracted with a different temporal rate.

- A TSA/STSA NAL unit type can be used for all VCL NAL units with a particular value of nuh_layer_id while another NAL unit type can be used for all VCL NAL units with another particular value of nuh_layer_id in the same access unit.

- Design alignment with CRA.

# Restriction on TemporalId in enhancement layers

- Pictures with a smaller TemporalId in enhancement layers cannot refer to pictures with a greater TemporalId in referenced layers.

- TemporalId of NAL units with nuh_layer_id equal to i shall be greater than TemporalId of the NAL units with nuh_layer_id equal to j, where i is greater than j and the j-th layer is used for inter-layer prediction of the i-th layer.

# On temporal rate and bitstream extraction

- Allowing different temporal rates for each layer.

- A layer can have a temporal rate other than another layer.

- Each layer can be extracted with a different temporal rate.

- TSA/STSA need not be aligned across layers.

- A flag is proposed, which indicates whether temporal rates and TSA/STSA are synchronized across layers.

- The flag may be signalled in VPS.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| temporal_rate_synchronize_all_layers_flag | u(1) |
| if( !temporal_rate_synchronize_all_layers_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| temporal_rate_synchronize_flag[ i ] | u(1) |
| } |  |

temporal_rate_synchronize_all_layers_flag equal to 1 specifies that the temporal rates of all layers are equal to the temporal rate of the base layer.

temporal_rate_synchronize_flag[ i ] equal to 1 specifies that the temporal rate of the ith layer equal to the temporal rate of the base layer.

# References

- J. Chen et. al., “SHVC Test Model 1 (SHM 1),” Document JCTVC-L1007, the 12th JCTVC meeting, Geneva, CH, Jan. 2013

- G. Tech et. al., “MV-HEVC Draft Text 3,” Document JCT3V-C1004, the 3rd JCT3V meeting, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).