| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0173 |
| --- | --- |

| Title: | Removing a Level Restriction on Coding Tree Block Size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eran Pinhasov Shevach Riabtsev Assaf Menachem CSR Israel Advanced Technology Center P.O. Box 15015, Haifa 31905, Israel | Email: | eran.pinhasov@csr.com shevach.riabtsev@csr.com assaf.menachem@csr.com |
| Source: | CSR Plc. |  |  |

_____________________________

# Abstract

This contribution recommends the removal of a level-specific restriction on Coding Tree Block (CTB) size, currently restricted to 32x32 or 64x64 for level 5 and higher. This contribution claims that the restriction adds substantial cost and complexity to hardware implemented encoders which target HEVC 4K video.

Removing the said CTB size restriction would also simplify the upgrade of legacy AVC\H.264 hardware encoders to 4K HEVC while maintaining their native 16x16 architectures and processing pipes, reducing both implementation costs and development cycle. Upgrading to HEVC is not only motivated by compression efficiency, but also by the new parallel-processing and streaming tools, especially needed for high-resolution video.

In addition, this contribution states that the said restriction limits implementation flexibilities and increases time-to-market, thus possibly delaying the industry’s wide adoption of HEVC for 4K content.

Due to the newly reported implications, it is recommended to consider removal of the CTB size restriction from the standard.

# Problem Statement

The HEVC specification [1] restricts the value of CtbSizeY to 32 or 64 for level 5 and higher. It is reported in “Comparison of the Coding Efficiency of Video Coding Standards - Including HEVC” ‎[3] that usage of large CTBs increases compression efficiency as resolution gets higher, and also improves decoding performance. However, it can be deduced from [3] that encoding performance is significantly reduced:

- Encoder with maximum 32x32 CTB size requires 41% more time than that with 16x16 CTB size

- Encoder with maximum 64x64 CTB size requires 72% more time than that with 16x16 CTB size

It is reported in [2] that CTB sizes of 32x32 or higher are better for high resolution content (e.g. 4Kx2K), since they reduce line buffer storage requirements (~1KB) and reduce decoder overheads (10% increase in overall worst-case decode time comparing to the case of 32x32 CTBs).

However, in case of 4K HEVC encoders, especially hardware implemented ones, significant complexity and cost are added when minimal CTB size of 32x32 is mandatory (compared to 16x16 minimal CTB size). In particular, the area penalty imposed on encoders is of much greater scale, than the area saving obtained by the line buffer storage reduction described in [2].

Moreover, the said restriction breaks legacy AVC\H.264 (or multi-standard) processing pipe, consequently imposing additional implementation effort when upgrading to HEVC 4K.

## Implementation cost evaluation for large CTBs

Video encoder architectures typically require on-chip memory for storing the following principal components:

- Target image data (in CTB units) read from off-chip memory (e.g. DRAM)

- Reference data read from off-chip memory (e.g. DRAM) usually requires large memory, depending on ME search size.

- Buffering between pipeline stages, storing: coefficients, predictors (Inter & Intra), reconstructed and filtered data.

Analysis of a potential pipelined encoder architecture (which includes the basic HEVC building blocks) shows that using CTB size of 32x32/64x64 adds at least 25KB/100KB compared to 16x16 CTB respectively, resulting in significant area/cost penalty. The exact on-chip memory requirements may vary depending on the architecture.

In addition, due to large CTB sizes, significant memory increase is expected in proprietary components inside the pipeline of encoders and decoders, such as pre or post video processing (e.g. scaler, de-interlacer, image-enhancement etc.) and reference image cache. These proprietary components typically require line-buffers.

# Conclusion

The performance advantages and line buffer savings offered by the level-specific CTB size restriction (i.e. minimum 32x32 CTB size for level 5 and higher), impose substantial cost and complications on hardware implemented HEVC encoders, in particular AVC\H.264 encoders upgraded to 4K HEVC. The newly added parallel-processing tools make HEVC very attractive for high-resolution video, regardless of the compression efficiency benefits (partially obtained by using large CTB sizes).

Therefore this contribution recommends removal of the minimum 32x32 CTB size restriction for level 5 and higher.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1003_v34, March 2013.

- W. Wan, T. Hellman, “Adding a Level Restriction on Coding Tree Block Size,” Document JCTVC-J0334, July 2012.

- J.-R. Ohm, G. J. Sullivan, H. Schwarz, T. K. Tan and T. Wiegand, “Comparison of the Coding Efficiency of Video Coding Standards—Including High Efficiency Video Coding (HEVC)”, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 22, No. 12, Dec-2012

# Patent rights declaration(s)

CSR, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

Remove restriction (e) in Section A.4.1.