| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0174 |
| --- | --- |

| Title: | SCE2: Cross-check of Subtest 2 (M0093, LG and Qualcomm) by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution is a cross-verification of proposal K0093 on deblocking filter modifications for SHVC, which reports on Subtest 2 in SCE2. The cross-checker confirms perfect match of bitrates and PSNR. The new template should be used for the results (with the correct single layer results). The software matches the description in the document.

# Introduction

The cross-checked document JCTVC-M0093 [3] presents a combination of the two tests described in the SCE2 description [2] Subtest 2, namely, the document [4] from LG and the document [5] from Qualcomm. The luma filtering part in the cross-verified document corresponds to the description in subtest 2.2 (Qualcomm) and the chroma filtering parts corresponds to the description specified in subtest 2.1 (LG).

# Cross-verification results

The cross-verifier reports perfect match of BD-rate results on the common test condition sequences. The obtained encoding and decoding time are approximate since the results have been obtained on heterogeneous computational cluster.

The provided source code have been studied and found to match the description in the proposal JCTVC-M0093.

Table 1 Objective results for LB-Main on additional Type 1 sequences.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | -0.2% | -0.2% |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.2% | -0.1% | -0.3% | -0.3% |  |  |  |
| Overall (Test vs Ref) | 0.0% | -0.2% | -0.2% | -0.1% | -0.3% | -0.3% |  |  |  |
| Overall (Test vs single layer) | 12.4% | 13.5% | 13.2% | 10.2% | 9.9% | 9.4% |  |  |  |
| EL only (Test vs Ref) | 0.0% | -0.2% | -0.2% | -0.1% | -0.4% | -0.4% |  |  |  |
| Enc Time[%] | 93.7% |  |  | 96.2% |  |  |  |  |  |
| Dec Time[%] | 94.2% |  |  | 91.3% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% |  |  |  | 0.0% | -0.3% | -0.4% |
| Class B | 0.0% | -0.2% | -0.2% | -0.1% | -0.4% | -0.4% | -0.1% | -0.6% | -0.6% |
| Overall (Test vs Ref) | 0.0% | -0.2% | -0.2% | -0.1% | -0.4% | -0.4% | 0.0% | -0.5% | -0.5% |
| Overall (Test vs single layer) | 19.3% | 31.8% | 32.7% | 16.5% | 28.0% | 30.3% | 14.8% | 28.1% | 31.9% |
| EL only (Test vs Ref) | 0.0% | -0.2% | -0.2% | -0.1% | -0.5% | -0.5% | -0.1% | -0.6% | -0.6% |
| Enc Time[%] | 83.7% |  |  | 85.0% |  |  | 88.2% |  |  |
| Dec Time[%] | 98.9% |  |  | 102.8% |  |  | 101.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% |  |  |  | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.4% |
| Overall (Test vs Ref) | 0.0% | -0.1% | -0.1% | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.3% |
| Overall (Test vs single layer) | 26.1% | 36.3% | 38.1% | 22.5% | 32.8% | 35.6% | 22.3% | 34.0% | 38.6% |
| EL only (Test vs Ref) | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 83.8% |  |  | 84.9% |  |  | 87.0% |  |  |
| Dec Time[%] | 98.2% |  |  | 99.6% |  |  | 100.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |  |  |  | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.4% | 0.0% | -0.4% | -0.3% |
| Overall (Test vs Ref) | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.4% | 0.0% | -0.3% | -0.3% |
| Overall (Test vs single layer) | 28.2% | 37.2% | 38.3% | 24.6% | 32.6% | 34.6% | 23.7% | 33.1% | 36.8% |
| EL only (Test vs Ref) | 0.0% | -0.1% | -0.2% | -0.1% | -0.3% | -0.4% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 80.4% |  |  | 83.7% |  |  | 84.7% |  |  |
| Dec Time[%] | 96.4% |  |  | 95.8% |  |  | 96.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

The results for additional sequences (to be used in the subjective test) have also been obtained.

# Results on additional sequences

The results on additional sequence match the ones provided by the proponent.

Table 2. Cross-check of additional sequences

| Sequence | BL QP | EL QP | Scalability |  |  | bitrate | Y psnr | U psnr | V psnr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DucksTakeOff | 34 | 36 | 2X | RA | SHM1.0 | 1837.4880 + 2763.2016 | 27.5183 | 31.5425 | 36.1466 |
|  |  |  |  |  | Proposal | 1837.4880 + 2762.7416 | 27.5174 | 31.5455 | 36.1461 |
| DucksTakeOff | 38 | 34 | SNR | RA | SHM1.0 | 3341.6840 + 3017.7504 | 28.3323 | 31.7833 | 36.3303 |
|  |  |  |  |  | Proposal | 3341.6840 + 3015.6728 | 28.3309 | 31.7889 | 36.3297 |
| WestWindsEasy | 34 | 36 | 2X | LDP | SHM1.0 | 1054.9904 + 990.7768 | 33.1639 | 43.1609 | 46.2575 |
|  |  |  |  |  | Proposal | 1054.9904 + 991.9512 | 33.1647 | 43.1600 | 46.2564 |
| WestWindsEasy | 38 | 34 | SNR | LDP | SHM1.0 | 1486.0288 + 1318.3384 | 34.2474 | 43.7977 | 46.7466 |
|  |  |  |  |  | Proposal | 1486.0288 + 1319.3984 | 34.2463 | 43.7963 | 46.7474 |

# Conclusions

The cross-checker confirms perfect match of bitrates and PSNR provided by the proponent. Version 1 results of the proposal contained different numbers for the single layer, thus making the results different for comparison against the single layer (probably, a different template has been used). The description of the algorithm in the document matches the source code.

# References

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-L1009, Geneva, Jan 2013.

- L. Guo, Y. He, D.-K. Kwon, J. Zan, J.-W. Kang, “Description of SCE2: Inter-layer Texture Prediction Signaling and Deblocking”, JCTVC-L1102, Geneva, Jan 2013.

- C. Kim, B. Jeon, L. Guo, G. Van Der Auwera, J. Chen, M. Karczewicz, " SCE2.2: Deblocking boundary strength modification for Intra BL," JCTVC-M0093, Incheon, Apr. 2013

- T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek), C. Kim, J. Park, B. Jeon (LG), “TE2-3.1.7: Inter-layer intra prediction signaling with residual skip flag JCTVC-L0069, Geneva, Jan. 2013.

- L. Guo, V. Seregin, J. Chen, M. Karczewicz, " TE2: Results of Test 3.1.1 on Inter-layer Texture Prediction Signaling", JCTVC-L0284, Geneva, Jan. 2013.