| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0175_r1 |
| --- | --- |

| Title: | On support for alpha channel in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC – R&D |  |  |

_____________________________

# Abstract

Alpha channel signals are used in professional (studio) video coding applications and their usage may become popular also for frame composition at the receiver side. Briefly, the alpha channel complements the information contained in the main bitstream by providing, for each image pixel, its degree of transparency. To support the coding and embedding of alpha channel in the range extensions of the HEVC standard, this contribution introduces the concept of secondary picture. A secondary picture is a complementary picture which is sent together with the primary coded picture and is compressed and managed as a monochrome picture coded with the same coding tools specified in the syntax of the HEVC standard. In this document, the definitions, syntax and semantics required by the proposed secondary picture are introduced first and then different options, to address different application requirements, are proposed and discussed.

# Introduction

Alpha channel signals are used in professional and studio video coding applications for object insertion, manipulation and post-production editing. The alpha channel conveys information which complements the primary coded pictures by indicating for each pixel its degree of transparency or equivalently its degree of opacity. Alpha channels are videos consisting of monochrome frames whose pixels assume values in the range r = [αtransparent, αopaque] where αtransparent is typically equal to zero and αopaque is greater than αtransparent. Examples of alpha channel signals are depicted in Figure 1 and Figure 2 where both the primary and the associated alpha channel pictures are shown. Given the direct relationship between the primary coded pictures and their corresponding alpha channel frames, it is beneficial to encode and embed the alpha channel in the bitstream so that post-production, editing and other operations are simplified. In this context, this document proposes to support alpha channel signals in HEVC by means of the secondary picture concept. A secondary picture is a frame which follows the primary coded picture in the bitstream and can be coded using the same syntax and coding tools of the HEVC standard, treating each picture as a monochrome image. The arrangement for the primary and auxiliary pictures is shown in Figure 3. Moreover, in addition to the coded data relative to alpha channel signals, some applications may benefit by the insertion of some data useful in the editing or display process. Beside proposing secondary picture support, this contribution also proposes to signal additional information useful for the aforementioned purposes. The remainder of this document is structured as follows: Section 2 describes the changes in the syntax and semantics to support alpha channel signals by means of the proposed secondary picture. Moreover, Section 2 also proposes how to support the re-use of the alpha channel from previously decoded secondary pictures. Finally, Section 3 describes the changes in the syntax and semantics to support the clipping and threshold processes on the decoded alpha channel values.

| (a) | (b) |
| --- | --- |

Figure 1: Example of alpha channel signal: (a) primary picture for a sign language interpreter video, (b) associated alpha channel extracted after chroma keying whereas the values are either αtransparent = 0 or αopaque = 255.

| (a) | (b) |
| --- | --- |

Figure 2: Example of alpha channel extracted from the VenueVu test sequence used in AHG 7 and 8: (a) first frame of the primary picture, (b) alpha channel whereas the pixels take values in the range [αtransparent = 0, αopaque = 255].

Figure 3: Proposed secondary picture concept for alpha channel support in HEVC.

# Proposed changes to the current working draft for range extensions specification

This section proposes the definitions, syntax and semantics changes to provide support for secondary pictures in HEVC. Beside the general support for alpha channel, some applications as logo insertion or sign language interpreter insertion require an alpha channel to allow the receiver to switch on/off the content superimposed to the main program. An example of use of the alpha channel for sign language interpreter insertion is depicted in Figure 4. As may be noted, the two sign language interpreters (hereafter also denoted as signers) occupy an image area which remains fixed along several frames. In these circumstances, it would be desirable to signal for each secondary picture that no coded data is sent and instead the alpha channel values should be replaced with the ones from previously decoded secondary pictures. The arrangement for the primary and secondary picture when this latter is replaced from the previous is shown in Figure 5. In the following, the syntax and semantics changes required to support this functionality are proposed. The changes to the current working draft for range extensions are highlighted in yellow.

| (a) | (b) |
| --- | --- |

Figure 4: Example of the alpha channel required to insert the sign language interpreter in a news program. The alpha channel associated to this application may be fixed for several frames along the time.

Figure 5: Re-use of the secondary picture for a subset of secondary pictures.

## Definitions

alpha blending A process not specified by this Recommendation | International Standard, in which an secondary coded picture is used in combination with a primary coded picture and with other data not specified by this Recommendation | International Standard in the display process. In an alpha blending process, the samples of a secondary coded picture are interpreted as indications of the degree of opacity (or, equivalently, the degrees of transparency) associated with the corresponding samples of the primary coded picture.

secondary coded picture A picture that supplements the primary coded picture and may be used in combination with other data not specified by this Recommendation | International Standard in the display process. A secondary coded picture has the same syntactic and semantic restrictions as a monochrome primary coded picture. A secondary coded picture must have the same width and height in luma samples of the primary coded picture. Secondary coded pictures have no normative effect on the decoding process. See also primary coded picture.

interpretation sample value A possibly-altered value corresponding to a decoded sample value of an secondary coded picture that may be generated for use in the display process. Interpretation sample values are not used in the decoding process and have no normative effect on the decoding process.

primary coded picture: A coded representation of a picture containing all coding tree units of the picture. The only pictures that have a normative effect on the decoding process are primary coded pictures.

## Syntax

7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| seq_parameter_set_extension_rbsp() |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.2.3	Sequence parameter set extension RBSP syntax

| seq_parameter_set_extension_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| secondary_format_idc | ue(v) |
| if( secondary_format_idc != 0 ) { |  |
| bit_depth_secondary_minus8 | ue(v) |
| alpha_opaque_value | ue(v) |
| alpha_transparent_value | ue(v) |
| } |  |
| additional_extension_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.2.3	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) { |  |
| secondary_pic_present_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

7.4.3.2	Sequence parameter set extension RBSP semantics

sps_seq_parameter_set_id identifies the sequence parameter set associated with the sequence parameter set extension. The value of sps_seq_parameter_set_id shall be in the range 0 to 15 inclusive.

secondary_format_idc equal to 0 indicates that there are no secondary coded pictures in the coded video sequence. secondary_format_idc equal to 1 indicates that exactly one secondary coded picture is present in each access unit of the coded video sequence. secondary_format_idc equal to 2 indicates that exactly one secondary coded picture may be present in each access unit of the coded video sequence. The value of secondary_format_idc shall be in the range of 0 to 2, inclusive. Values greater than 2 for secondary_format_idc are reserved for purposes to be specified in the future by ITU-T | ISO/IEC. When secondary_format_idc is not present, it shall be inferred to be equal to 0.

bit_depth_secondary_minus8 plus 8 specifies the bit depth of the samples of the sample array of the secondary coded picture. bit_depth_secondary_minus8 shall be in the range 0 to 6 inclusive.

alpha_opaque_value specifies the interpretation sample value of a secondary coded picture sample for which the associated luma and chroma samples of the primary coded picture in the same access unit are considered opaque for purposes of alpha blending.

alpha_transparent_value specifies the interpretation sample value of a secondary coded picture sample for which the associated luma and chroma samples of the primary coded picture in the same access unit are considered transparent for purposes of alpha blending.

The decoding of the secondary coded pictures is not required for conformance with this Recommendation | International Standard.

The syntax of each coded slice of a secondary coded picture shall obey to the same constraints as a coded slice of a monochrome primary coded picture. For the secondary coded picture the following restrictions apply:

a)	In regards to whether the primary coded picture is an IDR picture, the following applies:

–	If the primary coded picture is an IDR picture, the secondary coded slice syntax shall correspond to that of a slice having nal_unit_type equal to 19 or 20 (a slice of an IDR picture).

- Otherwise (the primary coded picture in not an IDR picture), the secondary coded slice syntax shall correspond to that of a slice having nal_unit_type equal to 0 or 1 (a slice of a non-TSA, non-STSA trailing picture).

b)	The slices of a secondary coded picture shall have the same height and width of the corresponding primary coded picture.

The (optional) decoding process for the decoding of secondary coded pictures is the same as if the secondary coded pictures were primary coded pictures in a separate coded video stream that differs from the primary coded pictures in the current coded video stream in the following ways:

–	The IDR or non-IDR status of each secondary coded picture shall be inferred to be the same as the IDR or non-IDR status of the primary picture in the same access unit, rather than being inferred from the value of nal_unit_type of the secondary coded picture.

–	The value of chroma_format_idc and the value of ChromaArrayType shall be inferred to be equal to 0 for the decoding of the secondary coded pictures.

–	The value of bit_depth_luma_minus8 shall be inferred to be equal to bit_depth_secondary_minus8 for the decoding of the secondary coded pictures.

additional_extension_flag equal to 0 indicates that no additional data follows within the sequence parameter set extension syntax structure prior to the RBSP trailing bits. The value of additional_extension_flag shall be equal to 0. The value of 1 for additional_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for additional_extension_flag in a sequence parameter set extension NAL unit.

7.4.3.3	Sequence parameter set extension RBSP semantics

secondary_pic_present_flag equal to 0 indicates that the secondary picture associated to the primary coded picture in the same access unit is not present and for purposes of alpha blending, the sample values of the picture corresponding to the first entry of the RPS list RefPicSetLtCurr be used as interpretation sample values for the current secondary picture. secondary_pic_present_flag equal to 1 indicates that the secondary picture is present in the same access unit of the primary coded picture. When secondary_format_idc is equal to 1, secondary_pic_present_flag shall be inferred equal to 1. When secondary_format_idc is equal to 0, secondary_pic_present_flag shall be inferred equal to zero. When secondary_format_idc is equal to 2, secondary_pic_present_flag shall be present in the PPS.

NOTE 1 – When seconday_pic_present_flag is equal to 0, the RPS list RefPicSetLtCurr shall contain at least one entry.

## Access unit structure

This subclause specifies the order of NAL units and coded pictures and their association to access unit for CVSs that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses Error: Reference source not found–Error: Reference source not found.

An access unit consists of one coded picture and zero or more non-VCL NAL units. The association of VCL NAL units to primary coded pictures is described in subclause Error: Reference source not found.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

The first of any of the following NAL units after the last VCL NAL unit of a coded picture specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present),

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 (when present),

–	first VCL NAL unit of a primary coded picture (always present).

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any prefix SEI NAL units are present, they shall not follow the last VCL NAL unit of the access unit.

–	The primary coded picture shall precede the corresponding secondary coded picture (when present).

–	When one or more coded slice of a secondary coded picture are present, they shall follow the slices relative to the primary coded picture.

–	NAL units having nal_unit_type equal to FD_NUT or SUFFIX_SEI_NUT, or in the range of RSV_NVCL45..RSV_NVCL47 or UNSPEC56..UNSPEC63 shall not precede the first VCL NAL unit of the coded picture.

–	When an end of sequence NAL unit is present, it shall be the last NAL unit in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

NOTE – VPS NAL units, SPS NAL units, PPS NAL units, prefix SEI NAL units, or NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 or UNSPEC48..UNSPEC55, may be present in an access unit, but cannot follow the last VCL NAL unit of the coded picture within the access unit, as this condition would specify the start of a new access unit.

The structure of access units not containing any NAL units with nal_unit_type equal to FD_NUT, VPS_NUT, SPS_NUT, PPS_NUT, RSV_VCL_N10, RSV_VCL_R11, RSV_VCL_N12, RSV_VCL_R13, RSV_VCL_N14, or RSV_VCL_R15, RSV_IRAP_VCL22, or RSV_IRAP_VCL23, or in the range of RSV_VCL24..RSV_VCL31, RSV_NVCL41..RSV_NVCL47, or UNSPEC48..UNSPEC63 is shown in Figure 7-6.

Figure 7-6 – Structure of an access unit not containing any NAL units with nal_unit_type equal to FD_NUT, SUFFIX_SEI_NUT, VPS_NUT, SPS_NUT, PPS_NUT, RSV_VCL_N10, RSV_VCL_R11, RSV_VCL_N12, RSV_VCL_R13, RSV_VCL_N14, RSV_VCL_R15, RSV_IRAP_VCL22, or RSV_IRAP_VCL23, or in the range of RSV_VCL24..RSV_VCL31, RSV_NVCL41..RSV_NVCL47, or UNSPEC48..UNSPEC63

# Support for clipping and threshold alpha channel sample values after decoding

This section proposes the syntax and semantics changes to support the processing relative to the interpretation values for the samples in the alpha channel. Changes to the current working draft for range extensions and specifically related to the support described in this section are highlighted in yellow.

## Syntax changes

7.3.2.3	Sequence parameter set extension RBSP syntax

| seq_parameter_set_extension_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| secondary_format_idc | ue(v) |
| if( secondary_format_idc != 0 ) { |  |
| bit_depth_secondary_minus8 | ue(v) |
| alpha_opaque_value | ue(v) |
| alpha_transparent_value | ue(v) |
| alpha_threshold_type | ue(v) |
| if( alpha_threshold_type == 1 ) |  |
| alpha_threshold_binary | ue(v) |
| } |  |
| additional_extension_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics changes

alpha_threshold_type equal to 0 specifies that the interpretation sample value for each decoded secondary coded picture sample value is equal to the decoded secondary coded picture sample value for purposes of alpha blending. alpha_threshold_type equal to 1 specifies that, for purposes of alpha blending, after decoding the secondary coded picture samples, any secondary coded picture sample value greater than alpha_threshold_binary should be set to alpha_opaque_value and any secondary coded picture sample value less than or equal to alpha_threshold_binary should be set to alpha_transparent_value. alpha_threshold_type equal to 2 specifies that, for purposes of alpha blending, after decoding the secondary coded picture samples, any secondary coded picture sample value that is greater than alpha_opaque_value should be set to alpha_opaque_value to obtain the interpretation sample value for the secondary coded picture sample, and any secondary coded picture sample value that is less than or equal to alpha_transparent_value should be set to alpha_transparent_value to obtain the interpretation sample value for the secondary coded picture sample. When alpha_threshold_type is not present it shall be inferred to be equal to 0.

alpha_threshold_binary indicates the threshold value to be used for purposes of alpha blending when alpha_threshold_type is equal to 1. alpha_threshold_binary shall be in the range of alpha_transparent_value to alpha_opaque_value inclusive. When alpha_threshold_binary is not present it shall be inferred equal to 1 << (bit_depth_aux_minus8 + 8) – 1.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).