| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0178 |
| --- | --- |

| Title: | AHG5: Range Extensions and High Bit Depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

The document presents the current capabilities of HEVC Range Extensions when operating at low (including negative) QPs, and proposes that such low QPs will be needed if the codec is to truly support high bit depths, as required by the mandate of the Range Extensions amendment. Possible sources of errors that may be caused by internal accuracy limitations currently present in HEVC are explored. It is claimed that some changes to those accuracies can mitigate the errors and thereby extend the operating range of HEVC. In addition two independent changes to the entropy coding are presented, which are stated to benefit both efficiency and practicality. These changes have been included in this proposal rather than in two separate proposals due to their applicability to high operating points.

# Introduction

HEVC Final Draft International Standard (FDIS) describes an 8 and 10 bit codec; this is to be extended to include 12 and 14 bit operation under the remit of ‘Range Extensions’. Although the software and standard have been written to allow input bit depths up to 14, the ability of the codec to code 14 bits needs to be studied.

Normally, and especially for the 8 and 10 bit codec, QPs in the positive range are studied. However, for each additional bit on the input over 8 bits, the QP can be 6 lower than 0, i.e.

minQP = -6*(bitDepth-8)

As can be seen later in Figure 4 of Section 5.1, the PSNR curves peak at around 90dB, although the excessive flattening of the curve is not ideal, and the system does not respond appropriately with 16-bit data.

# PSNR and MSE

Higher PSNRs are required in order to make higher bit depths meaningful.

PSNR is defined as a function of mean-square error (MSE) and bit depth (internal bit depth, using HEVC software terminology):

PSNR=10 * log10(((2bitDepth)-1)2 / MSE)

Using this equation, the following table of PSNRs for given bit depths and MSE can be derived:

Figure 1 - Table of PSNRs for a Range of Bit Depths and MSEs

If (for example) a 14-bit codec is only able to achieve 72.2 dB, then each output value is only accurate to within the range of ±4 of the corresponding original value. The two least significant bits are therefore effectively noise, so the codec is really a 12-bit codec with two additional random bits added at the output. (It is important to note that this is based upon averages, and that actually in some parts of the picture, better quality may be achieved.)

Extending this argument, when comparing SNRs in this purely numerical fashion, it could be thought that the best system would therefore be an 8-bit system with lossless encoding, achieving an infinite PSNR (MSE=0). However, this does not take into account the loss in rounding or truncating the video from n bits to 8 bits.

If a lossless (n-1)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12)/2=0.5.

If a lossless (n-2)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12+22+12)/4=1.5.

If a lossless (n-3)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12+22+32+42+32+22+12)/8=5.5.

If a lossless (n-4)-bit system were available to encode the n-bit data, then at the output, the n-bit MSE observed would be (0+12+22+32+42+52+62+72+82+72+62+52+42+32+22+12)/16=21.5.

Therefore, if the 14-bit system does not achieve an MSE of 21.5 (or less), which is equivalent to 71.0dB, and if the bit rate of a lossless 10-bit system were similar, then numerically speaking, only 10 bits are effectively being coded.

Consider a lossy, low bit depth (n-r)-bit system with MSE of ‘m’. If this system is used to code higher bit depth n-bit data, its MSE will therefore be given by (2r)2m.

e.g.	for a lossy (n-1)-bit system, MSE in an n-bit system would be 4m,
		for a lossy (n-2)-bit system, MSE in an n-bit system would be 16m,
		for a lossy (n-3)-bit system, MSE in an n-bit system would be 64m,
		for a lossy (n-4)-bit system, MSE in an n-bit system would be 256m.

Therefore for the case where lossy lower bit depth systems encode higher n-bit depth data, their loss is generally the main contributor for the MSE observed in the n-bit domain – simple PSNRs can be used as straight comparisons of quality.

The current Range Extension software (HM_10.0_RExt2.0 with some bug-fixes) peaks at 90dB at QP=-26 (as shown in Figure 4); this is certainly suitable for encoding 14-bit data, but if the standard were extended beyond 14-bit (e.g. for medical imaging), then the current settings are not suitable. However, at this operating point, more efficiency can be gained, as investigated in this proposal.

# Sources of Error

The core HEVC system (FDIS) has been designed for 8 and 10 bit operation. As the number of bits increases, the accuracies of parts of the system should be reconsidered. A simplified flow of data through the HEVC encoder is shown in Figure 2.

Figure 2 - Diagram showing magnitude bit depths in HEVC encoding path

The main sources of calculation noise (i.e. ignoring noise shaping caused by the various predictions and the RQT and RDOQ decision processes) in HEVC are:

- Transform matrix coefficient values.
Ideally, the inverse transform applied to transformed coefficients will reproduce the original input values. However, this is limited by the integer nature of the calculations. In HEVC, the transform matrix coefficients have 6 fractional bits (i.e. they have an inherent left-shift of 6).

- Shifting results to MAX_TR_DYNAMIC_RANGE after each stage of the transform.
The forward transform will (after compensation in the quantiser) result in values that are bitDepth + log2(size) bits in size. After the first stage of the transform, the coefficients’ width in bits should be at least bitDepth + log2(size) (though additional bits will help maintain more accuracy). However, in HEVC, these intermediates are shifted so that they never exceed MAX_TR_DYNAMIC_RANGE; similarly for the second stage.
In the inverse transform, the values into the first stage are MAX_TR_DYNAMIC_RANGE in size, although the value equivalent to ‘1’ is log2(size) smaller. The inverse transform does not shift by log2(size) after each stage; instead clipping to MAX_TR_DYNAMIC_RANGE is applied.

- Noise added during quantisation.
Although the quantiser and inverse quantiser will add noise when quantising, additional noise may be inadvertently added when the scaling lists are applied, and because the quantisation coefficients defined in the arrays ‘quantScales’ and ‘invQuantScales’ are not perfect reciprocals. These results are not studied here, although patterns with a QP periodicity of 6 can be seen in some data.

- Clipping at the output of the quantiser.
Following quantisation, the coefficients are clipped to limit the range of values that are required to be supported by the entropy coder. If this “ENTROPY_CODING_DYNAMIC_RANGE” (which is actually MAX_TR_DYNAMIC_RANGE in HEVC (FDIS), but split out in this report for the purpose of investigation) is too small, the quantised coefficients for low QPs can saturate it and become distorted as a result. The same clipping is applied at the input to the dequantiser.

In HEVC (FDIS), ENTROPY_CODING_DYNAMIC_RANGE of 15 is used for up to 10 bit operation, although coefficients in 32x32 blocks may be clipped for QP < -8.

The effects of transform matrix precision, MAX_TR_DYNAMIC_RANGE and ENTROPY_CODING_DYNAMIC_RANGE are explored in Section 5 on the study of the higher bit-depths.

# Source Data and Test Conditions

Five of the six sequences studied are from the SVT test set (1920x1080 50p at 16bit, scaled down from 4K video). The sixth sequence is Traffic_RGB (2560x1600 30p at 12 bit), as described for the standard Range Extension test conditions. Of these sequences, only the first 10 frames have been used, since numerous configurations have been tested (and are presented in the accompanying Excel spreadsheet). All frames have been studied for some tests and the trends are the same as those for 10 frames. First-frame thumbnails of these sequences can be seen in Figure 3.

Figure 3 - First frames of high bit depth test material

The HM10.0_RExt2.0 software has been modified to allow larger internal accuracies to be used.

Only the intra_main profile has been studied (at different bit depths).

The “FULL_NBIT” setting that enables the encoder to evaluate distortion using the full bit depth of the sample data when making decisions has been set to 1.

As defined in the standard Range Extensions test conditions, if the file bit depth is less than the internal bit depth (the codec’s input bit depth), then the samples are padded (with the LSBs set to 0); if the file bit depth is more than the internal bit depth, the samples are scaled and rounded.

Below, bitDepth is used to describe the internal bit depth; the file’s bit depth is not mentioned.

Systems with internal bit depth (bitDepth) up to 16 are considered. Although 14 is the limit for the Range Extensions amendment, studying the function of the system beyond the limit gives reassurance that the system functions within the limits. In addition, if 16 bits are later required by the standard (e.g. for medical imaging), then the required configuration is also known.

Only twelve consecutive even QPs at each bitDepth are considered, in each case, the minimum QP being -6 (bitDepth-8).

As the effects of transform matrix precision are considered, transform-skip has been disabled for some simulations (for example, see Figure 5). It has been seen that, if the transform matrices are unsuitable, the encoder (given the option) elects to use transform-skip for all TUs where possible, hiding the effect of the matrix precision. Similarly, (I)PCM has not been enabled, because this will be selected for many of the high bit simulations and prevent analysis of the response of the transforms.

Due to the high level of noise in the source sequences, it is observed that the encoder selects almost exclusively 4x4 TUs for the base HEVC configuration. To address this bias in the results, an option has been added to the software to apply a [1,2,1] or [1,4,6,4,1] 2-dimensional filter to the input images. Results are presented for DucksTakeOff with the [1,2,1] filter applied and InToTree with the [1,4,6,4,1] filter applied. In these sequences, it is observed that all available TU sizes are used, even with transform-skip enabled.

Note that this document gives only an overview of the results, and accompanying Excel spreadsheets allow exploration of many combinations of settings.

# Adjusting the Internal Accuracies

## Base HEVC Configuration

Figure 4 illustrates the performance of HEVC at a number of internal bit depths. The graph contains traces for 8-bit (QP 22 to 0), 10-bit (QP 10 to -12), 12-bit (QP -2 to -24), 14-bit (QP -14 to -36) and 16-bit (QP -26 to -48) processing. As can be seen, the system reaches a PSNR maximum at approximately 90dB, and becomes more inefficient as QP becomes more negative (as bit rate continues to increase).

Figure 4 - Bit rate vs. green channel PSNR for OldTownCross at QP range 22 to -48 with 6-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 15 and ENTROPY_CODING_DYNAMIC_RANGE = 15 (RDOQ disabled, transform-skip enabled)

Figure 5 - Bit rate vs. green channel PSNR for OldTownCross at QP range 22 to -48 with 6-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 15 and ENTROPY_CODING_DYNAMIC_RANGE = 15 (RDOQ disabled, transform-skip disabled)

As observed in Figure 4, the PSNR for bitDepth = 8 and bitDepth = 10 increases sharply at QPs of 4 (-8 for 10-bit) and below (rightmost three data points on each trace). At QP 4, the quantisation divisor for 8-bit is 1 (QP -8 for 10-bit), leaving the mismatch between the forward and inverse transforms and between the quantisation and dequantisation coefficients as the only sources of error. As the system tends towards lossless processing, the MSE approaches zero and the PSNR spikes upward.

Although these spikes would appear to indicate that a lower bit depth system is more efficient than a higher bit depth system, the cost of converting the higher bit depth (n+2)-bit video to n-bit video must also be considered as described in Section 2. This conversion will add a MSE of 1.5 to the almost lossless PSNR of the lower-accuracy system, resulting in a lower PSNR. An (n+2)-bit system operating at the same QP will have errors mainly in the two least significant bits, meaning it also approaches lossless processing when considering only n-bit accuracy.

In systems that are not limited by internal accuracy to a maximum PSNR, this “upward spike” pattern can be seen for each bitDepth as QP drops below (4 - (6 * (bitDepth - 8))). This is shown later in Figure 6.

The 16-bit system is affected by the clipping to 15-bit (MAX_TR_DYNAMIC_RANGE) in the last stage of the inverse transform (which affects all QPs, but does not affect transform-skip mode), and the clipping to 15-bit (ENTROPY_CODING_DYNAMIC_RANGE) before and after the entropy coder (which affects the lowest QPs).

## Increasing the Internal Accuracies

Experiments (see accompanying Excel spreadsheets) have determined that:

- The transform matrix precision should be at least bitDepth - 2.

- MAX_TR_DYNAMIC_RANGE should be at least 5 more than the bitDepth (for the maximum value of log2(size)). Additional accuracy has been shown to further improve coding efficiency and aspects of rate control at lower QPs, as explored at the end of this section.

- ENTROPY_CODING_DYNAMIC_RANGE should be at least 6 more than the bitDepth (1 for the “quantisation” factor applied by QPs less than (4 - (6 * (bitDepth - 8))) plus 5 for the maximum value of log2(size)). Additional accuracy has been shown to further improve coding efficiency at lower QPs.

- QUANT_SHIFT seems to be suitable at the standard value of 14 for up to the 16-bit system.

For the 16-bit system, the transform matrix precision should be set to 14, emulating the 8-bit base HEVC (FDIS) system (where the precision is 6) and MAX_TR_DYNAMIC_RANGE should be set to 21, emulating the 10-bit base HEVC (FDIS) system (where MAX_TR_DYNAMIC_RANGE is equal to 15). Since having more internal accuracy is rarely harmful, these parameters have also been tested at different bitDepths, producing the results illustrated in Figure 6. It can be seen that, for the same number of bits, significantly higher SNRs are achievable, and that the increased-accuracy system has PSNR/MSE operating points that are suitable for bitDepths of up to 16.

Note that with transform-skip enabled with these increased internal accuracies, similar curves are observed, although the ‘spikes’ are slightly less regular.

Figure 6 – Bit rate vs. green channel PSNR for OldTownCross at QP range 22 to -48 with 14-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 21 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled)

If HEVC Range Extensions is intended to produce a single new profile for all bit depths, then the system described above is suitable.

However, if different profiles are to be described for different maximum bitDepths, then having different parameter values may be useful for reducing hardware complexity in systems that do not require the highest profiles. This will require multiple sets of transform matrices (Appendix A describes a set of matrices for each transform matrix precision) if the transform matrix precision is to be varied. A set of possible profiles is proposed in Section 7, based upon the following table relating the bitDepth to the various minimum values:

*These values are limited to 15 as used in HEVC (FDIS).

†Although this value is recommended from these studies at these low QPs, HEVC (FDIS) uses 15, which may result in clipping of coefficients in 32x32 TUs at QPs less than -8.

‡Although this value is recommended from these studies at these low QPs, HEVC (FDIS) uses 6.

If different profiles are to be used, then it is recommended that these specifications be used as minima (noting that the HEVC FDIS 10-bit system does not quite meet these targets). For a 14-bit system, Figure 7 and Figure 8 show that when the transform matrix precision or MAX_TR_DYNAMIC_RANGE are set to 2 less than the recommended values from the above table, the PSNR is reduced, especially at the lowest QPs for the bit depth of 14. There is some benefit in increasing the parameters further, especially for matrix precision at very low QP.h Although the returns for this added complexity is diminished, it does provide a reliable monotonic response suitable for rate control. The system responds similarly at the other bit depths studied.

For the lowest 6 QPs for a given bitDepth, increasing MAX_TR_DYNAMIC_RANGE beyond bitDepth + 5 is beneficial, especially considering rate control. This is seen in a version of DucksTakeOff, smoothed with a [1 2 1] filter, where the SNR/bitrate response curve exhibits non-monotonic behavior. When MAX_TR_DYNAMIC_RANGE is set to bitDepth + 6, this problem is alleviated, because additional accuracy is available at the input of the quantiser (which is either not compressing the data much or is actually expanding the data at these operating points).

These cases can be explored in the accompanying Excel spreadsheets.

Figure 7 – Bit rate vs. green channel PSNR for OldTownCross at 14-bit QP range -14 to -36 with various-precision transform matrices, MAX_TR_DYNAMIC_RANGE = 19 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled)

Figure 8 – Bit rate vs. green channel PSNR for OldTownCross at 14-bit QP range -14 to -36 with 12-bit transform matrices, various values of MAX_TR_DYNAMIC_RANGE and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled)

Recommendations

In summary, the recommendations are:

- Use at least one additional transform matrix precision for higher accuracy.

  - The transform matrix precision is recommended to be at least bitDepth - 2.

  - It is preferable to have a single set for all higher accuracies, to simplify multi-profile encoders/decoders.

  - Proposed transform matrices are provided for matrix coefficient precisions 7 to 14 – see Appendix A.

  - It is suggested to use the 14-bit accuracy transform matrices as the coefficients will fit within 16-bit data types for software, and will provide sufficient accuracy to allow future extension to 16-bit video.

  - The choice of transform matrix precision could be configured by the bit depth of the input data and the profile, or alternately determined by parameters specified at the sequence, picture or slice level.

- Change MAX_TR_DYNAMIC_RANGE for higher accuracy.

  - MAX_TR_DYNAMIC_RANGE is recommended to be at least bitDepth + 5.

  - Multiple values of MAX_TR_DYNAMIC_RANGE should not present a problem for multi-profile decoders.

  - It is suggested to derive MAX_TR_DYNAMIC_RANGE = bitDepth + 5 (with a minimum of 15), to simplify encoder design.

  - Using MAX_TR_DYNAMIC_RANGE = bitDepth + 6 (with a minimum of 15) shows monotonic SNR/bitrate responses for all sequences at extremely low QPs, and is therefore preferable considering encoder rate control.

  - As with the transform matrix precision, the choice of MAX_TR_DYNAMIC_RANGE could be configured by the bit depth of the input data and the profile.

- Change ENTROPY_CODING_DYNAMIC_RANGE for higher accuracy.

  - ENTROPY_CODING_DYNAMIC_RANGE is recommended to be at least bitDepth + 6.

  - Multiple values of ENTROPY_CODING_DYNAMIC_RANGE should not present a problem for multi-profile decoders.

  - It is suggested to derive ENTROPY_CODING_DYNAMIC_RANGE = bitDepth + 6.

# Compression Efficiency

The current HEVC CABAC entropy coder codes syntax elements using the following processes:

- The location of the last significant coefficient (in scan order) in the TU is coded.

- For each 4x4 coefficient group (groups are processed in reverse scan order), a significant-coefficient-group flag is coded, indicating whether or not the group contains non-zero coefficients. This is not required for the group containing the last significant coefficient and is assumed to be 1 for the top-left group (containing the DC coefficient). If the flag is 1, then the following syntax elements pertaining to the group are coded immediately following it:

  - Significance map:
For each coefficient in the group, a flag is coded indicating whether or not the coefficient is significant (has a non-zero value). No flag is necessary for the coefficient indicated by the last-significant position.

  - Greater-than-one map:
For up to eight coefficients with significance map value 1 (counted backwards from the end of the group), this indicates whether the magnitude is greater than 1.

  - Greater-than-two flag:
For up to one coefficient with greater-than-one map value 1 (the one nearest the end of the group), this indicates whether the magnitude is greater than 2.

  - Sign bits:
For all non-zero coefficients, sign bits are coded as equiprobable CABAC bins, with the last sign bit (in reverse scan order) possibly being instead inferred from parity when sign bit hiding is used.

  - Escape codes:
For any coefficient whose magnitude was not completely described by an earlier syntax element, the remainder is coded as an escape code.

For a 16-bit, 14-bit or even 12-bit system at the operating point where the MSE is less than 1 (typically at QPs -34, -22 and -10 respectively), the system typically yields very little compression (for 16-bit, it actually inflates the source data). The coefficients are generally large numbers, and therefore are almost always escape-coded.

For that reason, two proposed changes have been made to the entropy coder to allow for higher bit depths, as described in the following two sections.

## Entropy Coding of Fixed Bits

To increase efficiency at higher bit depths, the process of the HEVC entropy coder is changed to the following scheme, where a number of fixed bits BF for each significant coefficient are bypass-coded. This means that the CABAC syntax elements following the significance map, apply to only the most significant part of each coefficient, and therefore are still useful at the higher (lower QPs) operating points. In essence, the CABAC engine that was developed for 8 and 10-bit operation can be used as it was originally intended.

The changes to the current HEVC process (see above) are indicated below in bold:

- The location of the last significant coefficient (in scan order) in the TU is coded.

- For each 4x4 coefficient group (groups are processed in reverse scan order), a significant-coefficient-group flag is coded, indicating whether or not the group contains non-zero coefficients. This is not required for the group containing the last significant coefficient and is assumed to be 1 for the top-left group (containing the DC coefficient). If the flag is 1, then each significant coefficient C is split into a most-significant part CMSB and a least-significant part CLSB, where
CMSB = (abs(C) – 1) >> BF
and
CLSB = (abs(C) – 1) – (CMSB << BF)
and the following syntax elements pertaining to the group are coded immediately following it:

  - Significance map:
For each coefficient in the group, a flag is coded indicating whether or not the coefficient C is significant (has a non-zero value). No flag is necessary for the coefficient indicated by the last-significant position.

  - Greater-than-one map:
For up to eight coefficients with significance map value 1 (counted backwards from the end of the group), this indicates whether CMSB is greater than or equal to 1.

  - Greater-than-two flag:
For up to one coefficient with greater-than-one map value 1 (the one nearest the end of the group), this indicates whether CMSB is greater than or equal to 2.

  - Sign bits:
For all non-zero coefficients, sign bits are coded as equiprobable CABAC bins, with the last sign bit (in reverse scan order) possibly being instead inferred from parity when sign bit hiding is used.

  - Fixed bits:
For each non-zero coefficient in the group, the value of CLSB is coded.

  - Escape codes:
For any coefficient whose magnitude was not completely described by an earlier syntax element, the remainder is coded as an escape code.

The value of BF would be signalled by the encoder in the slice header (with the sequence parameter set (SPS) indicating that the feature is enabled), since the best value is source dependent, and some encoders may wish to adaptively control this function. However, a simple derivation for BF based on QP is as follows:

BF = max(0, int(QP / -6))

The results using this simple derivation for the fixed-bit scheme are shown in Figure 9.

Figure 9 - Bit rate vs. green channel PSNR for OldTownCross with 12-bit transform matrices, MAX_TR_DYNAMIC_RANGE = 22 and ENTROPY_CODING_DYNAMIC_RANGE = 22 (RDOQ disabled, transform-skip disabled) with proposed scheme.

The saving in bit rate for the system with fixed bits (relative to the system without fixed bits) varies from 4% at QP -6 (10-bit) up to 35% at QP -48 (16-bit).

Enabling transform-skip produces similar, though slightly more irregular, bit rate/PSNR curves. Other settings can be explored in the accompanying Excel spreadsheet.

## CABAC Bit Alignment

Since the quantity of data being coded is somewhat higher than observed for HEVC (FDIS) operating points, an additional stage has been implemented.

This additional stage causes the CABAC stream to be bit-aligned prior to coding the bypass data for each coefficient group. This allows quicker (and in-parallel) decoding of the bypass data, since the values can now be read directly out of the stream, removing the requirement for long-division when decoding bypass bins.

### HEVC CABAC Entropy Coding of EP bins

The current HEVC (FDIS) CABAC entropy coder decodes one EP bin as follows:

ivlOffset = (ivlOffset<<1) | read_bits(1)

if ( ivlOffset >= ivlCurrRange )

the decoded EP value (binVal) is a 1. ivlOffset -= ivlCurrRange

else

the decoded EP value (binVal) is a 0.

A worked example of this process, decoding two bins is:

let ivlCurrRange=458 and ivlOffset=303, and the next two bits in the stream are both 1.

cycle 1:

ivlOffset = (303*2)+1 = 607

ivlOffset >= 458, so the EP value is 1. ivlOffset = (ivlOffset – 459) = 149

cycle 2:

ivlOffset = (149*2)+1 = 299

ivlOffset < 458, so the EP value is 0 (ivlOffset unchanged)

As can be seen, to decode two EP values, this sequential process is followed, akin to a long division.

However, for the cases where ivlCurrRange happens to be the value 256 at the point of decoding EP bins, then the algorithm can be simplified to the following:

To read n EP bits,

EP bits = top n bits but 1 of ivlOffset.

Apply a left shift of n to ivlOffset (except for most significant bit, which must remain as a 0), and insert n bits into the LSBs.

If ivlCurrRange can always be made to be 256, this latter implementation would give significant throughput and complexity advantages. For example, all of the sign bits for a coefficient group could be read and decoded in a single operation. A decoder would also be able to decode the worst-case exponential-Golomb code in a single operation on the bit-stream. The algorithm described and developed in the next sections implements the alignment procedure, which achieves this goal.

### Algorithm

The only step required to bit-align the stream is:

ivlCurrRange = 256		(in both the encoder and decoder)

As already stated, the result of this is that the encoding and decoding process is significantly simplified; allowing the binary data to be read directly out of ivlOffset in raw form, and therefore many bits at a time can simultaneously be processed by the decoder.

However, note that the act of setting ivlCurrRange to 256 incurs a loss of, on average, 0.55 bits:

if ivlCurrRange was already 256, there is no loss;
if ivlCurrRange was 510, then approximately 1 bit will be lost;
the average over all valid values of ivlCurrRange is 0.55 bits (the loss is not exactly linear with the value of ivlCurrRange).

Applying this algorithm prior to each coefficient group’s bypass (and sign) data would give a noticeable loss at the normal HEVC 8-bit operating points; however, when considering higher bit depths where there is considerably more bypass data, this loss is less significant, and the requirement to increase throughput is more important. The effects on coding efficiency of this algorithm can be seen in the accompanying Excel spreadsheet; graphs are not included here as the differences are negligible.

To increase the range of operating points where this alignment can be used, the algorithm is further developed, as described in the next section.

### Algorithm Refinement

The algorithm described in Section  has been further refined to reduce the average cost from 0.55 bits to 0.30 bits (and further).

In the above algorithm, ivlCurrRange was always aligned to 256. However, as an alternative, ivlCurrRange can be aligned to one of a number N of predetermined values V={vi}. If the values are regularly spaced over the range [256, 512), (for example V={256, 384} for N=2), then the loss-per-alignment is approximately (0.5 / N) + 0.05.

Since ivlCurrRange is no longer a simple power of 2, this would still require division, although the use of look-up tables could be employed. However, to circumvent this need for division, the bin (or bins) immediately following the alignment process can be coded using (unequal) symbol ranges that are powers of two. This would not result in any further loss in bit efficiency (i.e. the loss-per-alignment is still only (0.5 / N) + 0.05).

The proposed refinement is to align to 256 and 384 (N=2): when ivlCurrRange is less than 384 the same process as in Section  is used, otherwise the following process is used:

When aligning to 384, the symbol ranges for [0,1] for the subsequent bin are [256,128].
If a 0 is coded, ivlCurrRange is set to 256, making the cost to encode the bin 0.5 bits.
If a 1 is coded, the range is set to 128 (and 256 is added to ivlOffset) and the system is renormalised (again, ivlCurrRange becomes 256), making the cost to encode the bin 1.5 bits.

Since 0 and 1 are expected with equal probability, the average cost to encode the bin immediately following alignment is still 1 bit. For equi-probable (EP) bins, 1 bit is expected per bin.

This proposed refinement is illustrated in Figure 10, which gives a loss of 0530 (=(0.5 / N) + 0.05) bits for alignment. This loss is confirmed by experiment, and can be explored in the accompanying Excel spreadsheet.

Figure 10 - Bit-alignment procedure and subsequent symbol ranges for alignment case where N = 2

The algorithm for the decoder to align using the N=2 refinement can be expressed as:

if (ivlCurrRange >= 384)

if (ivlOffset < 256)		nextEPBitIsZero = true	Note

else 				ivlOffset -= 128		Note

ivlCurrRange = 256

In addition, Figure 11 illustrates the case where N=4 (i.e. V={256, 320, 384, 448}), which gives an average loss of 0.15 bits per alignment. A similar decoder algorithm can be constructed, although the implementation complexity is increased, and is therefore not recommended.

Figure 11 - Bit-alignment procedure and subsequent symbol ranges for alignment case where N = 4

### Further Bit-loss Mitigation Methods

A number of methods can be used to further mitigate the incurred loss:

- Whether or not to bit-align could be selected depending on an estimate of the expected quantity of bypass-coded data (for example, based on the number of greater-than-one flags equal to 1): if there is less bypass data, then throughput would be less important, and therefore alignment would be unnecessary (and vice versa).

- Rather than code bypass data at the end of each coefficient group, all the bypass data for a TU can be coded together after the CABAC bin data for the TU. The loss is therefore 0.55 bits per coded TU, rather than 0.55 bits per coefficient group.

These further bit-loss mitigation methods are not proposed as this time.

# Profiles

As a starting point for discussion regarding profiles to be used in Range Extensions, the following options are proposed:

*These profiles are defined in HEVC (FDIS).

Note that MAX_TR_DYNAMIC_RANGE may exhibit non-monotonic behaviour at low QPs for some sequences. This behaviour is alleviated when MAX_TR_DYNAMIC_RANGE is set to the same value as ENTROPY_CODING_DYNAMIC_RANGE.

Range Extension Coding Tools to include the two entropy coding modifications described in Sections 6.1 and 6.2.

The points of the above new profiles are,

- Main10 4:0:0, Main10 4:2:2 and Main 10 4:4:4 are for the chroma format extension of the FDIS version. To simplify the implementation, no new tools are included (no LM chroma mode, additional lossless coding tools, etc.), and only changes associated with supporting the additional chroma sampling formats are included.

- High profiles support greater bit depths, and would give the opportunity for new tools to be defined (which may improve coding efficiency or throughput or reduce complexity). New tools and higher internal precisions (for example those proposed in this document) would be included in these profiles.

- In addition to these profiles, Main 10 Intra or High 12 Intra profiles could also be defined.

If the High 16 profiles are not restricted to a minimum bitDepth of 13 bit data, then with the profiles proposed above, there are two methods of encoding 12-bit data (i.e. using High 12 and High 16 parameters). Unless there is a significant advantage to coding the 12-bit data with a High 16 encoder, it would be better for interoperability if the 12-bit data was coded in a manner that could be decoded by the High 12 decoder (i.e. in High 12 profile).

No significant advantage is observed when encoding 12-bit data using a 16-bit encoder, as shown in Figure 7 and Figure 8. Considering this, it may be better to either constrain the 16-bit profiles so that they do not include support for bitDepths of 12 and below, or to adjust the definition of the profiles, as shown below. Either method would result in fewer conformance points.

Note: where the values of MAX_TR_DYNAMIC_RANGE and ENTROPY_CODING_DYNAMIC_RANGE have a minimum of 15.

However, as discussed earlier, for some sequences at the lower QPs, it is better to use a MAX_TR_DYNAMIC_RANGE equal to the ENTROPY_CODING_DYNAMIC_RANGE, as these settings reliably give a better response curve for rate control:

The result of these alternative definitions is that the 12-bit bit-streams are identical, whether they were encoded by a High 12 encoder or by a High 16 encoder.

These definitions would also aid implementation of encoders wishing to implement just (for example) 14 bits, since they do not require the additional internal accuracy for the 16-bit operation. To keep decoder overheads low, and as discussed in Section , only one transform matrix precision over that defined by HEVC (FDIS) is considered in this table; there should be little overhead in decoder implementation of using different MAX_TR_DYNAMIC_RANGEs and ENTROPY_CODING_DYNAMIC_RANGEs.

A comparison of the proposed system against base HEVC and AVC can be found in the accompanying Excel spreadsheet “High Bit Depth AVC Comparison.xlsm”, for the cases of PCM enabled and disabled. As can be seen, due to the fixed bit processing, PCM is now almost never chosen for any block for the proposed system (the bit-rates and SNRs are effectively unaffected by whether PCM is enabled or disabled).

In addition, a comparison of the proposed system against base HEVC can be found in the accompanying Excel spreadsheet “High Bit Depth Base HEVC Comparison.xlsm” for the standard 10 frames and all of the frames of the test sequence. Although there are slight differences between the 10 frame and all frame results, the trends of the 10 frame set can be seen to be representative of all the frames.

# Conclusion

In summary, the following changes to HEVC Range Extensions are proposed in order to support higher bit depths:

- Increase internal accuracies using profiles detailed in Section 7.

- Modify entropy coding to include the fixed-bit algorithm specified in Section 6.1.

  - The presence of fixed bit should be specified in the SPS.

  - The number of fixed bits BF should be specified in the slice header.

- Modify entropy coding to include the refined bit-alignment algorithm specified in Section 6.2.3.

  - The use of bit-alignment should be specified in the SPS.

    - Alternatively, this could be defined by the profile/level.

The base HEVC Range Extensions system that is limited to about 90dB has been extended to beyond 120dB. The various sources of error have been studied and recommendations have been made. In addition, changes to improve coding efficiency have been presented, and changes to improve throughput have also been presented.

With PCM enabled, these combined changes have resulted in ~11% BD-rate saving at 75 dB (12-bit), ~21% at 85 dB (14-bit) and ~18% at 95 dB (also at 14-bit) (16-bit values are currently unavailable). With PCM disabled, these changes have resulted in ~12% BD-rate saving at 75 dB, ~27% at 85 dB, and noting that beyond 90 dB, no BD-rate calculation is possible.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Increased-Precision Transform Matrices

This appendix details the transform matrices at various levels of precision as referred to in Section 5.2.

## 4x4 DST

The transform matrix is of the form:

where the values in the grid are defined by the matrix precision according to the following table (6-bit HEVC (FDIS) matrix values included for comparison):

|  | 6-bit | 7-bit | 8-bit | 9-bit | 10-bit | 11-bit | 12-bit | 13-bit | 14-bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | 29 | 58 | 117 | 233 | 467 | 934 | 1868 | 3736 | 7472 |
| b | 54 | 110 | 219 | 439 | 878 | 1755 | 3510 | 7021 | 14042 |
| c | 73 | 148 | 296 | 591 | 1182 | 2365 | 4730 | 9459 | 18919 |
| d | 83 | 168 | 336 | 672 | 1345 | 2689 | 5378 | 10757 | 21513 |
| e | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## Combined DCT Matrix

For ease of implementation, a single 32x32 DCT matrix M32 can be described, from which each smaller NxN DCT matrix MN is derived through subsampling according to the following:

MN[x][y] = M32[x][(2(5 – log2(N)))y]      for x, y = 0..(N – 1).

The combined matrix M32 is of the form:

with the values in the grid defined by the matrix precision according to the following table (6-bit HEVC (FDIS) matrix values included for comparison):

|  | 6-bit | 7-bit | 8-bit | 9-bit | 10-bit | 11-bit | 12-bit | 13-bit | 14-bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | 64 | 128 | 256 | 512 | 1024 | 2048 | 4096 | 8192 | 16384 |
| b | 83 | 167 | 334 | 669 | 1338 | 2676 | 5352 | 10703 | 21407 |
| c | 36 | 69 | 139 | 277 | 554 | 1108 | 2217 | 4433 | 8867 |
| d | 89 | 178 | 355 | 710 | 1420 | 2841 | 5681 | 11363 | 22725 |
| e | 75 | 151 | 301 | 602 | 1204 | 2408 | 4816 | 9633 | 19266 |
| f | 50 | 101 | 201 | 402 | 805 | 1609 | 3218 | 6436 | 12873 |
| g | 18 | 35 | 71 | 141 | 283 | 565 | 1130 | 2260 | 4520 |
| h | 90 | 180 | 360 | 721 | 1441 | 2882 | 5765 | 11529 | 23059 |
| i | 87 | 173 | 346 | 693 | 1386 | 2772 | 5543 | 11086 | 22173 |
| j | 80 | 160 | 319 | 639 | 1277 | 2554 | 5109 | 10217 | 20435 |
| k | 70 | 140 | 280 | 560 | 1119 | 2239 | 4478 | 8956 | 17911 |
| l | 57 | 115 | 230 | 459 | 919 | 1837 | 3675 | 7350 | 14699 |
| m | 43 | 85 | 171 | 341 | 683 | 1365 | 2731 | 5461 | 10922 |
| n | 25 | 53 | 105 | 210 | 420 | 841 | 1682 | 3363 | 6726 |
| o | 9 | 18 | 35 | 71 | 142 | 284 | 568 | 1136 | 2271 |
| p | 90 | 181 | 362 | 723 | 1446 | 2893 | 5786 | 11571 | 23143 |
| q | 90 | 179 | 358 | 716 | 1432 | 2865 | 5730 | 11460 | 22920 |
| r | 88 | 176 | 351 | 702 | 1405 | 2810 | 5619 | 11238 | 22476 |
| s | 85 | 170 | 341 | 682 | 1364 | 2727 | 5454 | 10908 | 21816 |
| t | 82 | 164 | 327 | 655 | 1309 | 2618 | 5236 | 10473 | 20946 |
| u | 78 | 155 | 311 | 621 | 1242 | 2484 | 4968 | 9937 | 19874 |
| v | 73 | 145 | 291 | 582 | 1163 | 2326 | 4653 | 9305 | 18611 |
| w | 67 | 134 | 268 | 537 | 1073 | 2146 | 4292 | 8584 | 17168 |
| x | 61 | 122 | 243 | 486 | 973 | 1945 | 3890 | 7780 | 15560 |
| y | 54 | 108 | 216 | 431 | 863 | 1725 | 3451 | 6901 | 13803 |
| z | 46 | 93 | 186 | 372 | 745 | 1489 | 2978 | 5956 | 11912 |
| A | 38 | 77 | 155 | 310 | 619 | 1238 | 2477 | 4953 | 9907 |
| B | 31 | 61 | 122 | 244 | 488 | 976 | 1951 | 3903 | 7806 |
| C | 22 | 44 | 88 | 176 | 352 | 704 | 1407 | 2815 | 5630 |
| D | 13 | 27 | 53 | 106 | 212 | 425 | 850 | 1700 | 3400 |
| E | 4 | 9 | 18 | 36 | 71 | 142 | 284 | 568 | 1137 |

For information, the smaller DCT matrices derived from the 32x32 matrix are presented here. The values in each grid are defined by the matrix coefficient precision according to the above table.

## 4x4 DCT

The matrix M4 is defined as the first 4 coefficients of every 8th row of the combined matrix M32.

## 8x8 DCT

The matrix M8 is defined as the first 8 coefficients of every 4th row of the combined matrix M32.

## 16x16 DCT

The matrix M16 is defined as the first 16 coefficients of every even row of the combined matrix M32.

This stage actually decodes a 0 from the bit-stream, which will be the next bit returned by the equi-probable bit decoding process.

This stage has adjusted ivlOffset, so that the next decoded EP bin will always produce a 1, as required.