| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0185 |
| --- | --- |

| Title: | SCE4: Cross-check results of test 4.1.1 on integer sample filtering for SNR scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250 San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document is the cross-check report on Samsung's proposal on integer sample filtering for the inter layer processing of SNR scalability, described in Section 4.1.1 of SCE4 document. We checked the implementation of the algorithm, the implementation of complexity assessment modules, the R-D performance, the average complexity results obtained by decoding, and the worst case complexity calculation. The algorithm submitted by the proponent is quite different from what is supposed to be tested in SCE4 4.1.1 (i.e., JCTVC-L0087). Based on the modified algorithm, we did the simulation and found the performance and complexity results match what are provided by the proponent.

# Introduction

In the previous meeting held in Geneva, SCE4 was established to evaluate a set of coding tools for inter layer prediction. It was agreed that JCTVC-L0087 was tested in SCE4 and defined in Section 4.1.1 of SCE4 document. However, the method submitted by the proponent for cross-check is quite different from JCTVC-L0087. There are two major differences as below.

First, in JCTVC-L0087, a 7-tap filter [ 1 -5 11 50 11 -5 1 0] / 64 is used to filter the integer pixels of the inter layer picture for SNR scalability. In the SCE4 submission, the proponent used a 5-tap filter [1 3 12 3 -1]/16 to replace the previous 7-tap filter. So both filter length and precision are changed.

Second, in JCTVC-L0087, integer pixel filtering is forcefully used for all the pixels in SNR scalability. In the SCE4 submission, integer pixel filtering is still forcefully used in ref_idx framework, but in IntraBL framework it is signaled by a 1-bit flag on a CU basis.

According to the meeting notes, SCE submission should be the same as what was proposed in the Geneva meeting, and any variant should be considered as a non-CE proposal. The aforementioned implementation discrepancy was brought up to SCE4 mailing list and also discussed within SCE4 coordinators. It was agreed that the implementation discrepancy did exist, but there was no consensus on whether the modified method should be considered as a valid SCE4 submission.

Our verifications of performance and complexity are based on the modified method. First, the implementation of the method is consistent with the submitted description. Second, the implementation of the complexity assessment module in the SHM-1.0 decoder and the calculation of worst case complexity correctly reflect the complexity of the proposed method.

# Experimental Results

The cross-check experiments were conducted on 64-bit Linux cluster under SHVC CTC and the additional test condition defined by SCE4. The results exactly match what are reported by the proponent. The detailed data are shown in the enclosed spreadsheet. The summary is shown as below.

Table 1 BD-rate with IntraBL framework

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -3.8% | -0.7% | -0.6% |
| Class B | -1.0% | 0.1% | 0.3% |
| Overall (Test vs Ref) | -1.8% | -0.1% | 0.0% |
| Overall (Test vs single layer) | 12.7% | 28.7% | 32.7% |
| EL only (Test vs Ref) | -2.7% | -0.9% | -0.7% |
| Enc Time[%] | 101.1% |  |  |
| Dec Time[%] | 98.2% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -4.9% | -0.9% | -0.7% |
| Class B | -1.4% | 0.2% | 0.5% |
| Overall (Test vs Ref) | -2.4% | -0.1% | 0.2% |
| Overall (Test vs single layer) | 19.3% | 34.3% | 39.4% |
| EL only (Test vs Ref) | -3.3% | -0.9% | -0.6% |
| Enc Time[%] | 98.4% |  |  |
| Dec Time[%] | 96.2% |  |  |
| BL Match | Matched |  |  |
|  | LD-B HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -3.4% | -0.7% | -0.7% |
| Class B | -0.8% | -0.1% | 0.2% |
| Overall (Test vs Ref) | -1.5% | -0.3% | -0.1% |
| Overall (Test vs single layer) | 21.8% | 33.1% | 37.0% |
| EL only (Test vs Ref) | -2.1% | -0.8% | -0.6% |
| Enc Time[%] | 96.2% |  |  |
| Dec Time[%] | 97.2% |  |  |
| BL Match | Matched |  |  |

TABLE 2 BD-rate with RefIdx framework

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -3.7% | -0.9% | -0.9% |
| Class B | 0.2% | -0.5% | -0.6% |
| Overall (Test vs Ref) | -0.9% | -0.6% | -0.7% |
| Overall (Test vs single layer) | 14.7% | 29.2% | 33.9% |
| EL only (Test vs Ref) | -1.3% | -1.0% | -1.1% |
| Enc Time[%] | 104.2% |  |  |
| Dec Time[%] | 132.0% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -4.9% | -1.1% | -0.9% |
| Class B | -0.8% | -0.7% | -0.5% |
| Overall (Test vs Ref) | -2.0% | -0.8% | -0.7% |
| Overall (Test vs single layer) | 20.9% | 33.5% | 38.5% |
| EL only (Test vs Ref) | -2.7% | -1.5% | -1.3% |
| Enc Time[%] | 106.5% |  |  |
| Dec Time[%] | 139.2% |  |  |
| BL Match | Matched |  |  |
|  | LD-B HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -3.2% | -0.8% | -0.7% |
| Class B | -0.1% | -0.9% | -1.1% |
| Overall (Test vs Ref) | -1.0% | -0.9% | -1.0% |
| Overall (Test vs single layer) | 23.1% | 33.5% | 38.1% |
| EL only (Test vs Ref) | -1.3% | -1.2% | -1.3% |
| Enc Time[%] | 107.7% |  |  |
| Dec Time[%] | 146.5% |  |  |
| BL Match | Matched |  |  |

Table 3 Average complexity with IntraBL framework

|  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 102% | 100% | 100% | 106% | 105% |
| Class B | 101% | 100% | 100% | 103% | 103% |
| Overall | 101% | 100% | 100% | 104% | 104% |
|  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 104% | 101% | 101% | 112% | 111% |
| Class B | 103% | 101% | 101% | 107% | 107% |
| Overall | 103% | 101% | 101% | 109% | 108% |
|  | LD-B HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 102% | 100% | 100% | 107% | 107% |
| Class B | 101% | 100% | 100% | 103% | 103% |
| Overall | 101% | 100% | 100% | 104% | 104% |

Table 4 Average complexity with RefIdx framework PU-based implementation

|  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 102% | 100% | 100% | 107% | 106% |
| Class B | 102% | 102% | 102% | 105% | 105% |
| Overall | 102% | 101% | 101% | 106% | 105% |
|  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 103% | 100% | 100% | 112% | 111% |
| Class B | 104% | 102% | 102% | 110% | 109% |
| Overall | 103% | 102% | 101% | 110% | 110% |
|  | LD-B HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 102% | 100% | 100% | 108% | 108% |
| Class B | 103% | 102% | 102% | 107% | 106% |
| Overall | 102% | 101% | 101% | 107% | 107% |

Table 5 Average complexity with RefIdx framework Picture-based implementation

|  | RA HEVC SNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 137% | 125% | 124% | 128% | 126% |
| Class B | 140% | 132% | 131% | 126% | 124% |
| Overall | 139% | 130% | 129% | 127% | 125% |
|  | LD-P HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 160% | 140% | 138% | 149% | 146% |
| Class B | 166% | 149% | 147% | 148% | 145% |
| Overall | 164% | 147% | 145% | 148% | 145% |
|  | LD-B HEVC SNR |  |  |  |  |
|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| Class A | 139% | 126% | 124% | 132% | 130% |
| Class B | 143% | 133% | 132% | 129% | 128% |
| Overall | 142% | 131% | 129% | 130% | 128% |

Table 6 Worst case compared with SHM-1.0

| ref_idx picture-based implementation |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SNR Scalability |  |  |  |  |  |
|  | SHM |  | Proposed |  | Increase |  |
| Mult | 0 |  | 11 |  | INF |  |
| Adds | 0 |  | 9 |  | INF |  |
| MemBand(2D:4×2) | 2 |  | 3 |  | 130% |  |
| MemBand(2D:8×2) | 3 |  | 4 |  | 140% |  |
| ref_idx PU-based implementation |  |  |  |  |  |  |
|  | SNR Scalability |  |  |  |  |  |
|  | SHM |  | Proposed |  | Increase |  |
| Mult | 0 |  | 15 |  | INF |  |
| Adds | 0 |  | 12 |  | INF |  |
| MemBand(2D:4×2) | 4 |  | 7 |  | 169% |  |
| MemBand(2D:8×2) | 8 |  | 10 |  | 125% |  |
| IntraBL implementation |  |  |  |  |  |  |
|  |  | SNR Scalability |  |  |  |  |
|  |  | SHM |  | Proposed |  | Increase |
| Mult |  | 0 |  | 15 |  | INF |
| Adds |  | 0 |  | 12 |  | INF |
| MemBand(2D:4×2) |  | 4 |  | 5 |  | 154% |
| MemBand(2D:8×2) |  | 7 |  | 8 |  | 114% |