| cJoint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0188r1 |
| --- | --- |

| Title: | Upsampling based on sampling grid information for aligned inter layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye, and Yuwen He 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution presents an upsampling scheme achieving aligned sampling grids between the EL and the upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift. The proposed upsampling scheme includes signalling the sampling grid offsets between layers in the bitstream and incorporating the signaled offsets into phase filter selection. Simulation results show that, when BL and EL sequences have non-zero phase shift, the proposed scheme outperforms SHM-1.0 significantly. In the RefIdx framework, the average {Y, U, V} gains are {-8.8%, -10.4%, -10.5%}, {-6.6%, -4.8%, -4.4%}, {-6.2%, -4.1%, -4.0%}, and {-5.4%, -3.7%, -3.6%} for AI, RA, LDP, and LDB, respectively. Similar performance gain can be observed for the IntraBL framework.

# Problem Statement

In SHVC Test Model 1 [1], upsampling for inter layer prediction is design based on an assumption that the sampling grids of BL and EL pictures have no phase shift, as shown in Figure 1. To satisfy this assumption, JCT-VC provides the downsampler in [2], which downsamples EL sequences and generates the zero-phase shifted BL sequences. However, the assumption doesn’t always hold, because the downsampler in [2] is not mandated, and an SHVC codec is expected to be able to take input BL and EL sequences with non-zero phase shift. When such is the case, using the current upsampling process in [1], the sampling grids between the EL and the upsampled BL pictures are misaligned, which causes inaccurate inter layer prediction and thus loss of scalable coding performance.

(a)                                                  (b)

Figure 1 Overlaid sampling grids of BL (blue circles) and EL (white squares) pictures. (a) 2x spatial scalability; (b) 1.5x spatial scalability

There is another problem in SHVC Test Model 1 [1]. In Section G.6.2, a sample location (xRef16, yRef16) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture is calculated as below.

The variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW                                                       (G-3)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = (yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH                                                          (G-4)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset                                            (G-5)

where the value of offset is derived as follows:

if (ScaledH  is equal to PicHRL )
			offset = 0
		otherwise if (ScaledH  is equal to 1.5 * PicHRL )
			offset = 1
		otherwise if (ScaledH  is equal to 2.0 * PicHRL )
			offset = 2

In SHM-1.0 software, the sample location (xRef16, yRef16) is calculated by division-free operations, as below.

The variable xRef16 is derived as follows:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = (xP*iScaleX + iAddX) >> iShiftXM4;

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5 );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = (yP*iScaleY + iAddY) >> iShiftYM4;

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ((yP*iScaleY + iAddY) >> iShiftYM4) - iDeltaY;

The sample location (xRef16, yRef16) calculated by the method in SHVC Test Model 1 [1] and the method in SHM-1.0 should be the same, but it is not true currently. For example, using  the method in SHM-1.0 to calculate yRef16 with yP=1, 4, 7, 10, ..., in chroma planes for 1.5x spatial scalability, one uses the filter with phase index 9, but the phase index is 10, if calculated by the method in SHVC Test Model 1 [1]. Note that filter with phase index 10 is not defined in Table G-2 in [1], shown as below. This is a problem to be fixed in the current SHVC Test Model 1 [1].

Table 1 16-phase chroma resampling filter in Table G-2 of SHVC Test Model 1

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | n/a | n/a | n/a | n/a |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | n/a | n/a | n/a | n/a |
| 11 | −2 | 20 | 26 | −6 |
| 12 | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

# Proposed Algorithm

## Problem Fix

To fix the problem as mentioned in Section 1, we propose to change (G-5) as below. The changed place is highlighted.

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset                                            (G-5)

With this change, filter with phase index 9 is used to generate yP=1, 4, 7, 10, ..., in chroma planes for 1.5x spatial scalability, and the specification in SHVC Test Model and the software SHM-1.0 produce identical results.

## Upsampling based on sampling grid information

As mentioned in Section 1, there is no sampling grid information transmitted in the bitstream and the current upsampling design assumes zero phase shift between spatial layers. To solve the problem, we propose two options to signal the sampling grid offsets between spatial layers, as introduced in Section 2.2.1. Then we propose a method to incorporate the sampling grid offsets into phase filter selection of the upsampling process, such that the EL and upsampled BL pictures are always aligned, as introduced in Section 2.2.2.

### Sampling grid information signaling

Figure 2 gives an example of overlaid luma sampling grids of BL picture (blue circle) and EL picture (white square) with non-zero phase shift in 2x spatial scalability. The precision of the inter layer phase shift is 1/16-pixel in terms of BL sampling grid. The relative positions of the upper-left pixels in BL and EL pictures are examined, and the sampling grid offsets in the horizontal and vertical directions are determined by (1) and (2), respectively,

OffsetHori  =  PositionHori,BL - PositionHori,EL                                               (1)

OffsetVert  =  PositionVert,BL - PositionVert,EL                                                (2)

where PositionHori,BL and PositionHori,EL represent the horizontal positions of the upper-left pixels in the BL and EL pictures, respectively, and PositionVert,BL and PositionVert,EL represent the vertical positions of the upper-left pixels in the BL and EL pictures, respectively. In the example of Figure 2, the sampling grid offsets OffsetHori and OffsetVert are both equal to 4. If the BL and EL sampling grids have zero-phase shift, as shown in Figure 1, the sampling grid offsets have the default values 0.

Figure 2 Overlaid luma sampling grids of BL picture (blue circle) and EL picture (white square) with non-zero phase shift in 2x spatial scalability.

In Table 2, we propose the syntax of the sampling grid information, followed by the semantics.

Table 2 Syntax of sampling grid information

| sampling_grid_information( ) { | Descriptor |
| --- | --- |
| sampling_grid_info_present_flag | u(1) |
| if ( sampling_grid_info_present_flag ) { |  |
| sampling_grid_abs_offset_x | u(4) |
| sampling_grid_abs_offset_y | u(4) |
| sampling_grid_sign_x | u(1) |
| sampling_grid_sign_y | u(1) |
| } |  |

sampling_grid_info_present_flag equal to 1 specifies sampling_grid_abs_offset_x, sampling_grid_abs_offset_y, sampling_grid_sign_x, and sampling_grid_sign_y are presented in sampling_grid_information(), sampling_grid_info_present_flag equal to 0 specifies sampling_grid_abs_offset_x, sampling_grid_abs_offset_y, sampling_grid_sign_x, and sampling_grid_sign_y are not presented in sampling_grid_information(). When not present, sampling_grid_info_present_flag is inferred to be equal to 0.

sampling_grid_abs_offset_x and sampling_grid_sign_x together specify the value of variable SamplingGridOffsetX as follows:

SamplingGridOffsetX  = (1 - sampling_grid_sign_x * 2) * sampling_grid_abs_offset_x

The value of sampling_grid_abs_offset_x should be in the range of 0 to 15, inclusive.

sampling_grid_abs_offset_y and sampling_grid_sign_y together specify the value of variable SamplingGridOffsetY as follows:

SamplingGridOffsetY  = (1 - sampling_grid_sign_y * 2) * sampling_grid_abs_offset_y

The value of sampling_grid_abs_offset_y should be in the range of 0 to 15, inclusive.

We propose two options of where to signal sampling_grid_information( ).

Option 1: sampling_grid_information( ) can be signaled as part of SPS, as shown in Table 3.

Table 3 Signaling sampling grid information in SPS

| sps_extension ( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sampling_grid_information() |  |
| } |  |

Option 2: sampling_grid_information( ) can be signaled in VPS extension, which is used to specify the high level information about the scalable bitstream, e.g., the dependency information between layers. When signaled in VPS, the relative sampling grid offsets between the current layer and one or more of its reference layers can be signaled together with the layer dependency information, as shown in Table 4.

Table 4 Signaling sampling grid information in VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| ...... |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) { |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if (direct_dependency_flag[ i ] [ j ] ) |  |
| sampling_grid_information( i , j ) |  |
| } |  |

### Upsampling using signaled sampling grid offsets

As introduced in Section 1, the sample location (xRef16, yRef16) calculated by either method in SHVC Test Model 1 or SHM-1.0, referred to as default location, is only accurate when the BL and EL sampling grids have no phase shift; in other words, zero phase shift downsampling filters are used. The default location should be adjusted by the sampling grid offsets that we propose to be transmitted in SPS extension or VPS extension (see Section 2.2.1), when the BL and EL sampling grids have non-zero phase shift. The adjustment can be summarized in two aspects.

–	For luma, the accurate location is obtained by subtracting the default location by the offset.

–	For chroma, the accurate location is obtained by substracting the default location by half of the offset.

In the following, the detailed adjustments for SHVC Test Model 1 and SHM-1.0 are introduced, which provide identical results.

In SHVC Test Model 1, the calculation in Section G.6.2 of [1] is proposed to be changed as below. The changed places are highlighted.

The variable xRef16 is derived as follows:

–	If cIdx is equal to 0, the variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW- SamplingGridOffsetX                 (G-3)

–	Otherwise, the variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW- SamplingGridOffsetX/2              (G-4)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = (yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – SamplingGridOffsetY                  (G-5)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset – SamplingGridOffsetY/2 (G-6)

where the value of offset is derived as follows:

if (ScaledH  is equal to PicHRL )
			offset = 0
		otherwise if (ScaledH  is equal to 1.5 * PicHRL )
			offset = 1
		otherwise if (ScaledH  is equal to 2.0 * PicHRL )
			offset = 2

The calcuation in SHM-1.0 is changed as below. The changed places are highlighted.

The variable xRef16 is derived as follows:

–	If cIdx is equal to 0, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) - SamplingGridOffsetX;

–	Otherwise, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) - SamplingGridOffsetX/2;

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5 );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ( (yP*iScaleY + iAddY) >> iShiftYM4) - SamplingGridOffsetY;

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = ((yP*iScaleY + iAddY) >> iShiftYM4) – iDeltaY - SamplingGridOffsetY/2;

Since the sampling grid offsets, which ranges from -15 to 15, are considered in phase filter selection, the subset of the16 phase filters defined in the current SHVC Test Model [1] is not sufficient for all the possible offsets. To solve this problem, we designed all the missing phase filters, as shown in Table 5 and Table 6. The new phase filters are DCT-IF filters with the frequency characteristics close to the existing phase filters.

Table 5 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | -3 | 63 | 4 | -2 | 1 | 0 |
| 2 | -1 | 2 | -6 | 62 | 9 | -3 | 1 | 0 |
| 3 | -1 | 3 | -8 | 60 | 13 | -5 | 2 | 0 |
| 4 | -1 | 4 | -10 | 57 | 19 | -7 | 3 | -1 |
| 5 | -1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | -1 | 4 | -11 | 49 | 29 | -9 | 4 | -1 |
| 7 | -1 | 4 | -12 | 45 | 35 | -10 | 4 | -1 |
| 8 | -1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | -1 | 4 | -10 | 35 | 45 | -12 | 4 | -1 |
| 10 | -1 | 4 | -9 | 29 | 49 | -11 | 4 | -1 |
| 11 | -1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | -1 | 3 | -7 | 19 | 57 | -10 | 4 | -1 |
| 13 | 0 | 2 | -5 | 13 | 60 | -8 | 3 | -1 |
| 14 | 0 | 1 | -3 | 9 | 62 | -6 | 2 | -1 |
| 15 | 0 | 1 | -2 | 4 | 63 | -3 | 1 | 0 |

Table 6 16-phase chroma resampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | -2 | 62 | 4 | 0 |
| 2 | -2 | 58 | 10 | -2 |
| 3 | -4 | 56 | 14 | -2 |
| 4 | -4 | 54 | 16 | -2 |
| 5 | -6 | 52 | 20 | -2 |
| 6 | -6 | 46 | 28 | -4 |
| 7 | -4 | 42 | 30 | -4 |
| 8 | -4 | 36 | 36 | -4 |
| 9 | -4 | 30 | 42 | -4 |
| 10 | -4 | 28 | 46 | -6 |
| 11 | -2 | 20 | 52 | -6 |
| 12 | -2 | 16 | 54 | -4 |
| 13 | -2 | 14 | 56 | -4 |
| 14 | -2 | 10 | 58 | -2 |
| 15 | 0 | 4 | 62 | -2 |

# Experimental Results

The proposed scheme doesn’t influence the coding performance under the current CTC, because the BL and EL sequences have zero-phase shift. To show the efficiency of the proposed scheme in coding misaligned spatial layers, we generate the BL sequences using the downsampler in [3]. With this downsampler, the sampling grid offsets between BL and EL pictures are the same as those used during the development of H.264/AVC SVC extension, as shown in Figure 3. It was considered as the downsampler for SHVC development in the San Jose meeting, Feb. 2012, but was later replaced by the downsampler in [2], because the committee decided to use BL and EL sequences without phase shift. In the simulation, the sampling grid offsets for 2x and 1.5x spatial scalability are set to 4 and23, respectively, which are transmitted in SPS (as introduced in Option 1 of Section 2.2.1) and incorporated into phase filter selection (as introduced in Section 2.2.2). Except different BL sequences, other test conditions used in this experiment are the same as SHVC CTC. The scheme is integrated into SHM-1.0, and the changed places are marked by macro SAMPLING_GRID in the accompanying software package.

(a)                                                                (b)

Figure 3 Overlaid sampling grids of BL (blue circles) and EL (white squares) pictures used in the development of H.264/AVC SVC extension. (a) 2x spatial scalability; (b) 1.5x spatial scalability.

Table 7 and  summarize of the BD-rate performances in RefIdx and IntraBL frameworks, respectively. As can be seen, the coding performance is significantly improved by using aligned inter layer prediction achieved by the proposed upsampling scheme.

Table 7 BD-rate in RefIdx framework

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -14.3% | -16.1% | -16.8% |  |  |  |
| Class B | -7.8% | -9.0% | -8.8% | -8.0% | -9.8% | -9.8% |
| Overall (Test vs Ref) | -9.7% | -11.0% | -11.1% | -8.0% | -9.8% | -9.8% |
| Overall (Test vs single layer) | 12.9% | 15.0% | 14.7% | 11.3% | 10.5% | 10.1% |
| EL only (Test vs Ref) | -17.4% | -18.7% | -18.8% | -22.1% | -23.9% | -24.0% |
| Enc Time[%] | 99.6% |  |  | 104.1% |  |  |
| Dec Time[%] | 94.9% |  |  | 100.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -10.6% | -6.1% | -7.5% |  |  |  |
| Class B | -5.5% | -3.8% | -2.9% | -6.2% | -5.1% | -4.7% |
| Overall (Test vs Ref) | -7.0% | -4.4% | -4.2% | -6.2% | -5.1% | -4.7% |
| Overall (Test vs single layer) | 20.2% | 33.3% | 34.2% | 17.9% | 28.5% | 31.2% |
| EL only (Test vs Ref) | -12.9% | -10.4% | -10.1% | -16.8% | -15.6% | -15.2% |
| Enc Time[%] | 98.3% |  |  | 98.3% |  |  |
| Dec Time[%] | 97.7% |  |  | 96.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -9.0% | -3.4% | -4.8% |  |  |  |
| Class B | -4.7% | -3.3% | -2.7% | -6.4% | -4.9% | -4.7% |
| Overall (Test vs Ref) | -5.9% | -3.3% | -3.3% | -6.4% | -4.9% | -4.7% |
| Overall (Test vs single layer) | 26.9% | 38.0% | 39.1% | 23.4% | 33.2% | 35.7% |
| EL only (Test vs Ref) | -10.3% | -7.7% | -7.7% | -16.0% | -14.4% | -14.3% |
| Enc Time[%] | 99.5% |  |  | 99.9% |  |  |
| Dec Time[%] | 98.9% |  |  | 103.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -7.8% | -2.8% | -4.2% |  |  |  |
| Class B | -3.9% | -2.8% | -2.4% | -5.8% | -4.5% | -4.2% |
| Overall (Test vs Ref) | -5.0% | -2.8% | -2.9% | -5.8% | -4.5% | -4.2% |
| Overall (Test vs single layer) | 28.8% | 38.9% | 40.1% | 25.3% | 33.4% | 36.4% |
| EL only (Test vs Ref) | -9.0% | -6.9% | -7.0% | -14.6% | -13.4% | -13.1% |
| Enc Time[%] | 95.3% |  |  | 99.7% |  |  |
| Dec Time[%] | 96.9% |  |  | 100.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 8 BD-rate in IntraBL framework

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -14.3% | -16.4% | -17.1% |  |  |  |
| Class B | -7.8% | -9.1% | -8.9% | -8.0% | -9.7% | -9.7% |
| Overall (Test vs Ref) | -9.7% | -11.2% | -11.3% | -8.0% | -9.7% | -9.7% |
| Overall (Test vs single layer) | 12.5% | 13.6% | 13.3% | 11.0% | 10.9% | 10.3% |
| EL only (Test vs Ref) | -17.4% | -18.8% | -18.9% | -21.9% | -23.6% | -23.7% |
| Enc Time[%] | 99.4% |  |  | 100.4% |  |  |
| Dec Time[%] | 95.5% |  |  | 98.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -10.6% | -6.0% | -7.4% |  |  |  |
| Class B | -5.5% | -3.9% | -3.0% | -6.2% | -5.0% | -4.4% |
| Overall (Test vs Ref) | -6.9% | -4.5% | -4.3% | -6.2% | -5.0% | -4.4% |
| Overall (Test vs single layer) | 19.4% | 32.1% | 33.0% | 17.0% | 28.8% | 31.4% |
| EL only (Test vs Ref) | -12.9% | -10.4% | -10.1% | -16.7% | -15.3% | -14.8% |
| Enc Time[%] | 97.9% |  |  | 81.8% |  |  |
| Dec Time[%] | 97.7% |  |  | 82.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.7% | -3.0% | -4.3% |  |  |  |
| Class B | -4.5% | -3.2% | -2.4% | -6.2% | -4.3% | -3.8% |
| Overall (Test vs Ref) | -5.7% | -3.2% | -2.9% | -6.2% | -4.3% | -3.8% |
| Overall (Test vs single layer) | 26.3% | 36.6% | 38.0% | 23.0% | 33.6% | 36.4% |
| EL only (Test vs Ref) | -9.9% | -7.5% | -7.2% | -15.2% | -13.3% | -12.9% |
| Enc Time[%] | 96.3% |  |  | 89.7% |  |  |
| Dec Time[%] | 94.5% |  |  | 89.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -7.9% | -2.3% | -3.6% |  |  |  |
| Class B | -4.1% | -3.2% | -2.6% | -6.0% | -4.8% | -4.5% |
| Overall (Test vs Ref) | -5.2% | -2.9% | -2.9% | -6.0% | -4.8% | -4.5% |
| Overall (Test vs single layer) | 28.4% | 37.4% | 38.5% | 25.1% | 33.2% | 35.7% |
| EL only (Test vs Ref) | -9.3% | -7.1% | -7.1% | -14.9% | -13.7% | -13.5% |
| Enc Time[%] | 104.1% |  |  | 102.3% |  |  |
| Dec Time[%] | 100.1% |  |  | 102.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

This contribution presents an upsampling scheme achieving aligned sampling grids of EL and upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift. The proposed scheme includes two parts: 1) signalling the sampling grid offsets between layers in the bitstream and 2) incorporating the signaled offsets into phase filter selection. Simulations were performed using BL and EL sequences with non-zero phase shift. It is reported that the proposed scheme outperforms SHM-1.0 significantly. In the RefIdx framework, the average {Y, U, V} gains are {-8.8%, -10.4%, -10.5%}, {-6.6%, -4.8%, -4.4%}, {-6.2%, -4.1%, -4.0%}, and {-5.4%, -3.7%, -3.6%} for AI, RA, LDP, and LDB, respectively. The proposed scheme ensures that SHVC can achieve consistent scalable coding performance over a wider range of sequences.  We suggest adopting the proposed scheme into the SHVC working draft.

# Reference

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 1 (SHM 1),” JCT-VC document, JCTVC-L1007, Geneva, Switzerland, Jan. 2013.

- J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m24499, 100th MPEG meeting, Geneva, Switzerland, Apr. 30-May 4, 2012.

- J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m23485, 99th MPEG meeting, San Jose, USA, Feb. 1-10, 2012.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).