| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0190 |
| --- | --- |

| Title: | Requirements for medical imaging applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Uwe-Erik Martin, Nikolaus Wirsz, Andreas Klingler Siemens Healthcare Imaging and Therapy Systems Hartmannstr. 16 91052 Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This documents presents three different use cases for medical imaging applications: interactive browsing in medical image content, export of medical image content from an archive, and transmission and storage of mixed content. Requirements for compression of medical image content in the context of HEVC range extensions are derived from this use cases and summarized, most notably: support for efficient coding of 12-bit, 14-bit, and 16-bit content, support for efficient reversible (i.e., lossless) and diagnostically acceptable irreversible coding, and support for efficient compression of mixed content.

# Introduction

Medical imaging applications impose specific requirements on the underlying image/video compression scheme. These requirements are derived in the following, based on common medical use cases.

# Example use cases

## Use case 1: interactive browsing in medical image content

A common use case in the medical domain is the interactive transmission of medical image content from a server (e.g., a PACS – Picture Archiving and Communication System) to a client.

The medical image content can be, e.g.:

- visible light content (e.g., endoscopic video): 8-bit YUV 4:2:0 video sequences,

- ultrasound (US) image data: 8-bit monochrome, continuous-tone 2D+t image sequences (but usually represented in 8-bit RGB format),

- computed tomography (CT) image data: 12-bit, 14-bit, or 16-bit monochrome, continuous-tone 3D image volumes (i.e., sequences of 2D slices; note that in this context the term ‘slice’ refers to the reconstruction of a 2D image by a medical scanner and should not be confused with the term ‘slice’ as defined by the HEVC standard),

- magnetic resonance (MR) image data: 12-bit, 14-bit, 16-bit monochrome, continuous-tone 3D image volumes.

Note that there is a systematic difference between the different medical imaging modalities. On the one hand, there is timed media data (e.g., an US stream or endoscopic video),which has similar timing properties compared to “regular” video content (e.g., in broadcast, videoconferencing, etc.). On the other hand, there is volume data (e.g., CT, MR), where there is no associated timing information. For the latter, depending on the requirements for random access, inter-frame (i.e., inter-slice) prediction can be used, since adjacent slices of a volume have high similarity.

Depending of the type of the client (e.g., the respective computational resources), a client receives different representations of the archived data, e.g.:

- rich client: transmission of 12-bit/14-bit/16-bit medical (CT or MR) content, filtering (e.g., sharpening of edges) and windowing (i.e., adaptive mapping from a 12-bit/14-bit/16-bit representation to 8-bit representation),

- thin client: transmission of 8-bit content (i.e., windowing done at the server).

For browsing through the medical images, lossy compression of the images can be accepted. However, diagnostically acceptable irreversible compression is preferred. For examining an image after browsing is finished, the image has to be transmitted with reversible (i.e., lossless) compression. The bit depth for lossless coding is 8, 12, 14, or 16 bit, depending on the type of medical content and the type of client. In order to support interactivity, a low-delay coding mode is required.

## Use case 2: export of medical image content from an archive

This use case defines the export of medical image content from an image archive (e.g., a PACS) to a storage medium (e.g., a DVD – Digital Versatile Disc).

Depending on the type of medical content (see use case 1), the compression of 8-bit, 12-bit, 14-bit, and 16-bit monochrome, continuous-tone content should be supported. The content needs to be stored in a reversible (i.e., lossless) fashion.

## Use case 3: transmission and storage of mixed content

The transmission and storage of mixed content (e.g., a user interface containing medical image content, rendered by a server) can be seen as an extensions of use cases 1 and 2. For several medical application, not only the raw medical image content is transmitted and stored but artificial content is generated or added. This artificial content results, e.g., from colorized ultrasound images, screen content overlays (patient data, scale information, annotations, etc.), or volume-rendered computed tomography image data. The resulting image sequences are usually represented in 8-bit RGB format.

For this use case, the compression of 8-bit RGB mixed content (i.e., simultaneous compression of continuous-tone medical content and screen content) should be supported. Reversible (i.e., lossless) compression is required on some cases (e.g., export to storage medium as of use case 2).

# Summary of requirements

The following requirements are extracted from the example use cases presented in the previous section:

- Support for efficient reversible (i.e., lossless) and diagnostically acceptable irreversible compression of 8-bit, 12-bit, 14-bit, and 16-bit monochrome, continuous-tone image content.

- Support for efficient compression of 2D+t image data as well as 3D image volume data. (Note that still image coding of slices can be one solution for coding 3D image volumes.)

- Support for efficient reversible (i.e., lossless) and diagnostically acceptable irreversible compression of 8-bit RGB image content.

- Support for efficient compression of mixed content (i.e., continuous-tone content and screen content).

# Recommendations

In order to support medical imaging applications, the requirements derived from different use cases of medical imaging applications should be supported by the specification of HEVC range extensions.

In order to properly study the impact of the presented requirements on the HEVC specification for range extensions (especially the potential need for additional coding tools), we propose to extend the standardization schedule for range extensions by (at least) 6 months.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO) and from EIT ICT Labs’ Action Line “Networking Solutions for Future Media” under activity n◦ 13090.

# Patent rights declaration(s)

Siemens may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).