| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0193 |
| --- | --- |

| Title: | Non-RCE2: Improved sample-based weighted intra prediction for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Research and Technology Center Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Eugen Wige, André Kaup Universität Erlangen-Nürnberg Multimedia Communications and Signal Processing Cauerstr. 7 91058 Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This proposal presents an intra prediction scheme designed for lossless coding, which extends proposal JCTVC-M0052. The proposed coding method comprises a pixel-wise prediction based on original samples. It is realized as two intra prediction modes, which replace PLANAR mode and DC mode for lossless coding. (The algorithm proposed in JCTVC-M0052 only replace PLANAR mode.) In order to perform the prediction, a four-sample template around the pixel that is to be predicted is compared to the respective template of a four-pixel neighbourhood. For each reference template, the sum of absolute differences (SAD) is determined. Whereas for the one mode (DC) the pixel for which the SAD comparison gives the smallest value is chosen, for the second mode (PLANAR) a table look-up of the SAD value gives the respective weighting factor for each neighbourhood pixel. In a final step for the weighted prediction (PLANAR), the predictor for the current pixel is calculated as the weighted average of the neighbourhood pixels. In comparison to the unmodified HEVC Test Model HM-10.0_RExt2.0 configured for lossless coding by disabling/bypassing transformation, quantization, and in-loop filters, the proposed method reportedly provides average bitrate savings for of 11.0% with SC and 7.6% without SC for AI-Main, 6.9% with SC and 3.3% without SC for RA-Main, and 6.1% with SC and 2.7% without SC for LB-Main configuration settings.

# Introduction

As reported in [4], medical applications require image/video content to be stored losslessly. Regulations for these kind of applications require lossless transmission and storage, since coding artefacts might degrade the quality of a medical diagnosis. Due to the large amounts of medical image data that has to be archived in repositories (e.g., PACS – Picture Archiving and Communication System), high coding efficiency is important. Therefore, an efficient lossless coding scheme for HEVC is proposed.

# Proposed method

## Overview

The algorithm presented in this document is based on an algorithm presented in [1] and [3], where a backward adaptive sample-based weighted intra prediction mode is proposed. This contribution extends the scheme by a second mode, which improves the compression efficiency when coding screen content material. The properties of the algorithm are given in the following:

- Two dedicated intra prediction modes replacing PLANAR and DC mode are used to signal the usage of the proposed method.

- A four-sample template around the pixel that is to be predicted is compared to the respective template of a four-pixel neighbourhood. For each reference template, the sum of absolute differences (SAD) is determined. This is the same procedure as described in [3].

- For the one mode (PLANAR) a table look-up of the SAD value gives the respective weighting factor for each neighbourhood pixel. This is the same procedure as described in [3].

- For the second mode (DC) the weighting factor for the pixel where the SAD calculation gives the smallest value is set to one whereas for all other pixels the weighting factor is set to zero. This alternative determination of the modes in the second mode is the only difference between this proposal and JCTVC-M0052 [3].

- The predictor for the current pixel is calculated as the weighted average of the neighbourhood pixels. For DC mode, the “weighting” is reduced to using a single neighbouring pixel as predictor.

- The prediction error is calculated and further coded.

- After reconstruction of the respective pixel value, subsequent pixels are processed.

# Simulation results

The proposed method was implemented into HEVC Test Model HM-10.0_RExt2.0. The common test conditions for RCE2 as defined in [2] were used..  The simulation platform is a homogenous cluster (consisting of Intel(R) Core(TM)2 Quad CPU Q9400 @ 2.66GHz CPUs).

## Simulation results for the RCE2 test sequences

The coding simulation results for the RCE2 test sequences as proposed in [2] are depicted in the following table where “Reference” stands for the anchor results using HM-10.0_RExt2.0 and “Tested” stands for the simulation results using the proposed two-mode SWP prediction algorithm for lossless coding.

Table 1. Simulation results for two-mode SWP

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -11.2% |
| Class B | 2.1 | 2.3 | -6.8% |
| SC(GBR) | 7.5 | 9.0 | -14.3% |
| RangeExt | 2.2 | 2.3 | -4.4% |
| Overall (w/o SC) | 3.2 | 3.5 | -7.6% |
| Overall (w/ SC) | 5.3 | 6.3 | -11.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.9 | -6.3% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 104.7 | -10.4% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 13.3 | 14.3 | -3.3% |
| Overall (w/ SC) | 51.0 | 59.5 | -6.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.0 | -5.0% |
| Class B | 2.6 | 2.6 | -1.4% |
| SC(GBR) | 301.9 | 354.5 | -9.6% |
| RangeExt | 2.5 | 2.6 | -1.0% |
| Overall (w/o SC) | 20.5 | 21.5 | -2.7% |
| Overall (w/ SC) | 161.2 | 188.0 | -6.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Additionally, the combinations of the improved SWP scheme with other algorithms have been tested.

Table 2. Simulation results for two-mode SWP combined with RDPCM (L0117)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.4 | -12.7% |
| Class B | 2.1 | 2.3 | -7.2% |
| SC(GBR) | 7.5 | 9.3 | -16.4% |
| RangeExt | 2.2 | 2.3 | -4.6% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.3% |
| Overall (w/ SC) | 5.3 | 6.4 | -12.3% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.2 | -7.4% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 106.7 | -12.1% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 13.3 | 14.4 | -3.8% |
| Overall (w/ SC) | 51.0 | 60.6 | -8.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.3 | -6.0% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 362.1 | -11.2% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 20.5 | 21.7 | -3.1% |
| Overall (w/ SC) | 161.2 | 191.9 | -7.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3. Simulation results for two-mode SWP combined with SAP (L0176)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.5 | -13.6% |
| Class B | 2.1 | 2.3 | -7.4% |
| SC(GBR) | 7.5 | 9.1 | -15.5% |
| RangeExt | 2.2 | 2.3 | -4.9% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.8% |
| Overall (w/ SC) | 5.3 | 6.4 | -12.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.3 | -7.8% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 105.3 | -11.6% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 13.3 | 14.5 | -4.0% |
| Overall (w/ SC) | 51.0 | 59.9 | -7.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.3 | -6.3% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 358.5 | -10.7% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 20.5 | 21.7 | -3.3% |
| Overall (w/ SC) | 161.2 | 190.1 | -7.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4. Simulation results for two-mode SWP combined with SAP-HV

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.4 | -13.1% |
| Class B | 2.1 | 2.3 | -7.1% |
| SC(GBR) | 7.5 | 9.4 | -17.1% |
| RangeExt | 2.2 | 2.3 | -4.6% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.5% |
| Overall (w/ SC) | 5.3 | 6.5 | -12.8% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.4 | -7.6% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 107.5 | -12.7% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 13.3 | 14.5 | -3.9% |
| Overall (w/ SC) | 51.0 | 61.0 | -8.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.4 | -6.1% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 366.6 | -11.9% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 20.5 | 21.7 | -3.2% |
| Overall (w/ SC) | 161.2 | 194.1 | -7.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

# Text changes for Proposed Draft Amendment

This chapter is based on document JCTVC-L1005_v1-JCTVC-L1003_v23.

It is suggested to replace subsection 8.4.4.2.1 and additionally add section 8.4.4.2.7.

Replace 8.4.4.2.1 (Changes are marked in yellow)

8.4.4.2.1		General intra sample prediction

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The intra sample prediction process according to predModeIntra and trans_quant_bypass_flag applies as follows:

- If predModeIntra is equal to INTRA_PLANAR or INTRA_DC and trans_quant_bypass_flag is equal to 1, the process specified in subclause 8.4.4.2.7 is applied.

- Otherwise the following process is applied:

The nT * 4 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1, are derived as follows:

–	The neighbouring location ( xN, yN ) is specified by

( xN, yN ) = ( xB + x, yB + y )		(8-1)

- The current luma location ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) are derived as follows:

( xBY, yBY ) = ( cIdx  = =  0 ) ? ( xB, yB ) : ( xB * SubWidthC, yB * SubHeightC )	(8-2)

( xNY, yNY ) = ( cIdx  = =  0 ) ? ( xN, yN ) : ( xN * SubWidthC, yN * SubHeightC )	(8-3)

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

  - The variable availableN is equal to FALSE.

  - CuPredMode[ xNY ][ yNY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

- Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xN, yN ) is assigned to p[ x ][ y ].

When at least one sample p[ x ][ y ] with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process for intra sample prediction in subclause Error: Reference source not found is invoked with the samples p[ x ][ y ] with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1, nT, and cIdx as inputs, and the modified samples p[ x ][ y ] with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1 as output.

Depending on the value of predModeIntra, the following ordered steps apply:

- When cIdx is equal to 0 or ChromaArrayType is equal to 3, the filtering process of neighbouring samples specified in Error: Reference source not found is invoked with the sample array p and the transform block size nT as inputs, and the output is reassigned to the sample array p.

- The intra sample prediction process according to predModeIntra applies as follows:

  - If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p, the transform block size nT, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, (predModeIntra is in the range of INTRA_ANGULAR2..INTRA_ANGULAR34), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nT, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

Add section 8.4.4.2.7

8.4.4.2.7		Specification of intra prediction mode INTRA_SWP

The section describes the process which is invoked for prediction if trans_quant_bypass_flag is equal to 1 and predModeIntra is equal to INTRA_PLANAR or INTRA_DC.

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

- the residual sample array values r[ x ][ y ], with x, y = 0..nT − 1.

Output of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The SWP_Table is given depending on chroma_format_idc and cIdx as follows:

- If chroma_format_idc is equal to 3 or cIdx is equal to 0:

SWP_Table = {2097152, 1812405, 1566320, 1353648, 1169852, 1011012, 873739, 755104, 652577,

563972, 487397, 421219, 364027, 314600, 271884, 234968, 203065, 175493, 151665, 131072,

113275, 97895, 84603, 73116, 63188, 54609, 47194, 40786, 35248, 30462, 26326, 22752, 19662,

16993, 14686, 12692, 10968, 9479, 8192, 7080, 6118, 5288, 4570, 3949, 3413, 2950, 2549, 2203,

1904, 1645, 1422, 1229, 1062, 918, 793, 686, 592, 512, 442, 382, 330, 286, 247, 213, 184, 159,

138, 119, 103, 89, 77, 66, 57, 50, 43, 37, 32, 28, 24, 21, 18, 15, 13, 12, 10, 9, 7, 6, 6, 5, 4, 4, 3, 3,

2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1}

SWP_Max_Argument = 104

- Otherwise the following applies:

- If chroma_format_idc is equal to 2:

SWP_Table = {2097152, 1720368, 1411279, 1157722, 949720, 779089, 639114, 524288, 430092,

352820, 289430, 237430, 194772, 159779, 131072, 107523, 88205, 72358, 59358, 48693,

39945, 32768, 26881, 22051, 18089, 14839, 12173, 9986, 8192, 6720, 5513, 4522, 3710, 3043,

2497, 2048, 1680, 1378, 1131, 927, 761, 624, 512, 420, 345, 283, 232, 190, 156, 128, 105, 86, 71,

58, 48, 39, 32, 26, 22, 18, 14, 12, 10, 8, 7, 5, 4, 4, 3, 2, 2, 2, 1, 1, 1, 1, 1, 1}

SWP_Max_Argument = 77

- Otherwise, chroma_format_idc is equal to 1:

SWP_Table  = {2097152, 1541128, 1132525, 832255, 611597, 449443, 330281, 242713, 178361,

131072, 96321, 70783, 52016, 38225, 28090, 20643, 15170, 11148, 8192, 6020,	4424, 3251,

2389, 1756, 1290, 948, 697, 512, 376, 276, 203, 149, 110, 81, 59, 44, 32, 24, 17, 13, 9, 7, 5, 4, 3,

2, 1, 1, 1, 1}

SWP_Max_Argument = 49

The nT * 4 + 4 neighbouring samples p[ x ][ y ] that are constructed samples, with x = −2..−1, y = −2..nT−1 and x = −2..nT−1, y = −2..−1, are derived as follows:

–	The neighbouring location ( xN, yN ) is specified by

( xN, yN ) = ( xB + x, yB + y )

- The current luma location ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) are derived as follows:

( xBY, yBY ) = ( cIdx  = =  0 ) ? ( xB, yB ) : ( xB * SubWidthC, yB * SubHeightC )

( xNY, yNY ) = ( cIdx  = =  0 ) ? ( xN, yN ) : ( xN * SubWidthC, yN * SubHeightC )

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If the variable availableN is set to FALSE, p[ x ][ y ] is set to ( 1 << ( bitDepth – 1 ) ).

- Otherwise, the sample at the location ( xN, yN ) is assigned to p[ x ][ y ].

- Each sample predSample[ x ][ y ] is derived as follows:

- If x is smaller than ( nT – 1 ), predSample[ x ][ y ] is calculated as follows:

predSample[ x ][ y ] = (p[ x–1 ][ y ] * w-1,0 + p[ x–1 ][ y–1 ] * w-1,-1 + p[ x ][ y–1 ] * w0,-1

+ p[ x+1 ][ y–1 ] * w1,-1 + (w-1,0 + w-1,-1 + w0,-1 + w1,-1)>>1 )  /  ( w-1,0 + w-1,-1 + w0,-1 + w1,-1 )

- Otherwise, predSample[ x ][ y ] is calculated as follows:

predSample[ x ][ y ] = (p[ x–1 ][ y ] * w-1,0 + p[ x–1 ][ y–1 ] * w-1,-1 + p[ x ][ y–1 ] * w0,-1

+ (w-1,0 + w-1,-1 + w0,-1 )>>1 )  /  ( w-1,0 + w-1,-1 + w0,-1 )

- If predModeIntra is equal to INTRA_PLANAR the weighting factors wm,n are looked up in a table SWP_Table depending on x as follows:

  - If ( SADm,n >> ( bitDepth – 8 ) ) is greater than SWP_Max_Argument, wm,n is set to 0.

  - Otherwise wm,n is set to ( SWP_Table[ SADm,n >> ( bitDepth – 8 ) ] >> ( bitDepth – 8 ) ).

  - If x is smaller than ( nT – 2 ), SADm,n is calculated as follows:

SADm,n = | p[ x–1 ][ y ] – p[ x–1+m ][ y+n ] | + | p[ x–1 ][ y–1 ] – p[ x–1+m ][ y–1+n ] | +

| p[ x ][ y–1 ] – p[ x+m ][ y–1+n ] | + | p[ x+1 ][ y–1 ] – p[ x+1+m ][ y–1+n ] |

  - Otherwise:

SADm,n = | p[ x–1 ][ y ] – p[ x–1+m ][ y+n ] | + | p[ x–1 ][ y–1 ] – p[ x–1+m ][ y–1+n ] | +

| p[ x ][ y–1 ] – p[ x+m ][ y–1+n ] |

- If all of the resulting weights from the table look-up are zero or predModeIntra is equal to INTRA_DC (in this case the same SAD calculation process is applied as for INTRA_PLANAR), the weight wm,n for the sample with the smallest SAD value is chosen to be one whereas all other weights are chosen to be zero, i.e., the sample with the smallest SAD value is used as the predictor.

- The neighbouring sample p[ x ][ y ] is updated as follows:

p[ x ][ y ] = r[ x ][ y ] + predSample[ x ][ y ]

# Conclusion

Lossless coding is need in several professional applications, e.g., in the medical domain. This contribution presented an intra prediction scheme designed for lossless coding, which shows significant gain over the current HEVC design. Therefore, we propose to include the proposed technique in the Proposed Draft Amendment (PDAM) for Range Extensions of HEVC.

# Acknowledgment

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# References

- P. Amon, A. Hutter, E. Wige, A. Kaup,“ Intra prediction for lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC- L0161/M27497, Geneva, Switzerland, January 2013.

- W. Gao, M. Zhou, P. Amon, S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC- L1122/M28403, Geneva, Switzerland, January 2013.

- P. Amon, A. Hutter, E. Wige, A. Kaup,“RCE2: Sample-based weighted intra prediction for lossless coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0052/M28487, Incheon, Korea, April 2013.

- P. Amon, A. Hutter, U.-E. Martin, N. Wirsz, “Requirements for medical imaging applications”, Joint Collaborative Team on Video Coding (JCT-VC) of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0190/M28643, Incheon, Korea, April 2013.

# Patent rights declaration(s)

Siemens AG and Universität Erlangen-Nürnberg may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).