| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, Korea, 18–26 Apr. 2013 | Document: JCTVC-M0194 |
| --- | --- |

| Title: | SCE 1.3.1: Inter-layer Intra Mode Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mei Guo, Shan Liu, Shawmin Lei 2860 Junction Ave, San Jose, CA 95134, USA | Email: | mei.guo@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_________________________

# Abstract

This contribution presents the experimental results of the inter-layer intra mode prediction for Scalable Extension Core Experiments 1.3.1. The intra mode at enhancement layer is mainly predicted from the intra mode of the co-located unit at base layer in order to exploit the correlation between intra modes from different layers. It is reported that 0.4% and 0.1% coding gain can be achieved in AI2x and AI1.5x cases respectively.

# Introduction

- The intra mode coding at enhancement layer (EL) in SHM1.0 [1] is the same as that in HM-10[2], three most probable modes are derived for each luma PU from the intra modes of the neighboring PUs (i.e., the PU on the left of the current PU and the one on top of the current PU). If the current intra mode is equal to any of the most probable modes, a flag prev_intra_pred_flag equal to 1 is transmitted followed by the index of that most probable mode in the most probable mode list; otherwise, the remaining mode is transmitted following the flag prev_intra_pred_flag equal to 0. Thus, the selection of most probable modes influences the efficiency of intra mode coding.

We proposed to predict the intra mode at EL with the intra mode of the collocated block at base layer (BL) in order to exploit the correlation between intra modes from different layers in JCTVC-L0260 [3]. The proposed method is currently evaluated in Scalable Extension Core Experiment (SCE) 1.3[4]. The proposed method will be introduced in Section 2 and Section 3 will show the simulation results.

# Algorithm Description

- In order to exploit the correlation between the intra modes from different layers, we propose to predict the intra mode at EL mainly from the intra mode of the collocated unit at BL. Same as that in SHM1.0[1] and HM-10[2], three most probable modes (denoted as MPM[ x ] with x=0…2) are used at EL and they are derived for each luma PU from the intra mode of the corresponding unit at BL (denoted as modeBL), the intra mode of neighboring PU on the left of the current PU (denoted as modeLeft) and the one of the neighboring PU on top of the current PU  (denoted as modeTop). The intra mode at BL, which corresponds to the 4x4 unit A (as shown in Figure 1) within the current unit at enhancement layer, is derived as modeBL.

Figure 1.  Illustration of a selected 4x4 unit inside the current unit larger than 4x4

For each of modeBL, modeLeft and modeTop, if it is available and is not DC/Planar, it is put into MPM according to the priority (i.e., modeBL>modeLeft>modeTop) with the repetition removed. When three modes are available in MPM, {modeBL, modeLeft, modeTop} are directly used to predict the current intra mode at EL; otherwise, MPM is further generated as follows. Denote the number of modes available in MPM as numAvai.

- numAvai = 2, MPM = {MPM[0], MPM[1], DC}

- numAvai = 1, MPM = {MPM[0], MPM[0]-1, MPM[0]+1}

- numAvai = 0, MPM = {DC, VER, HOR}

# Results

The proposed method is implemented on top of SHM1.0 [1], which is also used as the anchor of simulation. The experimental results are shown in Table 1, while the detailed results are given in the attached excel file CE1.3.1_WithParsing_MTK.xls.

Table 1 Coding performance of the proposed method

There exists parsing dependency in the bit-stream of EL, since the decoded intra mode is required in mode dependent coefficient scan (MDCS). Thus, we further test the proposed method when MDCS is disabled to remove the parsing dependency. The results are shown in Table 2, while the detailed results are given in the attached excel file CE1.3.1_NoParsing_MTK.xls.

Table 2 Coding performance of the proposed method when MDCS is disabled

In JCTVC-M0196 [5], we tested the proposed method in this contribution on top of the method of Intra coding in difference domain in JCTVC-M0324 [6], which is currently evaluated in SCE 1.2.6[4]. We use the results reported in [6] as anchor. The results of the proposed method are shown in Table 3 and Table 4 shows the results when MDCS is disabled. The detailed results are given in the attached excel files CE1.3.1_IntraDiff_WithParsing.xls and CE1.3.1_IntraDiff_NoParsing.xls respectively.

Table 3 Coding performance of the proposed method on top of SCE1.2.6

Table 4 Coding performance of the proposed method on top of SCE1.2.6 when MDCS is disabled

# Conclusions

The results of the method in SCE 1.3.1 are reported in the contribution. The proposed method achieves 0.4% and 0.1% coding gain in AI2x and AI1.5x cases respectively.

# References

[1] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 1 (SHM 1)”, JCTVC-L1007, Geneva, Switzerland, 14–23 Jan. 2013.

[2] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent)”, JCTVC-L1003, Geneva, Switzerland, 14–23 Jan. 2013.

[3] M. Guo, S. Liu, S. Lei (MediaTek), J. Park, J. Kim, B. Jeon, “Non-TE5.1: Inter-layer Intra mode prediction”, JCTVC-L0260, Geneva, Switzerland, 14–23 Jan. 2013.

[4] A. Tabatabai, K. Rapaka, A. Saxena, S. Liu, “Description of Core Experiment 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, Geneva, Switzerland, 14–23 Jan. 2013.

[5] M. Guo, S. Liu, S. Lei (MediaTek), “Non-SCE 1:Simplified Intra Coding at Enhancement Layer for SHVC”, JCTVC-M0196, Incheon, Korea, 18–26 Apr. 2013.

[6] K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm), J. Park, B. Jeon (LG), C. Auyeung, A. Tabatabai (Sony), “SCE1 Category 2.2 and 2.6 : Difference domain Intra Prediction”, JCTVC-M0324, Incheon, Korea, 18–26 Apr. 2013.

# Proposed specification text

### 8.4.2 Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

|  | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- candIntraPredModeBL is derived as follows:

- if log2CbSize is equal to 2,

The specification in clause G6.1 applies to derive the luma position of the reference layer picture (xPbRef, yPbRef) with (xPb, yPb).

- Otherwise,

The specification in clause G6.1 applies to derive the luma position of the reference layer picture (xPbRef, yPbRef) with ((xPb+(1<<(log2CbSize-1))), ( yPb+(1<<(log2CbSize-1)))).

- If CuPredMode[xPbRef][yPbRef] of reference layer picture is Mode_Intra, candIntraPredModeBL is set equal to IntraPredModeY[xPbRef][yPbRef] of reference layer picture

- Otherwise,

candIntraPredModeBL is set to -1.

- The candModeList[ x ] with x = 0..2 is derived as follows, uiPredNum is set equal to 0:

- If candIntraPredModeBL is larger than 1 (i.e., CuPredMode[xPbRef][yPbRef] of reference layer picture is Mode_Intra, and candIntraPredModeBL is neither DC nor Planar).

candModeList[0] = candIntraPredModeBL;

uiPredNum++;

- If candIntraModeA is larger than 1 and candIntraModeA is not equal to candIntraModeBL

candModeList[uiPredNum] = candIntraModeA;

uiPredNum++;

- If candIntraModeB is larger than 1, and candIntraModeB is not equal to candIntraModeBL, and candIntraModeB is not equal to candIntraModeA,

candModeList[uiPredNum] = candIntraModeB;

uiPredNum++;

- If uiPredNum is equal to 0

candModeList[0] = INTRA_DC

candModeList[1] = INTRA_ANGULAR26

candModeList[2] = INTRA_ANGULAR10

- Otherwise if uiPredNum is equal to 1

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )

- Otherwise if uiPredNum is equal to 2

candModeList[2] = INTRA_DC

- IntraPredModeY[ xPb ][ yPb ] is derived by applying the following procedure:

- If prev_intra_luma_pred_flag[ xPb ][ yPb ] is equal to 1, the IntraPredModeY[ xPb ][ yPb ] is set equal to  candModeList[ mpm_idx ].

- Otherwise, IntraPredModeY[ xPb ][ yPb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..2 is modified as the following ordered steps:

  - When candModeList[ 0 ] is greater than candModeList[ 1 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 1 ] ) = Swap( candModeList[ 0 ], candModeList[ 1 ] )	(8-1)

  - When candModeList[ 0 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 2 ] ) = Swap( candModeList[ 0 ], candModeList[ 2 ] )	(8-2)

  - When candModeList[ 1 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 1 ], candModeList[ 2 ] ) = Swap( candModeList[ 1 ], candModeList[ 2 ] )	(8-3)

- IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

  - IntraPredModeY[ xPb ][ yPb ] is set equal to rem_intra_luma_pred_mode[ xPb ][ yPb ].

# For i equal to 0 to 2, inclusive, when IntraPredModeY[ xPb ][ yPb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xPb ][ yPb ] is incremented by one.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).