| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, Korea, 18–26 Apr. 2013 | Document: JCTVC-M0195 |
| --- | --- |

| Title: | SCE4.2.3: Inter-layer Adaptive Filter on Reconstructed Base Layer Pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mei Guo, Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA | Email: | mei.guo@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of the inter-layer adaptive filter on reconstructed base-layer pixels in Scalable Extension Core Experiment 4.2.3. The proposed adaptive filtering method is applied to the reconstructions of base layer, which is followed by the fixed upsampling in SHM1.0. The inter-layer adaptive filter is proposed to further reduce the redundant texture information of enhancement layer in inter-layer texture prediction by improving the predictors of enhancement layer. Compared with SHM1.0 anchor with Intra_BL framework, average 0.6%, 0.6%, 0.3%, 0.5%, 1.9%, 0.6%, 0.7%, and 2.9% BD-rate gains are reported.

# Introduction

In scalable video coding, the inter-layer intra prediction is an important tool to reduce the redundant texture information of enhancement layer (EL). The predictor in EL is generated by block-wise upsampling the co-located base layer (BL) reconstruction signal. In SHM1.0 [1], fixed 8-tap and 4-tap FIR upsampling filters are applied to luma and chroma components, respectively.

In this contribution, we present the results of the method of the inter-layer adaptive filter on reconstructed BL pixels, which has been proposed in JCTVC-L0256 [2] and is currently evaluated in Scalable Extension Core Experiment(SCE) 4 [3]. As shown in Figure 1, the inter-layer adaptive filter is first applied to the reconstructions of BL. Then the EL prediction is generated from the filtered BL pixels with the fixed upsampling in SHM1.0 [1].  The proposed method will be introduced in Section 2 and Section 3 will show the simulation results.

Figure 1 Illustration of Inter-layer adaptive filter.

# Algorithm Description

In this contribution, we propose to apply inter-layer adaptive filter to BL reconstructions followed by fixed upsampling in SHM1.0 [1] to generate the predictor of EL. A 5x5 diamond shape filter is proposed to filter the reconstructed BL pixels, which is shown in Fig. 2. This filter is 2-D non-separable, represented by seven distinct filter coefficients {hi } i=1, …, 7.

Figure 2 5x5 diamond shape filter

All adaptive filter coefficients are achieved by minimizing the difference between the filtered BL reconstructions and the collocated original pixels at EL. The last coefficient h7 is predicted with all the other ones. Afterwards, they are coded with exponential golomb codes.

## The decision on whether adaptive filte is used to filter BL reconstructions is first made for each frame, which is signaled with one flag for each frame. If it is true, the coding unit (CU) based decision is further made during rate-distortion based encoding process. A flag to indicate whether the BL reconstructions or filtered BL reconstructions would be used in fixed upsampling is required to be transmitted for each CU. Both decisions are made based on the distortion (calculated with SSD measure) and estimated bit rates for filter coefficients.

# Results

The proposed method is implemented on top of SHM1.0 [1] with Intra_BL framework, which is also used as the anchor of simulation. When the proposed method is only applied to luma component, the simulation results are given in Table 1. We also provided the results when the proposed method is applied to both luma and chroma component, which are shown in Table 2.

As required in SCE4 [3], we provide the results of memory bandwidth requirement and computational complexity with the methodologies defined in JCTVC-L0440 [4]. The results of the proposed method (only applied to luma component) in average case are shown in Table 3 while Table 4 gives the results in worst case.

Table 1 Coding performance of inter-layer adaptive filter on reconstructed BL pixels (applied to Luma component)

Table 2 Coding performance of inter-layer adaptive filter on reconstructed BL pixels (applied to Luma and chroma components)

Table 3 Memory bandwidth requirement and computational complexity in average case

Table 4 Memory bandwidth requirement and computational complexity in worst case

# Conclusions

This contribution presented the experimental results of the inter-layer adaptive filter on reconstructed BL pixels in SCE 4.2.3. The proposed adaptive filtering method is applied to the reconstructions of BL, which is followed by the fixed upsampling in SHM1.0. Compared with SHM1.0 anchor with Intra_BL framework, average 0.6%, 0.6%, 0.3%, 0.5%, 1.9%, 0.6%, 0.7%, and 2.9% BD-rate gains are reported. The results of memory bandwidth requirement and computational complexity are also provided.

# References

[1] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 1 (SHM 1)”, JCTVC-L1007, Geneva, Switzerland, 14–23 Jan. 2013.

[2] M. Guo, P. Lai, S. Liu, C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek), “Non-TE4: Inter-layer Adaptive Filters”, JCTVC-L0256, Geneva, Switzerland, 14–23 Jan. 2013.

[3] J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of HEVC Scalable Extensions Core Experiment 4 (SCE4): Inter-layer filtering”, JCTVC-L1104, Geneva, Switzerland, 14–23 Jan. 2013.

[4] E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L440, Geneva, Switzerland, 14–23 Jan. 2013.

# Proposed specification texts

- 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| ...... |  |
| if( !dependent_slice_segment_flag ) { |  |
| ...... |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if(num_layer_id > 0){ |  |
| iaf_flag | u(1) |
| if( iaf_flag ) { |  |
| for( i = 0; i < 7; i++) |  |
| iaf_coeff[i] | kes(v) |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ...... |  |
| } |  |
| slice_qp_delta | se(v) |
| ...... |  |
| byte_alignment( ) |  |
| } |  |

iaf_flag equal to 1 specifies that the inter-layer adaptive filtering process is enabled for the current slice and the coefficients are present in the current slice header; equal 0 specifies that the inter-layer adaptive filtering process is disabled for the current slice and the coefficients are not present in the current slice header.

iaf_coeff[ i ] specifies the i-th filter coefficient used in the inter-layer adaptive filtering process. k-th order exponential-golomb (Exp-Golomb) code with a sign bit at the end is used where the order k for the i-th filter coefficient is equal to the value of iafkMap[ i ] which is specified as follows:

iafkMap[ i ] = {2, 3, 4, 3, 3, 5, 0 } with i = 0..6	(7-1)

The restrictions apply for all values of i is different. When i=0..5, the values of iaf_coeff[i] shall be in the range of −256 to 255, inclusive. With the restrictions for i=0..5, iaf_coeff[6] is restricted by calculating centralCoeff and restricting the values thereof to be in the range of 0 to 511, inclusive.

centralCoeff = (iaf_coeff[ 6 ] + (1<<8)−m(2*iaf_coeff [ m ]) ), with m=0..5

iaf_coeff[ 6 ]  = centralCoeff

- 7.3.8.5Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if (iaf_flag) |  |
| iaf_cu_flag[x0][y0] | u(1) |
| } |  |
| else { |  |
| ...... |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if(( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) ¦¦ PredMode[ x0 ][ y0 ] = = MODE_TEXTURE_RL) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

iaf_cu_flag[x0][y0] equal to 1 specifies the inter-layer adaptive filtering process is applied on the current coding unit; equal to 0 specifies the inter-layer adaptive filtering process is not applied on the current coding unit.

- G8.1.4.1.5 Inter layer adaptive filtering process for luma sample of reference layer picture

Inputs to this process are

–	a (PicWRL)x(PicHRL) array rlPicSampleL,

–	a luma sample location ( xPRef, yPRef ) relative to the top-left luma sample of the reference layer picture.

Output of this process is a filtered luma sample value iafLumaSampleL.

The value of the filtered luma sample iafLumaSampleL is derived as follows:

iafLumaSampleL = Clip( (iaf_coeff[0] * (rlLumaSampleL[xPRef, Clip3(0, (PicHRL -1), (yPRef-2))]+ rlLumaSampleL[xPRef, Clip3(0, (PicHRL -1), (yPRef+2))] ) +

iaf_coeff[1] * (rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef +1)), Clip3(0, (PicHRL -1), (yPRef+1))] + rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef -1)), Clip3(0, (PicHRL -1), (yPRef-1))] ) +

iaf_coeff[2] * (rlLumaSampleL[xPRef, Clip3(0, (PicHRL -1), (yPRef+1))] + rlLumaSampleL[xPRef, Clip3(0, (PicHRL -1), (yPRef-1))] ) +

iaf_coeff[3] * (rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef -1)), Clip3(0, (PicHRL -1), (yPRef+1))] + rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef +1)), Clip3(0, (PicHRL -1), (yPRef-1))] ) +

iaf_coeff[4] * (rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef -2)), yPRef] + rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef + 2)), yPRef] ) +

iaf_coeff[5] * (rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef -1)), yPRef] + rlLumaSampleL[Clip3(0, (PicWRL-1), (xPRef + 1)), yPRef] ) +

iaf_coeff[6] * rlLumaSampleL[xPRef, yPRef]  + (1<<7)) >> 8)

- H8.8.3 Resampling process of luma samples

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

–	a (PicWRL)x(PicHRL) array rlPicSampleL,

- an iaf_cu_flag specifying whether the inter-layer adaptive filtering process is applied on the current block

Output of this process is a (nPbW)x(nPbH) array predSamplesL of luma prediction samples.

The (nPbW)x(nPbH) array predSamplesL of luma prediction samples are derived as follows:

- If iaf_cu_flag is equal to 1

- If ScaleW and ScaleH are both equal to 1

The luma sample of temporal reference layer picture t_rlPicSampleL[xR, yR] with (xR = xCb..(xCb+nPbW-1), yR=yCb..(yCb+nPbH-1)) are derived by invoking the Inter layer adaptive filtering process for luma sample of reference layer picture in subcause G8.1.4.1.5 with rlPicSampleL and luma sample location of reference layer picture (xR, yR) given as inputs and t_rlPicSampleL[xR, yR] as output

Then, the luma prediction sample predSamplesL[ x ][ y ] with ( x = 0 .. nPbW – 1, y = 0 .. nPbH – 1 ) are derived by invoking the luma sample interpolation process specified in subclause  with t_rlPicSampleL and luma sample location ( xCb + x,  yCb + y ) given as inputs and predSamplesL[ x ][ y ] as output.

- Otherwise

The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( xCb, yCb ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

xRef = xRef16 >> 4;

yRef = yRef16 >> 4;

The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( (xCb+nPbW-1), (yCb+nPbW-1) ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

xRefR = (xRef16 >> 4) + 1;

yRefB = (yRef16 >> 4) + 1;

The luma sample of temporal reference layer picture t_rlPicSampleL[xR, yR] with (xR = Clip3(0, (PicWRL-1), (xRef-3))..Clip3(0, (PicWRL-1),(xRefR+3)), yR=Clip3(0, (PicHRL-1), (yRef-3))..Clip3(0,(PicHRL-1),(yRef+3))) are derived by invoking the Inter layer adaptive filtering process for luma sample of reference layer picture in subcause G8.1.4.1.5 with rlPicSampleL and luma sample location of reference layer picture (xR, yR) given as inputs and t_rlPicSampleL[xR, yR] as output

Then, the luma prediction sample predSamplesL[ x ][ y ] with ( x = 0 .. nPbW – 1, y = 0 .. nPbH – 1 ) are derived by invoking the luma sample interpolation process specified in subclause  with t_rlPicSampleL and luma sample location ( xCb + x,  yCb + y ) given as inputs and predSamplesL[ x ][ y ] as output.

- Otherwise

The luma prediction sample predSamplesL[ x ][ y ] with ( x = 0 .. nPbW – 1, y = 0 .. nPbH – 1 ) are derived by invoking the luma sample interpolation process specified in subclause  with rlPicSampleL and luma sample location ( xCb + x,  yCb + y ) given as inputs and predSamplesL[ x ][ y ] as output.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).