| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, Korea, 18–26 Apr. 2013 | Document: JCTVC-M0196 |
| --- | --- |

| Title: | Non-SCE 1: Simplified Intra Coding at Enhancement Layer for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mei Guo, Shan Liu, Shawmin Lei 2860 Junction Ave, San Jose, CA 95134, USA | Email: | mei.guo@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_________________________

# Abstract

This contribution presents a method that only utilizes intra differential coding at enhancement layer in order to both improve the intra coding at enhancement layer with intra differential coding and reduce the increase of complexity introduced by intra differential coding. No additional flag is transmitted to indicate whether conventional intra coding that uses the spatial prediction or intra differential coding is used. It is reported that 0.4% and 0.5% coding gain can be achieved in AI2x and AI1.5x cases respectively. The encoding time is 100% and 98% in AI2x and AI1.5x cases while the decoding time is 103.7% and 102.5% in those two cases respectively.

# Introduction

- The inter-layer differential coding method is currently evaluated in Scalable Extension Core Experiment (SCE) 1.2.2 and 1.2.6[1]. In the inter-layer differential coding method, the difference between the pixels of current neighbors and that of scaled collocated base layer (BL) neighbors are used to generate a difference prediction based on the intra prediction mode. The generated difference prediction signal is added to the scaled collocated BL block signal to form the final prediction. In the methods evaluated in SCE 1[1], the encoder needs to select the coding mode between conventional intra coding and intra differential coding based on the rate-distortion performance at enhancement layer (EL). Then, one flag is transmitted for each coding unit (CU) to signal whether the conventional intra coding method or intra differential coding method is used at the decoder. As is reported in SCE 1, 0.8% and 0.6% BD-Rate reduction can be achieved. But the encoding time and decoding time is around 160% and 110%. The coding efficiency of EL is improved at the cost of increased complexity.

- In this contribution, we proposed a method to reduce the increase of complexity from intra differential coding and keep most of the coding gain from that. The proposed method will be introduced in Section 2 and Section 3 will show the simulation results.

# Algorithm Description

Due to the correlation between texture information from different layers, BL provides nice predictors of EL in many cases. At EL, intra differential coding is expected to be more efficient than conventional intra coding using spatial predictions. In this contribution we proposed to only use intra differential coding at EL. There will no need to transmit additional flag to signal whether the conventional intra coding or intra differential coding is used for each CU. Thus, the parsing module of EL is kept the same as that of single layer in HM10 [2].

# Results

The proposed method is implemented on top of SHM1.0 [3], which is also used as the anchor of simulation. The experimental results are shown in Table 1 while the detailed results are given in the attached excel file (i.e., Sim_Intra_EL.xls).

Table 1 Coding performance of the proposed method

We also combined the inter-layer intra mode prediction method proposed in [4] with the proposed method. The mode dependent coefficient scan (MDCS) is turned off to remove the parsing dependency in the bit-stream of EL, since the decoded intra mode is required in MDCS. The results are shown in Table 2 while the detailed results are given in the attached excel file (i.e., Sim_Intra_EL_And_Inter_Layer_Intra_Mode_MDCSOff.xls). It can be observed that the coding gain from those two methods are additive.

Table 2 Coding performance of the proposed method and the inter-layer intra mode prediction in [4].

In this contribution, we also tested the inter-layer intra mode prediction in [4] on top of the intra differential coding method proposed in JCTVC-M0324 [5] evaluated in SCE 1[1]. We use the results reported in [5] as anchor. The results of inter-layer intra mode prediction are shown in Table 3 and Table 4 shows the results when MDCS is disabled. The detailed results are given in the attached excel files CE1.3.1_IntraDiff_WithParsing.xls and CE1.3.1_IntraDiff_noParsing.xls respectively.

Table 3 Coding performance of the inter-layer intra mode prediction[4] on top of SCE1.2.6

Table 4 Coding performance of the inter-layer intra mode prediction[4] on top of SCE1.2.6 when MDCS is disabled

# Conclusions

We proposed a simplified intra coding method to reduce the increase of complexity from intra differential coding and keep most of coding gain from that. It is reported that 0.4% and 0.5% coding gain can be achieved in AI2x and AI1.5x cases respectively. The encoding time is 100% and 98% in AI2x and AI1.5x cases while the decoding time is 103.7% and 102.5% in those two cases respectively.

# References

[1] A. Tabatabai, K. Rapaka, A. Saxena, S. Liu, “Description of Core Experiment 1: Intra Prediction Improvements in SHVC”, JCTVC-L1101, Geneva, Switzerland, 14–23 Jan. 2013.

[2] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent)”, JCTVC-L1003, Geneva, Switzerland, 14–23 Jan. 2013.

[3] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 1 (SHM 1)”, JCTVC-L1007, Geneva, Switzerland, 14–23 Jan. 2013.

[4] M. Guo, S. Liu, S. Lei (MediaTek), “	SCE1.3.1:Inter-Layer Intra Mode Prediction”, JCTVC-M0194, Incheon, Korea, 18–26 Apr. 2013.

[5] K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm), J. Park, B. Jeon (LG), C. Auyeung, A. Tabatabai (Sony), “SCE1 Category 2.2 and 2.6 : Difference domain Intra Prediction”, JCTVC-M0324, Incheon, Korea, 18–26 Apr. 2013.

# Proposed specification texts

- 8.4.4.2.2 Reference sample substitution process for intra sample prediction

Inputs to this process are:

- reference samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 for intra sample prediction,

- the upsampled neighboring samples of reference layer picture bp[x][y], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- a transform block size nTbS,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the modified reference samples p[ x ][ y ] and bp[x][y] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 for intra sample prediction.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The values of the samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 are modified as follows:

- If all samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 are marked as "not available for intra prediction", the value 1  <<  ( bitDepth − 1 ) is substituted for the values of all samples p[ x ][ y ] and bp[x][y].

- Otherwise (at least one but not all samples p[ x ][ y ] are marked as "not available for intra prediction"), the following ordered steps are performed:

- When p[ −1 ][ nTbS * 2 − 1 ] is marked as "not available for intra prediction", search sequentially starting from x = −1, y = nTbS * 2 − 1 to x = −1, y = −1, then from x = 0, y = −1 to x = nTbS * 2 − 1, y = −1. Once a sample p[ x ][ y ] marked as "available for intra prediction" is found, the search is terminated and the value of p[ x ][ y ] is assigned to p[ −1 ][ nTbS * 2 − 1 ], and the value of bp[x][y] is assigned to bp[-1][nTbS * 2 – 1].

- Search sequentially starting from x = −1, y = nTbS * 2 − 2 to x = −1, y = −1, when p[ x ][ y ] is marked as "not available for intra prediction", the value of p[ x ][ y + 1 ] is substituted for the value of p[ x ][ y ] and the value of bp[ x ][ y + 1 ] is substituted for the value of bp[ x ][ y ].

- For x = 0..nTbS * 2 − 1, y = −1, when p[ x ][ y ] is marked as "not available for intra prediction", the value of p[ x − 1 ][ y ] is substituted for the value of p[ x ][ y ] and the value of bp[ x − 1 ][ y ] is substituted for the value of bp[ x ][ y ].

All samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 are marked as "available for intra prediction".

- 8.4.2.4.4 Specification of intra prediction mode INTRA_PLANAR

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- the upsampled neighboring samples of reference layer picture bp[x][y], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- the upsampled reference layer picture samples b[x][y], with x, y = 0..nTbS − 1

–	a variable nTbS specifying the transform block size.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived as follows:

predSamples[ x ][ y ] = b[x][y] + ( ( nTbS − 1 − x ) * (p[ −1 ][ y ]-bp[-1][y]) + ( x + 1 ) * (p[ nTbS ][ −1 ]-bp[nTbs][-1]) + 
		( nTbS − 1 − y ) * (p[ x ][ −1 ] – bp[x][-1])+ 
		( y + 1 ) * (p[ −1 ][ nTbS ]-bp[-1][nTbs]) + nTbS )  >>  ( Log2( nTbS ) + 1 )	(8-1)

        - Specification of intra prediction mode INTRA_DC

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- the upsampled neighboring samples of reference layer picture bp[x][y], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- the upsampled reference layer picture samples b[x][y], with x, y = 0..nTbS − 1

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived by the following ordered steps:

- A variable dcVal is derived as follows:

dcVal = 	(8-2)

where k = Log2( nTbS ).

- A variable dcValRef is derived as follows:

dcValRef = x'=0nTbS−1bp[x'][−1]+y'=0nTbS−1bp[−1][ y']+nTbS>>(k+1)

- Depending on the value of the colour component index cIdx, the following applies:

- If cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * (b[0][0] + dcVal – dcValRef) + p[ 0 ][ −1 ] + 2 )  >>  2	(8-3)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * (b[x][0] + dcVal – dcValRef) + 2 )  >>  2, with x = 1..nTbS − 1	(8-4)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * (b[0][y]+dcVal-dcValRef) + 2 )  >>  2, with y = 1..nTbS − 1	(8-5)

predSamples[ x ][ y ] = b[x][y]+dcVal-dcValRef, with x, y = 1..nTbS − 1	(8-6)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = b[x][y]+dcVal-dcValRef, with x, y = 0..nTbS − 1	(8-7)

        - Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- the upsampled neighboring samples of reference layer picture bp[x][y], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

- the upsampled reference layer picture samples b[x][y], with x, y = 0..nTbS − 1

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Figure 8-1 illustrates the total 33 intra angles and Table 8-1 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Figure 8-1 – Intra prediction angle definition (informative)

Table 8-1 – Specification of intraPredAngle

| predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-2 further specifies the mapping table between predModeIntra and the inverse angle parameter invAngle.

Table 8-2 – Specification of invAngle

| predModeIntra | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntra | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 + x ][ −1 ] – bp[-1 + x][-1], with x = 0..nTbS	(8-8)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ] - bp[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-9)

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ]-bp[-1 +x][-1], with x = nTbS + 1..2 * nTbS	(8-10)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8-11)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8-12)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = b[x][y]+
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-13)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = b[x][y]+ref[ x + iIdx + 1 ]		(8-14)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nTbS is less than 32, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = b[x][y] + (p[ x ][ −1 ] –bp[ x ][ -1 ])) + ( ( (p[ −1 ][ y ]-bp[-1][y]) – (p[ −1 ][ −1 ]-p[-1][-1]) )  >>  1 ) 	(8-15)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 ][ −1 + x ] – bp[-1][-1+x], with x = 0..nTbS	(8-16)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = (p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ]- bp[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ]),
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-17)

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ] –bp[-1][-1+x], with x = nTbS + 1..2 * nTbS	(8-18)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-19)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-20)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = b[x][y]
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-21)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = b[x][y]+ref[ y + iIdx + 1 ]		(8-22)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nTbS is less than 32, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = b[x][y] +( p[ −1 ][ y ]-bp[-1][y]) + ( ( (p[ x ][ −1 ]-bp[x][-1]) – (p[ −1 ][ −1 ] –bp[-1][-1]))  >>  1 )	(8-23)

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).