| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0200 |
| --- | --- |

| Title: | Layer decoding delay indication for SHVC and 3D/MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Karsten Suehring Yago Sanchez Thomas Schierl | Email: | robert.skupin@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution proposes to add syntax elements that allow signalling the minimum required decoding delay between the current and all reference layers. The signalling can be in units of slices, tiles, substreams or CTUs.

# Introduction

Parallel decoders can decode multiple layers in parallel. Without any restriction, a decoder is required to finalize decoding of all reference views in 3D- and MV-HEVC and with multi-loop decoding all reference layers in SHVC. This contribution proposes to signal the minimum required decoding delay between the current layer and all reference layers. A parallel decoder can start decoding of the enhancement layer, as soon as the specified reference layer delay is achieved.

# Proposal

This contribution proposes to signal additional syntax elements, which indicate a minimum delay for decoding the dependent layer/view based on the reference layer/view.

If high-level parallelization tools are used, these are used as units for the delay, i.e. the number of tiles, number of (WPP) substreams or number of slices. The actual number is indicated by the value of the syntax element min_spatial_segment_delay.

If no high-level parallelization tools are used, units of CTUs can be used. This can be indicated by setting ctb_based_delay_enabled_flag equal to 1. The effect of a CTU based signalling is similar to the signalling described in [7].

When tiles are used, the necessary delay can be reduced by constraining the upsampling filter or the disparity vector prediction as described in [5] and [6].

# Syntax and semantics

The following syntax elements are suggested to be added as an extension to VUI (which could be contained in the SPS extension):

| sps_extension ( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_vui_parameters() |  |
| } |  |

| sps_extension_vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| … |  |
| if( bitstream_restriction_flag ) { |  |
| ctb_based_delay_enabled_flag | u(1) |
| min_spatial_segment_delay | ue(v) |
| } |  |
| … |  |
| } |  |

ctb_based_delay_enabled_flag being equal to 1 indicates that the delay signalled with min_spatial_segment_delay is given in units of CTBs. ctb_based_delay_enabled_flag indicates that min_spatial_segment_delay is not given in units of CTBs.

min_spatial_segment_delay describes the decoding delay of the current layer introduced by coding dependencies with respect to the corresponding base layer in terms of spatial segments.

Depending on the value of min_spatial_segment_delay, the following applies:

If min_spatial_segment_delay is equal to 0, no limit on the minimum delay between decoding of layers is signalled

Otherwise (min_spatial_segment_delay is not equal to 0) and if ctb_based_delay_enabled_flag is equal to 1, it is a requirement of bit stream conformance that the following condition shall be true:

- With CtbSizeYA, PicWidthInCtbsYA and ctbAddrRsA being the CtbSizeY and PicWidthInCtbsY of a base layer A and the Ctb Address of a Ctb in base layer A in raster scan order, CtbSizeYB, PicWidthInCtbsYB and ctbAddrRsB being the CtbSizeY and PicWidthInCtbsY of a dependent layer/view B and the Ctb Address of a Ctb of a dependent layer B in raster scan order and CtbScalingFactorBA, CtbRowBA(ctbAddrRs) and CtbColBA(ctbAddrRs) being determined as follows

CtbScalingFactorBA = ( PicWidthInCtbsYA / PicWidthInCtbsYB )

CtbRowBA(ctbAddrRs) =

Ceil( ( Floor( ctbAddrRs / PicWidthInCtbsYB ) + 1 ) * CtbScalingFactorBA) - 1

CtbColBA(ctbAddrRs) =

Ceil( ( ( ctbAddrRs % PicWidthInCtbsYB ) + 1 ) * CtbScalingFactorBA ) - 1

When decoding the CTB with ctbAddrRsB of the current enhancement layer/view B, all necessary base layer resources are available when the base layer CTB with ctbAddrRsA being equal to PicWidthInCtbsYA * CtbRowBA( ctbAddrRsB) + CtbColBA(ctbAddrRsB) + min_spatial_segment_delay is decoded completely.

Otherwise (min_spatial_segment_delay is not equal to 0 and ctb_based_delay_enabled is equal to 0), it is a requirement of bit stream conformance that exactly one of the following conditions shall be true:

- In each picture parameter set that is activated within the coded video sequence, tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 0 (i.e. neither tiles nor WPP are used in the video sequence), and all base layer resources for the decoding process of any slice segment A of the current layer in bit stream order are available when the first slice segment C that follows (min_spatial_segment_delay -1) slices in bitstream order in the base layer after the last slice segment B in bitstream order that contains at least a part of the same image area with respect to slice A in the current layer is decoded completely.

- In each picture parameter set that is activated within the coded video sequence, tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0 (i.e. tiles are used in the video sequence), and all base layer resources for the decoding process of any tile A of the current layer in bitstream order are available when the first tile C that follows (min_spatial_segment_delay -1)  tiles in bitstream order after the last tile B in bitstream order that contains at least a part of the same image area with respect to tile A are completely decoded.

- In each picture parameter set that is activated within the coded video sequence, tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1 (i.e. WPP is used in the coded video sequence), and all base layer resources for the decoding process of the any CTB row A in the current layer are available when the first CTB row C that follows (min_spatial_segment_delay  -1) CTB rows after the last CTB row B of the base layer that covers at least part of the same image area with respect to CTB row A of the enhancement layer in bitstream order are completely decoded.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand (Eds.), “High Efficiency Video Coding (HEVC) text specification draft 10“, JCTVC-L1003, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J.Boyce (Eds.), “MV-HEVC Draft Text 3 (ISO/IEC 23008-2 PDAM2)“, JCT3V-C1004, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, S. Yea (Eds.), “3D-HEVC Test Model Description, draft specification“, JCT3V-C1005, Geneva, CH, Jan. 2013

- J. Chen, J. Boyce, Y. Ye, M. Hannuksela (Eds.), “SHVC Draft Text 1“, JCT3V-C1005, Geneva, CH, Jan. 2013

- K. Suehring , R. Skupin, Y. Sanchez, T. Schierl, “Independent tile upsampling for SHVC“, JCTVC-M0198, Incheon, KR, Apr. 2013

- G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl , “Restriction of inter-view prediction on tile boundaries“, JCT3V-D0051, Incheon, KR, Apr. 2013

- Y. Chen, V. Seregin, A.Ramasubramonian, L. Zhang, Y-K. Wang, “AHG7: Parallel decoding SEI message for MV-HEVC“, JCT3V-C0062, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).