| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0202 |
| --- | --- |

| Title: | Indication of tile boundary alignment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Sühring Robert Skupin Thomas Schierl | Email: | karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution proposes to add a flag to VUI of an enhancement layer that allows signalling that the tile boundaries of all base layers are aligned with tile boundaries of that enhancement layer. This flag can be used in a decoder as guide for a parallelization strategy that allows parallel decoding of base and enhancement layer tiles. This has previously been discussed in JCTVC-L0197. In combination with additional restrictions in inter-layer and inter-view prediction as presented in JCTVC-M0198 and JCT3V-Dxxxx a decoder can infer decoding independence between reference and enhancement layer tiles covering different regions based on this flag.

# Introduction

In SHVC [2] multi-loop scalable decoding all base layers have to be decoded to decode the enhancement layer. It is expected that decoders will process different layers in parallel.

If the complete decoded base layer picture is handed over to the enhancement layer decoder, this introduces a delay of at least one picture per layer. A tightly coupled decoder might pass decoded CTU rows to the enhancement layer decoder to reduce that delay.

Such a passing of CTU rows is only possible, if the decoding order in base and enhancement layer is the same. This might not be the case, when different tile structures are used in both layers.

Thus an alignment of tile boundaries in base and enhancement layer may be desirable for parallel low delay operation.

Tile boundaries can change per picture. Thus a decoder cannot detect at the beginning of decoding if the tile boundaries will be aligned over the whole sequence by analyzing the initial structure (except for tiles_fixed_structure_flag being equal to 1).

To solve that problem this contribution proposes to add a metadata flag to the enhancement layer VUI that indicates that the tile boundaries of the base layer are always aligned with the tile boundaries of the enhancement layer.

Especially for spatial scalability, it might be useful to use a partitioning of the enhancement layer into smaller tiles, so that each base an enhancement layer tile covers a similar number of samples. So the proposed flag signals only that all base layer tile boundaries exist also in the enhancement layer, but not the other way round.

In spatial scalability, if tiles are aligned, the enhancement layer tile can be decoded directly after the reference layer tile, when the upsampling filter is constrained at tile boundaries as described in [3].

In 3D and multi-view coding, if tiles are aligned, the dependent view tile can be decoded directly after the reference view tile, when the disparity vector prediction is constrained at tile boundaries as described in [4].

# Syntax

Changes relative to [1] are highlighted in yellow.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| … |  |
| if( bit stream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| tile_boundaries_aligned_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| restricted_ref_pic_lists_flag | u(1) |
| min_spatial_segmentation_idc | u(8) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

# Semantics

tile_boundaries_aligned_flag equal to 1 indicates that all tile boundaries of the corresponding base layer picture have corresponding tile boundaries in the given enhancement layer. That means, for any to enhancement layer picture samples that lie within the same enhancement layer tile, the collocated base layer samples shall also lie within the same base layer tile. tile_boundaries_aligned_flag equal to 0 indicates that there is no restriction on the tile configuration between corresponding base layer and the given enhancement layer.

# References

- B. Bross, “High Efficiency Video Coding (HEVC) text specification draft 10“, JCTVC-K1003, Shanghai, CN, Oct. 2012

- “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)“, ISO/IEC JTC1/SC29/WG11 N12957, Stockholm, Sweden, July 2012

- K. Suehring , R. Skupin, Y. Sanchez, T. Schierl, “Independent tile upsampling for SHVC“, JCTVC-M0198, Incheon, KR, Apr. 2013

- G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl, “Restriction of inter-view prediction on tile boundaries“, JCT3V-Dxxxx, Incheon, KR, Apr. 2013

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).