| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0203 |
| --- | --- |

| Title: | AHG 9: Signalling Inter-layer Prediction Indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry (LG Electronics) Miska M. Hannuksela, Kemal Ugur (Nokia) Do-Kyoung Kwon (Texas Instruments) | Tel: Email: | hendry.hendry@lge.com miska.hannuksela@nokia.com kemal.ugur@nokia.com d-kwon@ti.com |
| Source: | LG Electronics, Nokia, Texas Instruments |  |  |

_____________________________

# Abstract

Study on the trade-off between coding efficiency and implementation cost of enabling / disabling inter-layer prediction based on temporal sub-layer has been reported in JCTVC-L0278. Furthermore, the initial design for signalling method to support controlling the use of inter-layer prediction based on temporal sub-layer at sequence level has been provided in JCTVC-L0449. This contribution re-proposes the signalling of inter-layer prediction at sequence level as described in JCTVC-L0449 and also provides use-cases describing the asserted benefits of such signalling..

# Introduction

To reduce complexity, it is suggested that the use of inter-layer prediction (ILP) is controlled and that the encoder can indicate at sequence level for which types of pictures or sub-layers ILP is in use. For example, to support lightweight single loop decoding, ILP is used only when base layer picture is a random access picture (RAP) [1]. It was also suggested to enable / disable ILP according to temporal sub-layer [2][3]. For enabling / disabling ILP according to sub-layer, it has been shown that coding loss from disabling ILP at the highest temporal sub-layer is only 1.0% and 2.1% for RA_2x and RA_1.5x configurations, respectively, against SMuC [3].

To support the idea of enabling / disabling inter-layer prediction at sequence level, initial for signalling of inter-layer prediction indication at sequence level has been suggested in JCTVC-L0449 [4]. This contribution provides use-case scenario to describe how the signalling design in [4] is beneficial.

# Use-case scenarios

Controlling the use of ILP at different sub-layer allows encoder / decoder to control the amount of additional complexity to handle scalable bitstreams. Pictures in the sub-layer that are declared to be not used for ILP can simply be removed from input bitstream / skipped from decoding process. The following stages of controlling the use ILP can be available:

- Allow ILP only for base layer RAP pictures. This configuration is a lightweight option for many encoders as no multi-loop is necessary and inter-picture related syntax elements need not to be decoded.  It has been shown in [1] that by using ILP only for RAP pictures in base layer, about 5.9%, 8.5%, and 7.5% coding gain for luma for RA HEVC 2x, RA HEVC 1.5x, and RA HEVC SNR configuration, respectively, can be achieved, against simulcast.

- Allow only pictures at predefined first temporal sub-layer for ILP. For example, only picture at the first two temporal sub-layers as illustrated in Figure 1 are to be used for ILP.

Figure 1 – Example of disabling ILP for temporal sub-layer 2 and 3 in layer 0.

# Proposal

The proposed syntax and semantics are identical to those of JCTVC-L0449. It is noted that one of the mandates of AHG9 was to study inter-layer prediction constraints by taking JCTVC-L0449 as a starting point.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| for( j = 0; j <i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| for( i = 0; i<= vps_max_layers_minus1; i++ ) { |  |
| max_sublayer_for_ilp_plus1[ i ] | u(3) |
| } |  |
| } |  |

max_sublayer_for_ilp_plus1[ i ] equal to 0 indicates that non-RAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_sublayer_for_ilp_plus1[ i ] greater than 0 indicates that pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and with TemporalId greater than max_sublayer_for_ilp_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_sublayer_for_ilp_plus1[ i ] is unspecified.

# References

[1] “Lightweight single-loop scalability with SHVC,” JCTVC-L0111, the 12th JCTVC meeting, Geneva, CH, January 2013.

[2] “Inter layer prediction indication flags and pictures management,” JCTVC-L0258, the 12th JCTVC meeting, Geneva, CH, January 2013.

[3] “SHVC: On Inter-layer Prediction,” JCTVC-L0278, the 12th JCTVC meeting, Geneva, CH, January 2013.

[4] “Design Considered for Signalling Inter-layer Prediction Indication,” JCTVC-L0449, the 12th JCTVC meeting, Geneva, CH, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).