| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0204 |
| --- | --- |

| Title: | Ultra-low delay multi-layer decoding by interleaving of decoding units in SHVC and 3D/MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution proposes to allow a bitstream layout that enables multi-layer ultra-low delay decoding operation by parallel decoding of sub-pictures. Decoding units belonging to different layers/views are allowed to be interleaved within an access unit. The order is constrained in a way that all necessary reference layer/view data shall be available before sending an enhancement layer decoding unit. In addition this contribution proposes a clarification on the definition of access units and coded pictures in multi-layer coding environments.

# Introduction

Parallel decoders can decode multiple layers in parallel. Without any restriction, a decoder is required to finalize decoding of all reference views in 3D- and MV-HEVC and with multi-loop decoding all reference layers in SHVC. With constrained inter-layer/inter-view prediction, e.g. as described in [5], [6] and Error: Reference source not found, decoding of enhancement layer or dependent views can be started as soon as a specific area of the reference has been decoded. Such areas can for instance be slices, tiles or sub-streams.

With traditional bistream layout, it is still necessary to wait until all NAL units of a base layer/view (and maybe other dependent layers/views) have arrived in the CPB, before decoding of an enhancement layer/view can start, for example with 3 view each containing 3 decoding units:

Minimum delay for decoding unit 1 of view 3

Minimum Delay

Minimum Delay

Minimum Delay

view 1	view 2	view 3

The minimum decoding delay for decoding the first decoding unit of view three would include arrival of the complete first and second view.

To allow earlier access, we propose to allow interleaving of decoding units, for example with the above three views as follows:

Minimum delay

In this case, the delay for the first decoding unit of the third view can be significantly reduced to contain only the arrival of the first decoding unit of all views.

# Definitions

In the editing process for MV-HEVC definitions have been removed and an editing note has been added as follows:

Taken from the draft document of the MV-HEVC specification Error: Reference source not found

From: F.3 Definitions

[Ed. (MH): The present version of this annex uses the same definition for an access unit as in clause 3, which essentially states that an access unit contains one coded picture (with a particular value of nuh_layer_id). One coded picture is defined below essentially identically to a view component in MVC. It is an open issue whether an access unit should instead be defined to contain all view components with the same POC value.]

When allowing decoding units to be interleaved, it is not desirable for view components to be included in different access units. This means, that access units would have to be interleaved, which is conceptually undesirable.

Thus we suggest the following definitions:

access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded picture.

coded layer picture component: A coded representation of a layer picture component containing all coding tree units of a layer picture component.

coded picture: A coded representation of all coded layer picture components at a specific display time.

layer picture component: An array of luma samples in monochrome format or an array of luma samples and two corresponding arrays of chroma samples in 4:2:0, 4:2:2, and 4:4:4 colour format, which coded representation consists of the NAL units from a specific layer among all NAL units in an access unit.

picture: A picture is a set of all layer picture components at a specific display time.

If the specifications shall allow different GOP structures per layer or view (as for instance described in [8]), the following alternatives are proposed for coded picture and picture:

coded picture: A coded representation of a one or more coded layer picture components that are consecutive in bitstream order and are displayed at the same display time.

picture: A picture is a set of one or more layer picture components that are consecutive in bitstream order and are displayed at the same display time.

This enables multiple views at the same point in time to be collected in one access unit (which would be the case for low-delay GOPs), while views with other display times would form different access units. For non-consecutive VCL NAL units multiple views/layers at the same point in time would be collected into different access units. The drawback is that this would create multiple pictures at the same display time while the number of pictures is not equal to the number of view components. See, e.g. the figure below where different display time have a different number of access units depending on the decoding order.

The decoding order in the figure is depicted in red. The IDR picture would be an access unit consisting of the base layer and enhancement layer (see layer picture components with decoding order 1 and 2 in the figure), while the layer picture components with an intermediate display time (layer picture components with decoding order equal to 3 and 6) would consist of different access units.

# Interleaving of decoding units

Decoding units shall be ordered in a way, that all necessary reference sub-pictures are available, when an enhancement layer/dependent view is decoded. A decoder that decodes NAL units in sequential order, shall be able to decode each NAL unit in bitstream order. No caching shall be necessary.

The following text describes the bitstream constraint for such a bitstream order.

CtbRowBA(ctbAddrRs) and CtbColBA(ctbAddrRs) are defined as in [7] as follows and used in the following section:

CtbScalingFactorBA = ( PicWidthInCtbsYA / PicWidthInCtbsYB )

CtbRowBA(ctbAddrRs) =

Ceil( ( Floor( ctbAddrRs / PicWidthInCtbsYB ) + 1 ) * CtbScalingFactorBA) - 1

CtbColBA(ctbAddrRs) =

Ceil( ( ( ctbAddrRs % PicWidthInCtbsYB ) + 1 ) * CtbScalingFactorBA ) – 1

Order of VCL NAL units and association to coded pictures

This subclause specifies the order of VCL NAL units and association to coded pictures.

Each VCL NAL unit is part of a coded picture.

The order of the VCL NAL units within a coded layer picture component of a coded picture, i.e. VCL NAL units of a coded picture with the same layer_id_nuh value, is constrained as follows (same as 7.4.2.4.5):

–	The first VCL NAL unit of the coded layer picture component shall have first_slice_segment_in_pic_flag equal to 1.

–	Let sliceSegAddrA and sliceSegAddrB be the slice_segment_address values of any two coded slice segment NAL units A and B within the same coded layer picture component. When either of the following conditions is true, coded slice segment NAL unit A shall precede the coded slice segment NAL unit B:

–	TileId[ CtbAddrRsToTs[ sliceSegAddrA ] ] is less than TileId[ CtbAddrRsToTs[ sliceSegAddrB ] ].

–	TileId[ CtbAddrRsToTs[ sliceSegAddrA ] ] is equal to TileId[ CtbAddrRsToTs[ sliceSegAddrB ] ] and CtbAddrRsToTs[ sliceSegAddrA ] is less than CtbAddrRsToTs[ sliceSegAddrB ].

If a coded picture consists of more than one layer picture components the order of the VCL NAL units of all layer picture components is constrained as follows:

–	Let VCL NAL A be the the first VCL NAL unit A in a coded layer picture component layerPicA used as reference for another layer picture component layerPicB. Then VCL NAL unit A shall precede any VCL NAL unit B belonging to layerPicB.

–	Otherwise (not the first VCL NAL unit), if du_interleaving_enabled_flag is equal to 0, let VCL NAL A be any VCL NAL unit of a coded layer picture component layerPicA used as reference for another coded layer picture component layerPicB. Then VCL NAL unit A shall precede any VCL NAL unit B belonging to layerPicB.

–	Otherwise (not the first VCL NAL unit, and du_interleaving_enabled_flag is equal to 1), if tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 0 (i.e. neither tiles nor WPP are used in the video sequence), let layerPicA be a coded layer picture component that is used as reference for another coded layer picture component layerPicB. Let also VCL NAL unit B be any VCL NAL unit of the coded layer picture component layerPicB and VCL NAL unit A be the the preceeding VCL NAL unit from layerPicA with a value of slice_segment_address equal to sliceSegAddrA for which there are (min_spatial_segment_delay -1) VCL NAL units from layerPicA between VCL NAL unit A and VCL NAL unit B. Let also VCL NAL unit C be the next VCL NAL unit of the coded layer picture component layerPicB following VCL NAL unit B with a value of slice_segment_address equal to sliceSegAddrC. Let PicWidthInCtbsYA be the picture width in units of CTBs of layerPicA. Then, the following conditions shall be true:

–	There shall be always min_spatial_segment_delay VCL NAL units from layerPicA preceding VCL NAL unit B.

–	PicWidthInCtbsYA * CtbRowBA(sliceSegAddrC-1) + CtbColBA(sliceSegAddrC-1) shall be smaller than or equal to sliceSegAddrA-1.

–	Otherwise (not the first VCL NAL unit, and du_interleaving_enabled_flag is equal to 1), if tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1 (i.e. WPP is used in the video sequence), let sliceSegAddrA be the slice_segment_address of any segment NAL unit A of a coded layer picture component layerPicA that directly preceedes a slice segment VCL NAL unit B with slice_segment_address equal to sliceSegAddrB that belongs to a coded layer picture component layerPicB that uses layerPicA as reference. Let also PicWidthInCtbsYA be the picture width in units of CTBs of layerPicA. Then, the following condition shall be true:

–	( CtbRowBA(sliceSegAddrB) – Floor( (sliceSegAddrA) / PicWidthInCtbsYA) + 1) is equal or greater than min_spatial_segment_delay.

Otherwise (not the first VCL NAL unit, and du_interleaving_enabled_flag is equal to 1), if tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0 (i.e. tiles are used in the video sequence), let sliceSegAddrA be the slice_segment_address of any segment NAL unit A of a coded layer picture component layerPicA and slice segment VCL NAL unit B be the first following VCL NAL unit that belongs to a coded layer picture component layerPicB that uses layerPicA as reference with slice_segment_address equal to sliceSegAddrB. Let also PicWidthInCtbsYA be the picture width in units of CTBs of layerPicA. Then, the following condition shall be true:

–	TileId[ CtbAddrRsToTs[ PicWidthInCtbsYA * CtbRowBA(sliceSegAddrB-1) + CtbColBA(sliceSegAddrB-1) ] ] - TileId[ CtbAddrRsToTs[ sliceSegAddrA-1] ] shall be equal or greater than min_spatial_segment_delay.

# Syntax and Semantics

The following high level syntax elements are proposed to signal whether interleaving of layer/view components is allowed in the bitstream:

| video_parameter_set_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| du_interleaving_enabled_flag | u(1) |
| … |  |
| } |  |

du_interleaving_enabled_flag equal to 1, specifies that decoding units of different coded view/layer picture components may be interleaved within an access unit. When du_interleaving_enabled_flag is equal to 0, for each view/layer picture component, all decoding units follow each other directly in the bitstream (i.e. no interleaving is used.)

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand (Eds.), “High Efficiency Video Coding (HEVC) text specification draft 10“, JCTVC-L1003, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J.Boyce (Eds.), “MV-HEVC Draft Text 3 (ISO/IEC 23008-2 PDAM2)“, JCT3V-C1004, Geneva, CH, Jan. 2013

- G. Tech, K. Wegner, Y. Chen, S. Yea (Eds.), “3D-HEVC Test Model Description, draft specification“, JCT3V-C1005, Geneva, CH, Jan. 2013

- J. Chen, J. Boyce, Y. Ye, M. Hannuksela (Eds.), “SHVC Draft Text 1“, JCT3V-C1005, Geneva, CH, Jan. 2013

- K. Suehring , R. Skupin, Y. Sanchez, T. Schierl, “Independent tile upsampling for SHVC“, JCTVC-M0198, Incheon, KR, Apr. 2013

- G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl , “Restriction of inter-view prediction on tile boundaries“, JCT3V-D0051, Incheon, KR, Apr. 2013

- R. Skupin, K. Suehring, Y. Sanchez, T. Schierl, “Layer decoding delay indication for SHVC and 3D/MV-HEVC“, JCTVC-Mxxxx, Incheon, KR, Apr. 2013, JCTVC-M0200, Incheon, KR, Apr. 2013

- M. Hannuksela, A. Hallapuro, “SHVC HLS: support for unequal BL and EL GOP lengths“, JCTVC-L0171, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).