| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0206r1 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0043r1 |

| Title: | MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that MV-HEVC and SHVC drafts do not allow starting the decoding process from a CRA picture (with nuh_layer_id equal to 0 and a particular POC value), when some of the pictures with the same POC value and with nuh_layer_id greater than 0 are non-IRAP pictures. It is proposed to allow such decoding operation with the following modifications:

- The decoding of layers is started stepwise as follows:

  - The decoding of an enhancement layer (with a particular nuh_layer_id value) is started from an IRAP picture in that enhancement layer, when the decoding of all reference layers of that enhancement layer has been started.

  - The non-IRAP pictures with nuh_layer_id greater than 0 preceding, in decoding order, the first decoded IRAP picture with that nuh_layer_id value are not decoded.

- NoRaslOutputFlag is derived for IRAP pictures in each layer and the generation of unavailable reference pictures is clarified to be specific to the current layer.

In Revision 1, the proposed decoding process is modified so that rather than leaving pictures not decoded, unavailable pictures are generated for them.

# Problem Statement

MV-HEVC and SHVC drafts specify that CRA pictures need not be aligned across layers. In other words, for the same POC value, one layer may have a CRA picture, while another layer has another type of a picture. This allows different RAP periods in different layers. For example, more frequent RAP pictures can be used in the base layer, where they are cheaper due to a smaller spatial resolution.

It is allowed in HEVC that a bitstream starts with a CRA picture. This allows for example decoders to start decoding of a broadcast from a CRA picture (without a need to wait for the next IDR picture to appear in the received bitstream).

MV-HEVC and SHVC drafts have two problems for starting the decoding process from a CRA picture (with nuh_layer_id equal to 0). These problems are illustrated in Figure 1.

- Non-IRAP pictures with nuh_layer_id greater than 0 refer to pictures that do not exist in the DPB. This is illustrated in blue in Figure 1. Non-IRAP pictures with nuh_layer_id greater than 0 can be decoded only after the first IRAP picture having that nuh_layer_id value.

- RASL pictures with nuh_layer_id greater than 0 (associated with the first IRAP picture with that nuh_layer_id) refer to pictures that do not exist in the DPB. This is illustrated in red in Figure 1. Further details on the causes of this problem are described below.

NoRaslOutputFlag is assigned a new value only when decoding a picture with nuh_layer_id equal to 0, because its assignment takes place in subclause 8.1.1 (“Decoding process for a coded picture with nuh_layer_id equal to 0”). However, NoRaslOutputFlag is used in the decoding process of pictures with nuh_layer_id greater than 0 as if it was set layer-wise. For example, in subclause F.8.1.2: “If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.” The associated IRAP picture is defined as the previous IRAP picture in decoding order within the same layer (if present).

The decoding process for generating unavailable reference pictures (subclause 8.3.3) does not take the nuh_layer_id value into account. It is therefore unclear how it is applied, when invoked from the decoding process of a picture with nuh_layer_id greater than 0.

As a consequence of the issues described in the paragraphs above, no unavailable reference pictures with nuh_layer_id greater than 0 will be created for an IRAP picture with nuh_layer_id greater than 0 and hence RASL pictures with nuh_layer_id greater than 0 refer to pictures that do not exist in the DPB.

Figure 1. Illustration of problems when starting decoding from a CRA picture with nuh_layer_id equal to 0.

# Overview of the Proposal

It is proposed to tackle the problems described in the previous section as follows.

- The decoding of layers is started stepwise as follows:

  - When the first picture of a CVS, with a particular non-zero nuh_layer_id, is a non-IRAP picture (e.g. the picture indicated in blue in Fig. 1), the decoding process for generating unavailable reference pictures is invoked for it.

  - The pictures preceding the first IRAP picture, in decoding order, are not output.

- NoRaslOutputFlag is derived for the first IRAP picture in each layer and consequently its RASL pictures are not output.

# Proposed Specification Text Changes

The proposed specification text changes are color-coded consistently with the color coding above. Changes in definitions are indicated in green.

coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an IRAP access unit withcontaining an IRAP picture having nuh_layer_id equal to 0 and NoRaslOutputFlag equal to 1, followed by zero or more access units that are not IRAP access units withcontaining an IRAP picture having nuh_layer_id equal to 0 and NoRaslOutputFlag equal to 1, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit withcontaining an IRAP picture having nuh_layer_id equal to 0 and NoRaslOutputFlag equal to 1. [Ed. (MH): Consider defining an initial IRAP access unit to be an IRAP access unit containing an IRAP picture having nuh_layer_id equal to 0 and NoRaslOutputFlag equal to 1.]

intra random access point (IRAP) access unit: An access unit in which the coded picture with nuh_layer_id equal to 0 is an IRAP picture.

- 8.1	General decoding process

…

The decoding process is specified such that all decoders will produce numerically identical cropped decoded pictures. Any decoding process that produces identical cropped decoded pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

When the current picture is an IRAP picture that is the first picture in the CVS, the variable LayerInitialisedFlag[ 0 ] is set equal to 1, the variable LayerInitialisedFlag[ i ] is set equal to 0 for all values of i from 1 to 63, inclusive, and the variable FirstPicInLayerFlag[ i ] is set equal to 0 for all values of i from 1 to 63, inclusive.

When the current picture is an IRAP picture, the following applies:

–	If both of the following are true, the variable NoRaslOutputFlag is set equal to 1.

–	nuh_layer_id is equal to 0.

–	The current picture is an IDR picture, a BLA picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture with nuh_layer_id equal to 0, the variable  HandleCraAsBlaFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

When the current picture is an IRAP picture and LayerInitialised[ nuh_layer_id ] is equal to 0 and LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ] ][ j ], where j is in the range of 0 to NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1, inclusive, LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1.

When the current picture has nuh_layer_id equal to 0, the decoding process for a coded picture with nuh_layer_id equal to 0 specified in subclause  is invoked.

- 8.1.1	Decoding process for a coded picture with nuh_layer_id equal to 0

When the current picture is a BLA picture that has nal_unit_type equal to BLA_W_LP or is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable UseAltCpbParamsFlag to a value, UseAltCpbParamsFlag is set equal to the value provided by the external means.

–	Otherwise, the value of UseAltCpbParamsFlag is set equal to 0.

When the current picture is an IRAP picture, the following applies:

–	If the current picture with a particular nuh_layer_id is an IDR picture, a BLA picture, the first picture with that particular nuh_layer_id in the bitstream in decoding order, or the first picture with that particular nuh_layer_id that follows an end of sequence NAL unit in decoding order, the variable NoRaslOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value for the current picture, the variable  HandleCraAsBlaFlag is set equal to the value provided by the external means and the variable NoRaslOutputFlag is set equal to HandleCraAsBlaFlag.

–	Otherwise, the variable HandleCraAsBlaFlag is set equal to 0 and the variable NoRaslOutputFlag is set equal to 0.

…

- 8.3.3	Decoding process for generating unavailable reference pictures

- 8.3.3.1	General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1.

NOTE – This process is primarily specified only for the specification of syntax constraints for RASL pictures. [Ed. (MH) The note should be updated to discuss layer-wise decoding start-up too.] The entire specification of the decoding process for RASL pictures associated with an IRAP picture that has NoRaslOutputFlag equal to 1 is included herein only for purposes of specifying constraints on the allowed syntax content of such RASL pictures. During the decoding process, any RASL pictures associated with an IRAP picture that has NoRaslOutputFlag equal to 1 may be ignored, as these pictures are not specified for output and have no effect on the decoding process of any other pictures that are specified for output. However, in HRD operations as specified in Annex Error: Reference source not found, RASL access units may need to be taken into consideration in derivation of CPB arrival and removal times.

When this process is invoked, the following applies:

- For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies:

- The value of PicOrderCntVal for the generated picture is set equal to PocStFoll[ i ].

- The value of PicOutputFlag for the generated picture is set equal to 0.

- The generated picture is marked as "used for short-term reference".

- RefPicSetStFoll[ i ] is set to be the generated reference picture.

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

- For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies:

- The value of PicOrderCntVal for the generated picture is set equal to PocLtFoll[ i ].

- The value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb – 1 ) ).

- The value of PicOutputFlag for the generated picture is set equal to 0.

- The generated picture is marked as "used for long-term reference".

- RefPicSetLtFoll[ i ] is set to be the generated reference picture.

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id.

…

- F.7.4.2.4.2	Order of VPS, SPS and PPS RBSPs and their activation

- …

- When an SPS RBSP (with a particular value of sps_seq_parameter_set_id) is not already active for a nuh_layer_id value and it is referred to by activation of a PPS RBSP for that nuh_layer_id value (in which pps_seq_parameter_set_id is equal to the sps_seq_parameter_set_id value), it is activated for that nuh_layer_id value. This SPS RBSP is called the active layer SPS RBSP for that nuh_layer_id value until it is deactivated by the activation of another SPS RBSP for the same layer. An SPS RBSP, with that particular value of sps_seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0 or provided through external means. An activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from the first picture, in decoding order, an IDR or BLA picture having that nuh_layer_id value within the CVS, inclusive, until either the next IDR or BLA picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier. The nuh_layer_id value the NAL unit containing the SPS RBSP that is activated for nuh_layer_id equal to nuhLayerIdA shall be less than or equal to nuhLayerIdA. The same SPS RBSP may be the active layer SPS for more than one nuh_layer_id value.

…

- F.8.1	General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause  is invoked.

–	When ViewId[ nuh_layer_id ] is greater than 0, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause  is invoked.

- F.8.1.1	Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0

Each picture referred to in this subclause is a complete coded picture.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture. It is a requirement of bitstream conformance that PicOrderCntVal shall remain unchanged within an access unit.

–	The decoding process for RPS in subclause 8.3.2 is invoked for pictures with nuh_layer_id equal to that of CurrPic, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When FirstPicInLayerFlag[ nuh_layer_id ] is equal to 0CurrPic is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

- F.8.1.2	Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, if LayerInitialised[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

The following applies:

–	The decoded picture is marked as "used for short-term reference".

–	When TemporalId is equal to HighestTid, the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause Error: Reference source not found is invoked with latestDecLayerId equal to nuh_layer_id as input.

–	FirstPicInLayerFlag[ nuh_layer_id ] is set equal to 1.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).