| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 13th Meeting: Incheon, KR, 18 –26 April, 2013 | Document: JCTVC-M0210 WG11 Number: m28664 |
| --- | --- |

| Title: | On Signaling DPB Parameters and DPB Operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes signaling vps_max_dec_pic_buffering_minus1, vps_max_num_reorder_pics, vps_max_latency_increase_plus1 parameters for sub-layers for the CVS for operation points in VPS extension for SHVC and MV-HEVC. In a variant proposal these parameters could be signaled in VPS extension for the output layer sets instead of for operation points. The document also proposes the operation and bumping process for output order DPB of SHVC and MV-HEVC to use the above parameters from VPS extension if they are signaled for the selected operation point (or output layer set in the variant proposal). Otherwise the SPS level parameters are used.

# Introduction

Currently in SHVC Draft Text 1 [1] and MV-HEVC Dratf Text 3 [2] the parameters sps_max_num_reorder_pics[ ], sps_max_latency_increase_plus1[ ] and sps_max_dec_pic_buffering[ ] are only signaled in SPS of each layer.

This document proposes the following:

- This document proposes signaling vps_max_dec_pic_buffering_minus1, vps_max_num_reorder_pics, vps_max_latency_increase_plus1 parameters for sub-layers for the CVS for one or more operation points in VPS extension. It is asserted that this allows defining these parameters to allow bumping of pictures depending upon the particular set of layers included in the operation point under test.

Also it is asserted that signalling these syntax elements in VPS extension can help in session negotiation. For example vps_max_dec_pic_buffering_minus1[ id ][  j ] specifies the maximum required size of the decoded picture buffer for the CVS for layer with nuh_layer_id equal to id in units of picture storage buffers when HighestTid is equal to j. This information is useful for the decoder in terms of session negotiation based on its available DPB size in determining if it can select to operate at a certain operation point and the associated layer set.

In a variant proposal vps_max_dec_pic_buffering_minus1, vps_max_num_reorder_pics, vps_max_latency_increase_plus1 parameters could be signaled in VPS extension for the output layer sets instead of for operation points.

- This document additionally proposes the operation and bumping process for output order DPB to use the above signaled DPB buffering parameters from VPS extension if they are signaled for the selected operation point (or output layer set in the variant proposal). Otherwise the SPS level parameters from the active SPS (when layer_id of the current layer is equal to 0) or from the active layer SPS depending upon the layer_id of the current layer are used.

# Proposal

We propose following syntax and semantics. The proposed changes are highlighted below compared to SHVC Draft Text 1 [1] and are also applicable to MV-HEVC Draft Text 3 [2].

## Syntax and Semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| … |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| num_op_dpb_info_parameters | ue(v) |
| for( i = 0; i < num_op_dpb_info_parameters; i++ ) { |  |
| operation_point_layer_set_idx[ i ] | ue(v) |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| op_dpb_info_parameters( i, j ) |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

- num_op_dpb_info_parameters specifies the number of op_dpb_parameters( ) syntax structures present in the VPS extension RBSP. num_op_dpb_info_parameters decoders shall be in the range of 0 to vps_num_layer_sets_minus1, inclusive.

operation_point_layer_set_idx[ i ] specifies the index, into the list of layer sets defined by operation points to which the i-th op_dpb_info_parameters( ) syntax structure in the VPS extension applies.

The value of operation_point_layer_set_idx[ i ] should be in the range of 0 to vps_num_layer_sets_minus1, inclusive. It is requirement of bitstream conformance that

operation_point_layer_set_idx[ i ] shall not be equal to operation_point_layer_set_idx[ j ] for any j not equal to i.

| op_dpb_info_parameters( id , j ) { |  |
| --- | --- |
| vps_max_sub_layers_minus1[ id ] [ j ] | u(3) |
| vps_sub_layer_ordering_info_present_flag[ id ] [ j ] | u(1) |
| for( k = ( vps_sub_layer_ordering_info_present_flag[ id ] [ j ] ? 0 : vps_max_sub_layers_minus1[ id ] [ j ]); k <= vps_max_sub_layers_minus1[ id ] [ j ]; k++ ) { |  |
| vps_max_dec_pic_buffering_minus1[ id ] [ j ] [ k ] | ue(v) |
| vps_max_num_reorder_pics[ id ] [ j ] [ k ] | ue(v) |
| vps_max_latency_increase_plus1[ id ] [ j ] [ k ] | ue(v) |
| } |  |
| } |  |

vps_max_sub_layers_minus1[ id ] [ j ] plus 1 specifies the maximum number of temporal sub-layers that may be present in the CVS for layer with nuh_layer_id equal to j for the operation point associated with index id. The value of vps_max_sub_layers_minus1[ id ] [ j ] shall be in the range of 0 to 6, inclusive.

vps_sub_layer_ordering_info_present_flag[ id ] [ j ] equal to 1 specifies that vps_max_dec_pic_buffering_minus1[ id ][ j ][ k ], vps_max_num_reorder_pics[ id ][ j ][ k ], and vps_max_latency_increase_plus1[ id ][ j ][ k ] are present for layer with nuh_layer_id equal to j for the operation point associated with index id for vps_max_sub_layers_minus1[ id ][ j ] + 1 sub-layers. vps_sub_layer_ordering_info_present_flag[ id ][ j ] equal to 0 specifies that the values of vps_max_dec_pic_buffering_minus1[ id ][ j ][ vps_max_sub_layers_minus1[ id ][ j ] ], vps_max_num_reorder_pics[ id ][ j ][ vps_max_sub_layers_minus1[ id ][ j ] ], and vps_max_latency_increase_plus1[ id ][ j ][ vps_max_sub_layers_minus1[ id ][ j ] ] apply to all sub-layers for layer with nuh_layer_id equal to j for the operation point associated with index id.

vps_max_dec_pic_buffering_minus1[ id ][ j ][ k ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS for layer with nuh_layer_id equal to j for the operation point associated with index id in units of picture storage buffers when HighestTid is equal to k. The value of vps_max_dec_pic_buffering_minus1[ id ][ j ][ k ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause Error: Reference source not found), inclusive. When k is greater than 0, vps_max_dec_pic_buffering_minus1[ id ][ j ][ k ] shall be greater than or equal to vps_max_dec_pic_buffering_minus1[ id ][ j ][ k − 1 ]. When vps_max_dec_pic_buffering_minus1[ id ][ j ][ k ] is not present for k in the range of 0 to vps_max_sub_layers_minus1[ id ][ j ] − 1, inclusive, due to vps_sub_layer_ordering_info_present_flag[ id ][ j ] being equal to 0, it is inferred to be equal to vps_max_dec_pic_buffering_minus1[ id ][ j ][ vps_max_sub_layers_minus1[ id ][ j ] ].

vps_max_num_reorder_pics[ id ][ j ][ k ] indicates the maximum allowed number of pictures that can precede any picture in the CVS for layer with nuh_layer_id equal to j for the operation point associated with index id in decoding order and follow that picture in output order when HighestTid is equal to k. The value of vps_max_num_reorder_pics[ id ][ j ][ k ] shall be in the range of 0 to vps_max_dec_pic_buffering_minus1[ id ][ j ][ k ], inclusive. When k is greater than 0, vps_max_num_reorder_pics[ id ][ j ][ k ] shall be greater than or equal to vps_max_num_reorder_pics[ id ][ j ][ k − 1 ]. When vps_max_num_reorder_pics[ id ][ j ][ k ] is not present for k in the range of 0 to vps_max_sub_layers_minus1[ id ][ j ] − 1, inclusive, due to vps_sub_layer_ordering_info_present_flag[ id ][ j ] being equal to 0, it is inferred to be equal to vps_max_num_reorder_pics[ id ][ j ][ vps_max_sub_layers_minus1[ id ][ j ] ].

vps_max_latency_increase_plus1[ id ] [ j ][ k ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ id ][ j ][ k ], which specifies the maximum number of pictures that can precede any picture in the CVS for layer with nuh_layer_id equal to j for the operation point associated with index id in output order and follow that picture in decoding order when HighestTid is equal to k.

When vps_max_latency_increase_plus1[ id ][ j ][ k ] is not equal to 0, the value of VpsMaxLatencyPictures[ id ][ j ][ k ] is specified as follows:

VpsMaxLatencyPictures[ id ][ j ][ k ] = vps_max_num_reorder_pics[ id ][ j ][ k ] + 	vps_max_latency_increase_plus1[ id ][ j ][ k ] − 1

When vps_max_latency_increase_plus1[ id ][ j ][ k ] is equal to 0, no corresponding limit is expressed.

The value of vps_max_latency_increase_plus1[ id ][ j ][ k ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase_plus1[ id ][ j ][ k] is not present for k in the range of 0 to vps_max_sub_layers_minus1[ id ][ j ] − 1, inclusive, due to vps_sub_layer_ordering_info_present_flag[ id ][ j ] being equal to 0, it is inferred to be equal to vps_max_latency_increase_plus1[ id ][ j ][ vps_max_sub_layers_minus1[ id ][ j ] ].

Variant A:

In this case the parameters vps_max_dec_pic_buffering_minus1, vps_max_num_reorder_pics, vps_max_latency_increase_plus1 parameters are signaled in the VPS extension for output layer sets instead of for operation points.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| … |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| num_dpb_info_parameters | ue(v) |
| for( i = 0; i < num_dpb_info_parameters; i++ ) { |  |
| output_point_layer_set_idx[ i ] | ue(v) |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| op_dpb_info_parameters( i, j ) |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

- num_dpb_info_parameters specifies the number of oop_dpb_parameters( ) syntax structures present in the VPS extension RBSP. num_dpb_info_parameters decoders shall be in the range of 0 to num_output_layer_sets, inclusive.

output_point_layer_set_idx[ i ] specifies the index, into the list of target output layer sets to which the i-th oop_dpb_info_parameters( ) syntax structure in the VPS extension applies.

The value of output_point_layer_set_idx[ i ] should be in the range of 0 to num_output_layer_sets, inclusive. It is requirement of bitstream conformance that

output_point_layer_set_idx[ i ] shall not be equal to output_point_layer_set_idx[ j ] for any j not equal to i.

Additional Proposal:

Additionally This document proposes the operation and bumping process for output order DPB to use the above signaled DPB buffering parameters from VPS extension if they are signaled for the selected operation point (or output layer set in variant A proposal). Otherwise the SPS level parameters from the active SPS (when layer_id of the current layer is equal to 0) or from the active layer SPS depending upon the layer_id of the current layer are used.

The proposed changes are highlighted below compared to SHVC Draft Text 1 [1] and are also applicable to MV-HEVC Draft Text 3 [2].

- F.13.5.2 Operation of the output order DPB

F.13.5.2.1 General

The decoded picture buffer contains picture storage buffers. The number of picture storage buffers for nuh_layer_id equal to 0 is derived from the active SPS. The number of picture storage buffers for each non-zero nuh_layer_id value is derived from the active layer SPS for that non-zero nuh_layer_id value. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. The process for output and removal of pictures from the DPB as specified in subclause  is invoked, followed by the invocation of the process for picture decoding, marking, additional bumping, and storage as specified in subclause . The "bumping" process is specified in subclause  and is invoked as specified in subclauses  and .

F.13.5.2.2 Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows.

The decoding process for RPS as specified in subclause 8.3.2 is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers  containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause , and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture.

The variables MaxNumReorderPics[ TargetOp ][ currLayerId ][ HighestTid ], MaxLatencyIncreasePlus1[ TargetOp ][ currLayerId ][ HighestTid ], MaxLatencyPictures[ TargetOp ][ currLayerId ][ HighestTid ], MaxDecPicBufferingMinus1[ TargetOp ] [ currLayerId ][ HighestTid ] are derived as follows based on the current operation point under test:

–	If operation point DPB information parameters op_dpb_info_parameters() are present for the operation point under test TargetOp, MaxNumReorderPics[ TargetOp ][ currLayerId ][HighestTid ] is set to vps_max_num_reorder_pics[ HighestTid ] when currLayerId is equal to 0 or is set to vps_max_num_reorder_pics[ TargetOp ][ CurrLayerId ][ HighestTid ] for the currLayerId for the operation point under test when currLayerId is greater than 0. Otherwise if operation point DPB information parameters op_dpb_info_parameters() are not present for the operation point under test MaxNumReorderPics[ TargetOp ][ currLayerId ][ HighestTid ] is set to sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	If operation point DPB information parameters op_dpb_info_parameters() are present for the operation point under test TargetOp, MaxLatencyIncreasePlus1[ TargetOp ][ currLayerId ][ HighestTid ] is set to vps_max_latency_increase_plus1[ HighestTid ] when currLayerId is equal to 0 or is set to vps_max_latency_increase_plus1[ TargetOp ][ CurrLayerId ][ HighestTid ] for the currLayerId for the operation point under test when currLayerId is greater than 0. If operation point DPB information parameters op_dpb_info_parameters() are present for the operation point under test, MaxLatencyPictures[ TargetOp ][ currLayerId ][ HighestTid ] is set to VpsMaxLatencyPictures[ HighestTid ] when currLayerId is equal to 0 or is set to VpsMaxLatencyPictures[ TargetOp ][ CurrLayerId ][ HighestTid ] for the currLayerId for the operation point under test when currLayerId is greater than 0. Otherwise if operation point DPB information parameters op_dpb_info_parameters() are not present for the operation point under test,  MaxLatencyIncreasePlus1[ TargetOp ][ currLayerId ][ HighestTid ] is set to sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId and MaxLatencyPictures[ TargetOp ] [ currLayerId ][ HighestTid ] is set to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	If operation point DPB information parameters op_dpb_info_parameters() are present for the selected operation point under test TargetOp, MaxDecPicBufferingMinus1[ TargetOp ][ currLayerId ][ HighestTid ] is set to vps_max_dec_pic_buffering_minus1[ HighestTid ] when currLayerId is equal to 0 or is set to vps_max_dec_pic_buffering_minus1[ TargetOp ][ CurrLayerId ][ HighestTid ] for the currLayerId for the operation point under test when currLayerId is greater than 0. Otherwise if operation point DPB information parameters op_dpb_info_parameters() are not present for the operation point under test,  MaxDecPicBufferingMinus1[ TargetOp ][ currLayerId ][ HighestTid ] is set to sps_max_dec_pic_buffering_minus1[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

When one or more of the following conditions are true, the "bumping" process specified in subclause  is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than MaxNumReorderPics[ TargetOp ][ CurrLayerId ][ HighestTid ] sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	If MaxLatencyIncreasePlus1[ TargetOp ][ CurrLayerId ][ HighestTid ] sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to MaxLatencyPictures[ TargetOp ][ CurrLayerId ][ HighestTid ] SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB is greater than or equal to MaxDecPicBuffering[ TargetOp ][ CurrLayerId ][ HighestTid ] sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

F.13.5.2.3 Picture decoding, marking, additional bumping, and storage

The processes specified in this subclause happen instantaneously when the last decoding unit of access unit n containing the current picture is removed from the CPB.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture.

For each picture in the DPB that is marked as "needed for output" and that has a nuh_layer_id value equal to currLayerId, the associated variable PicLatencyCount[ currLayerId ] is set equal to PicLatencyCount[ currLayerId ] + 1.

The current picture is considered as decoded after the last decoding unit of the picture is decoded. The current decoded picture is stored in an empty picture storage buffer in the DPB, and the following applies:

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output" and its associated variable PicLatencyCount[ currLayerId ] is set equal to 0.

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

The current decoded picture is marked as "used for short-term reference".

When one or more of the following conditions are true, the "bumping" process specified in subclause  is invoked repeatedly until none of the following conditions are true.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than MaxNumReorderPics[ TargetOp ][ CurrLayerId ][ HighestTid ] sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	MaxLatencyIncreasePlus1[ TargetOp ][ CurrLayerId ][ HighestTid ] sps_max_latency_increase_plus1[ HighestTid ] is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] that is greater than or equal to SpsMaxLatencyPictures[ TargetOp ][ CurrLayerId ][ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

F.13.5.2.4 "Bumping" process

The "bumping" process consists of the following ordered steps:

- The pictures that are first for output are selected as the ones having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- These pictures are cropped, using the conformance cropping window specified in the active SPS for the picture with nuh_layer_id equal to 0 or in the active layer SPS for a nuh_layer_id value equal to that of the picture, the cropped pictures are output in ascending order of nuh_layer_id, and the pictures are marked as "not needed for output".

- Each picture storage buffer that contains a picture marked as "unused for reference" and that included one of the pictures that was cropped and output is emptied.

# Conclusion

This document proposes signaling vps_max_dec_pic_buffering_minus1, vps_max_num_reorder_pics, vps_max_latency_increase_plus1 parameters for sub-layers for the CVS for operation points in VPS extension. In a variant proposal these parameters could be signaled in VPS extension for the output layer sets instead of for operation points. We propose to adopt one of these two signaling approaches in SHVC and MV-HEVC draft texts.

The document also proposes the operation and bumping process for output order DPB to use the above parameters from VPS extension if they are signaled for the selected operation point (or output layer set in the variant proposal). Otherwise the SPS level parameters are used. We propose to adopt this in SHVC and MV-HEVC working drafts.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-L1008, Geneva, March 2013

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 3 (ISO/IEC 23008-2:201x/PDAM2),” JCTVC-C1004_d3, Geneva, January 2013.