| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0213 |
| --- | --- |

| Title: | SCE4.2.6: Adaptive up-sampling of base layer picture using bilateral filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Kiran Misra, and Andrew Segall 5700 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1-360-817-8451 jzhao@sharplabs.com, misrak@sharplabs.com, asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports the performance of JCTVC-L0252 in the SCE4.  JCTVC-L0252 proposed the use of non-linear and content adaptive “bilateral” filters for inter-layer prediction.  Filtering is performed following the upsampling operation and switched (enabled/disabled) on a CU basis.  Multiple filters can be enabled and signaled in the PPS.  Results are reported following SCE4 test conditions and show EL+BL rate improvements compared to SHM1.0 anchors of: -1.5% (AI 2x), -0.5% (AI 1.5x), -1.4% (RA 2x), -0.3% (RA 1.5x), -0.5% (RA SNR), -1.4% (LD-P 2x), -0.3 (LD-P 1.5x), and -0.9% (LD-P SNR).

# Introduction

We propose a non-linear and content adaptive upsampling process using SHM-1.0 with INTRA_BL framework. Here, when the base layer picture is used for reference, i.e. INTRA_BL mode is ON, multiple versions of the upsampled base layer pictures are made available for inter-layer prediction. The first up-sampled base layer corresponds to the fixed up-sampler of SHM-1.0, while a second up-sampled version is generated by performing bilateral filtering on the first up-sampled base layer picture. Additional versions may also be generated by changing the parameters of the bilateral filter.  In the encoder, the upsampler is selected at a CU level in the function checkRDCostIntraBL based on RD cost.

Signaling the filter is accomplished with an up-sampler index that is added to the CU.  The index is sent only when the INTRA_BL mode is selected.  The number of active filters and filter parameters are signalled in PPS or slice header.

# Bilateral Filter

A bilateral filter is a non-linear filter [1].  It reduces the noise in the upsampled base layer while preserving edges. We assert that this is beneficial for spatial scalability, as it removes low amplitude and high frequency content that is normally associated with noise, for e.g. due to quantization, loop filtering, while preserving high amplitude and high frequency information corresponding to edges and structure.  This is achieved by replacing the intensity value at each pixel by a weighted average of intensity values from nearby pixels within a window. The weight is based on a Gaussian distribution and relative pixel intensities.

# We propose to apply bilateral filtering to the upsampled base layer picture. A window size of 5x5 is suggested.  For a pixel at location (x,y), the bilateral filtered output is then expressed as:

Ox,y=f(a,b,Ia,b,Ix,y)∗Ia,bf(a,b,Ia,b,Ix,y)									 (1)

where the summation is performed on pixels within a supporting window, Ia,b denotes the input pixel value at location (a,b), and f(a,b, Ia,b, Ix,y ) is a function that maps the spatial distance and relative pixel intensity information at a location (a,b) to a weight value.

The design of the function f() provides the desirable non-linear and content adaptive properties.  Traditionally, the function consists of two parts – a spatial kernel and a gradient kernel – and is expressed as:

f(a,b,Ia,b,Ix,y)=S(a,b)∗e−(G(Ia,b,Ix,y)22∗σR2)	(2)

where S(a,b) denotes a two-dimensional Gaussian kernel with standard deviation σS, G(a,b,Ix,y) denotes the absolute difference between Ia,b, and Ix,y, and σR is a control parameter. In general, the larger difference between the two pixel values, the lower the weight value.  This preserves edges and avoids over smoothing.    We advocate using simple differences to calculate the gradient.

In our proposal, we use a 5x5 spatial kernel LUT for S(a,b):

{ 4,  5,  6,  5,  4, },

{ 5,  6,  7,  6,  5, },

{ 6,  7,  8,  7,  6, },

{ 5,  6,  7,  6,  5, },

{ 4,  5,  6,  5,  4, },

For the Gradient kernel, we use a LUT as below and  min(G(Ia,b, I x,y)>>2, 8) is used as the index to the LUT.

{ 16,  15, 12,  9,  6,  4,  2, 1, 0 };

# Syntax

The upsampler index is signalled at CU level together with INTRA_BL flag.  For adaptive upsampling, we anticipate the support of multiple non-linear filters at the decoder.  Here, we propose syntax that is flexible for handling multiple upsampling filters

Example syntax tables are shown in the tables below.  Here, the number of bilateral filters is signalled in PPS. A most_probable_upsampler_idx  is signalled in slice header.  At CU level, a mapped upsampler index with respect to most_probable_upsampler_idx  is signalled.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| if( getLayerId() > 0 ) { |  |
| num_bilateral_filter | ue(v) |
| for( i = 0; i < num_bilateral_filter; i++ ) |  |
| bilateral_filter_idx[ i ] | ue(v) |
| } |  |
| num_extra_slice_header_bits | u(3) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| } |  |
| if( getLayerId() > 0 ) { |  |
| most_probable_upsampler_idx | ue(v) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( getLayerId() > 0 ) { |  |
| cu_intra_bl_flag[ x0 ][ y0 ] | u(1) |
| if(cu_ intra_bl_flag[ x0 ][ y0 ] ) { |  |
| cu_mapped_upsampler_idx[ x0 ][ y0 ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

cu_mapped_upsampler_idx[ x0 ][ y0 ] specify the mapped up-sampler index. The array indices  x0 , y0  specify the location ( x0 , y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The up-sampler index is obtained using:

cu_upsampler_idx[ x0 ][ y0 ] = cu_mapped_upsampler_idx[ x0 ][ y0 ]
if( cu_mapped_upsampler_idx[ x0 ][ y0 ] = = 0 ) {
	cu_upsampler_idx[ x0 ][ y0 ] = most_probable_upsampler_idx
} else if(cu_mapped_upsampler_idx[ x0 ][ y0 ] <= most_probable_upsampler_idx ) {
	cu_upsampler_idx[ x0 ][ y0 ]--
}

Mapping upsampler index to bilateral filter

if( getLayerId() >0 ) {
	for( i = 0; i < num_bilateral_filter; i++ ) {
		upsampler_idx_minus1_to_bilateral_filter_idx[ i ] = bilateral_filter_idx[ i ]
	}
}
Alternatively, the upsampler index may be signaled as a distance from a predicted upsampler index.

# Results

The proposed adaptive uspampler with BLT is implemented into SHM-1.0. Here, we enable one bilateral filter at the decoder.  We conduct the evaluation using SCE4 and SHM common test condition [2] [3] relative to SHM-1.0 anchor. As can be seen from Table 1, the method provides improved performance at higher spatial scalability ratios.  Specifically, we observe an overall (EL+BL) bit-rate reduction of  -1.5%, -1.4% and  -1.4% for AI, RA and LD-P 2X test conditions.  This is equivalent to an EL bit-rate reduction of -1.9%, -1.8% and -1.8% for AI, RA and LD-P 2X, respectively.  Bit-rate reductions for other test conditions are also observed and in the range of -.3% to -.9% (EL+BL).

Table 1. Result of Proposed Adaptive Upsampling Method vs. SHM1.0 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.4% | -2.5% | -3.0% |  |  |  |
| Class B | -1.1% | -1.0% | -1.2% | -0.5% | -0.5% | -0.7% |
| Overall (Test vs Ref) | -1.5% | -1.4% | -1.7% | -0.5% | -0.5% | -0.7% |
| Overall (Test vs single layer) | 10.8% | 12.1% | 11.5% | 9.7% | 9.8% | 8.9% |
| EL only (Test vs Ref) | -1.9% | -1.8% | -2.1% | 1.2% | 1.1% | 0.7% |
| Enc Time[%] | 122.1% |  |  | 119.7% |  |  |
| Dec Time[%] | 174.7% |  |  | 135.1% |  |  |
| Enc Mem[%] | 102.7% |  |  | 102.1% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.5% | 0.1% | -2.1% |  |  |  | -0.5% | -0.1% | -1.0% |
| Class B | -0.9% | -0.3% | -0.7% | -0.3% | 0.1% | -0.6% | -0.5% | -0.4% | -0.7% |
| Overall (Test vs Ref) | -1.4% | -0.2% | -1.1% | -0.3% | 0.1% | -0.6% | -0.5% | -0.3% | -0.8% |
| Overall (Test vs single layer) | 17.7% | 32.0%% | 31.7% | 16.2% | 28.7% | 30.2% | 14.2% | 28.6% | 31.8% |
| EL only (Test vs Ref) | -1.8% | -0.5% | -1.5% | 1.6% | 2.0% | 1.1% | -0.2% | 0.0% | -0.5% |
| Enc Time[%] | 109.6% |  |  | 106.2% |  |  | 104.4% |  |  |
| Dec Time[%] | 127.2% |  |  | 116.4% |  |  | 119.9% |  |  |
| Enc Mem[%] | 101.0% |  |  | 101.0% |  |  | 100.7% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.1% | -1.0% | -2.5% |  |  |  | -0.5% | -1.0% | -1.5% |
| Class B | -1.1% | -0.9% | -1.2% | -0.3% | -0.6% | -1.0% | -1.1% | -1.4% | -1.6% |
| Overall (Test vs Ref) | -1.4% | -0.9% | -1.6% | -0.3% | -0.6% | -1.0% | -0.9% | -1.3% | -1.6% |
| Overall (Test vs single layer) | 24.3% | 35.4% | 36.1% | 22.1% | 32.4% | 34.7% | 21.2% | 32.8% | 37.0% |
| EL only (Test vs Ref) | -1.8% | -1.2% | -1.9% | 2.2% | 1.9% | 1.4% | -0.9% | -1.3% | -1.7% |
| Enc Time[%] | 99.9% |  |  | 108.8% |  |  | 104.4% |  |  |
| Dec Time[%] | 125.2% |  |  | 128.0% |  |  | 122.4% |  |  |
| Enc Mem[%] | 101.0% |  |  | 101.2% |  |  | 101.0% |  |  |

# Complexity Assessment

In our decoder, the bilateral filter is a 2D non-separable filter performed at the enhancement layer resolution. It is applied only to the CU that is INTRA_BL and has an upsampler index equal to 1 (upsampler index 0 corresponds to the fixed upsampler) . The bilateral window size we used is 5x5.

For 8-bit unsigned pixel to be filtered, the proposed 5x5 bilateral filter requires a 20-bit unsigned accumulator and outputs 8-bit unsigned data. The worst-case complexity analysis corresponds to 75  (25*2 for luma and 25 for chroma) multiplication operations and 108  additions (24*3 for luma and 24*3/2 for chroma ) per enhancement layer pixel, and 1 division per CU. In addition, we also considered that in CU based implementation, the upsampled block size need to be increased to support the bilateral filtering. The summary complexity analysis table below lists the average number of memory accesses, multiplication and add computed. Additionally it lists the average number of division operations per sample. For detail, please refer to the complexity analysis excel sheet.

Table 2. Complexity summary of proposed adaptive upsampling method

# Conclusions

In this document, a non-linear and content adaptive bilateral filtering process is proposed.  Average bit-rate reductions (BL+EL) of 1.4% are observed for AI, RA and LDP 2X conditions relative to SHM-1.0 anchors for the S-HEVC test conditions.

# References

- C. Tomasi and R. Manduchi, “Bilateral filtering for gray and color images,” IEEE International Conference on Computer Vision, pp.839-846, 1998.

- J. Chen, A. Segall et al, “Description of HEVC Scalable Extensions Core Experiment 4 (SCE4): Inter-layer filtering,” JCTVC-L1104, Geneva, CH, Jan. 2013

- X. Li, J. Boyce et al, “Common SHM test conditions and software reference configurations,” JCTVC-L1109, Geneva, CH, Jan. 2013

- J. Zhao, K. Misrak, A. Segall, “Non-TE4: Adaptive up-sampling of base layer picture using bilateral filters,” JCTVC-L0252, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).