| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 April 2013 | Document: JCTVC-M0214 WG11 Number: m28671 |
| --- | --- |

| Title: | On Color Gamut Scalable Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Louis Kerofsky Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8486 sdeshpande@sharplabs.com lkerofsky@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This document proposes:

1.  A new bit depth scaling process for reference pictures to support color gamut scalable coding.

2.  Signaling color gamut scalability and bit depth information regarding layers in VPS extension.

In version 2 of this document minor editorial change is made marked with track changes.

# Introduction

The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the ITU-R BT.709 recommendation [1], while UHDTV will use the ITU-R BT.2020 recommendation [2].  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD.  It is asserted that this will provide a more “life like” viewing experience, which is consistent with other UHDTV characteristics, such as high resolution.  A motivation for our proposed support for color gamut scalability is this difference between color gamuts of base and enhancement layers when complying with the ITU-R standards.

With the scalable extension design beginning within JCT-VC, it is important to completely address the requirements of future applications that require scalability.  As captured in previous documents, a primary motivation for the spatial extension work in JCT-VC is to support UHDTV spatial resolutions with HD resolution base layers.  Moreover, the project requirements document captures that an enhancement layer and a base layer in the developing system should support different color gamuts. An important point is that common consumer applications using BT.709 have an 8-bit sample depth while BT.2020 only defines 10-bit and 12-bit sample depths.  Bit-depth scalability thus becomes an essential part of supporting color gamut scalability between BT.709 and BT.2020.  Previously we presented technique for achieving color gamut scalable video coding based on extensions of the HEVC design [3] and on the SMuC design [4].  Recently, a draft design of the SHVC codec was produced [5] as well as a test model [6].  Here, we propose a system to support color gamut scalability to achieve these goals using the SHVC design.

This document proposes:

1.  A new bit depth scaling process for reference pictures to support color gamut scalable coding.

2. It additionally proposes signaling color gamut and bit depth information regarding each layer in VPS extension to support session negotiation allowing end devices to select layers to decode based on their bit depth and color support capability.

# Proposal

We describe the two proposals in this section.

Proposal 1.  A new bit depth scaling process for reference pictures to support color gamut scalable coding is proposed.

Proposal 2: Signaling color gamut and bit depth information regarding each layer in VPS extension is proposed.

Proposal 1:

A new bit depth scaling process for reference pictures to support color gamut scalable coding is proposed.

The proposed changes are highlighted below compared to JCTVC-L1007_v3 [6]. These changes are defined in Annex G (Reference index based scalable extension) but also apply to Annex H (Texture based scalable extension) where they are proposed to be invoked in the similar manner after invoking H.8.8.3 (which invokes G.8.1.4.3) and after invoking H.8.8.4 (which invokes G.8.1.4.4).

- G.8 Decoding process

- G.8.1 General decoding process

The specifications of subclause F.8.1 apply.

- G.8.1.1 Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause G.8.1.2 and G.8.3.4 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Prior to decoding the first slice of the current picture, subclause  G.8.1.2 is invoked.

–	At the beginning of the decoding process for each P or B slice, the marking process for reference picture lists construction specified in subclause G.8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause  is invoked.

- G.8.1.2 Marking process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ currLayerId ] ]; i++ ) {
	rlPic = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ currLayerId ][ i ] ]
	RefPicSetInterLayer[ i ] = rsbPic as derived in the clause G.8.1.4 followed by clause G.8.1.5 with rlPic given as input
	rsPic is marked as "used for long-term reference"
}

- G.8.1.3 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

- G.8.1.4 Resampling process for inter layer reference pictures

Input to this process is

- A cropped decoded reference layer picture rlPic.

Output of this process is

–	The resampled reference layer picture rsPic.

The variables ScaledW and ScaledH are set equal to the width and height of the current picture in units of luma samples, respectively. The variable rsPicSample specifies the resampled sample values of rsPic of the luma and chroma components. The variable rsPicMotion specifies the resampled motion field of rsPic.

The variables PicWRL and PicHRL are set equal to the width and height of the cropped decoded reference layer picture rlPic in units of luma samples, respectively. The variable rlPicSample specifies the sample values of rlPic of the luma and chroma components. The variable rlPicMotion specifies the motion field of rlPic.

[Ed. (JL): The variables ScaledW, ScaledH, PicWRL and PicHRL are tentatively defined in this subclause. The term and value of those variables may need further consideration if the scaled reference layer offset of avc-svc or a similar scheme is adopted in SHVC ]

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if ScaledW is equal to PicWRL and ScaledH is equal to PicHRL

  - rsPicSample is set to rlPicSample,

  - When sps_inter_layer_mfm_enable_flag is equal to 1, rsPicMotion is set to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.8.1.4.1 is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - When sps_inter_layer_mfm_enable_flag is equal to 1, the picture motion field resampling process as specified in subclause G.8.1.4.2 is invoked with the compressed motion field of rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

- The bit depth scaling process in the clause G.8.1.5 is invoked after the resampling process for inter layer reference pictures.

…

- G.8.1.5 Bit depth scaling process for reference pictures

This process is invoked after the resampling process in the clause G.8.1.4 for inter layer reference pictures.

Input to this process is

- The resampled inter layer reference picture rsPic.

Output of this process is

–	The resampled bit depth scaled inter layer reference picture rsbPic.

The variables nBdbY and nBdbC specify the bit depth of the samples of the luma array and bit depth of the samples of the chroma array of the current picture.

The variables nBdY and nBdC specify the bit depth of the samples of the luma array and bit depth of the samples of the chroma array of the resampled reference layer picture rsPic.

The following steps are applied to derive the resampled bit depth scaled inter layer reference picture rsbPic with bit depth scaling.

–	if nBdY is equal to nBdbY and nBdC is equal to nBdbC

–	rsbPic is set to rsPic,

- otherwise rsPic is derived by follows:

–   The bitdepth scaling process as specified in subclause G8.1.5.1 is invoked with the resampled sample values of rsPicSample as input, and with the resampled bit depth scaled sample values of  rsbPicSample as output.

- G.8.1.5.1 Bit depth scaling process for picture sample values

Inputs of this process are:

–	a (ScaledW)x(ScaledH) array rsPicSampleL of luma samples with bit depth nBdY,

–	a (ScaledW/2)x(ScaledH/2) array rsPicSampleCb of chroma samples of the component Cb with bit depth nBdC,

–	a (ScaledW/2)x(ScaledH/2) array rsPicSampleCr of chroma samples of the component Cr with bit depth nBdC.

Outputs of this process are:

–	a (ScaledW)x(ScaledH) array rsbPicSampleL of luma samples with bit depth nBdbY,

–	a (ScaledW/2)x(ScaledH/2) array rsbPicSampleCb of chroma samples of the component Cb with bit depth nBdbC,

–	a (ScaledW/2)x(ScaledH/2) array rsbPicSampleCr of chroma samples of the component Cr with bit depth nBdbC.

For each luma sample location ( xP = 0.. ScaledW -1, yP = 0.. ScaledH-1) inside the luma sample array rsPicSampleL, the corresponding luma sample value is derived as follows:

rsbPicSampleL[ xP, yP ] = rsPicSampleL[ xP, yP ] << (nBdbY- nBdY)

For each chroma sample location ( xP = 0.. ScaledW /2-1, yP = 0.. ScaledH /2-1) inside the chroma sample array for the component Cb rsPicSampleCb, the corresponding chroma sample value is derived as follows:

rsbPicSampleCb[ xP, yP ] = rsPicSampleCb[ xP, yP ] << (nBdbC- nBdC)

For each chroma sample location ( xP = 0.. ScaledW/2-1, yP = 0.. ScaledH/2-1) inside the chroma sample array for the component Cr rsPicSampleCr, the corresponding chroma sample value is derived as follows:

rsbPicSampleCr[ xP, yP ] = rsPicSampleCr[ xP, yP ] << (nBdbC- nBdC)

Proposal 2:

The information about bit depth of luma and chroma components of each layer and about chromaticity coordinates of the source primaries of each layer can be useful for session negotiation in allowing end devices to select layers to decode based on their bit depth and color support capability.

This document proposes signaling color gamut scalability and bit depth information regarding layers in VPS extension.

The proposed changes are highlighted below compared to JCTVC-L1008 draft text specification [5].

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j <NumScalabilityTypes; j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| bitdepth_colorgamut_info( i ) |  |
| } |  |

| bitdepth_colorgamut_info( id ){ |  |
| --- | --- |
| bit_depth_layer_luma_minus8[ id ] | ue(v) |
| bit_depth_layer_chroma_minus8[ id ] | ue(v) |
| layer_color_gamut[ id ] | u(1) |
| } |  |

bit_depth_layer_luma_minus8[ id ] + 8 specifies the bit depth of the samples of the luma array for the layer with layer id id, as specified by

BitDepthLY [ id ]     = 8 + bit_depth_layer_luma_minus8[ id ]

bit_depth_layer_luma_minus8[ id ] shall be in the range of 0 to 6, inclusive.

bit_depth_layer_chroma_minus8[ id ] + 8 specifies the bit depth of the samples of the chroma arrays for the layer with layer id id, as specified by

BitDepthLC  [ id ]    = 8 + bit_depth_layer_chroma_minus8[ id ]

bit_depth_layer_chroma_minus8[ id ] shall be in the range of 0 to 6, inclusive.

layer_color_gamut[ id ]  equal to 1 specifies that the chromaticity coordinates of the source primaries for layer id are  defined as per Rec. ITU-R BT.2020. layer_color_gamut[ id ]  equal to 0 specifies that the chromaticity coordinates of the source primaries for layer id are defined as per Rec. ITU-R BT.709.

- In a variant signaling approach separate bit depth may be signaled for chroma component Cb and Cr.

- In another variant signaling approach the color primaries other than BT.709 and BT.2020 may be signaled. For example a syntax element similar to colour_primaries syntax element signaled in HEVC VUI [7] could be signaled for each layer to indicate its color primary.

# Conclusions

This document proposes:

1.  A new bit depth scaling process for reference pictures to support color gamut scalable coding.

2.  Signaling color gamut scalability and bit depth information regarding layers in VPS extension.

We propose to adopt the proposed changes in SHVC and MV-HEVC.

# Patent rights declaration

Sharp may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- ITU-R Recommendation BT.709 “Parameter values for the HDTV standards for production and international programme exchange” Dec. 2010.

- ITU-R Recommendation BT.2020 “Parameter values for UHDTV systems for production and international programme exchange” April 2012.

- L. Kerofsky, A. Segall, S. Kim, K. Misra, “Color Gamut Scalable Video Coding”,JCTVC-K-0241, Shanghai, October 2012.

- L. Kerofsky, A. Segall, S. Kim, K. Misra, “Color Gamut Scalable Video Coding: New Results”, JCTVC-L-0334, Geneva, January 2013.

- J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Draft Text 1,” JCTVC-L1008, Geneva, January 2013

- J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, “SHVC Test Model 1 (SHM 1),” JCTVC-L1007_v3, Geneva, January 2013

- B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” JCTVC-L1003, Geneva, January 2013.