| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0215 |
| --- | --- |

| Title: | Non-SCE4: Adaptive up-sampling of base layer picture using Simplified Separable bilateral filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Kiran Misra, and Andrew Segall 5700 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1-360-817-8451 jzhao@sharplabs.com, misrak@sharplabs.com, asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes the use of non-linear and content adaptive “bilateral” filters for inter-layer prediction.  In previous proposals, the use of a bilateral filter has shown improvements in coding efficiency – especially as the sampling ratio between layers increases.  Here, a low complexity alternative is proposed that integrates a separable bi-lateral filter directly into the upsampling process.  As a result, the upsampling and filtering operation can be performed at the same time and without additional buffering.  There is no increase in memory bandwidth or on-chip memory, as opposed to sequentially applying the upsampling and filtering stages.  Moreover, the complexity of the method is asserted to be low – with a worst case of an additional 9 multiplications and 15 additions per pixel.  Results are reported following SCE4 test conditions and show EL+BL rate improvements compared to SHM1.0 anchors of: -0.9% (AI 2x), -0.2% (AI 1.5x), -0.8% (RA 2x), 0.0% (RA 1.5x), -1.0% (LD-P 2x), -0.3 (LD-P 1.5x). The proposed bilateral filtering is not applied to SNR scalability.

# Introduction

We propose a non-linear and content adaptive upsampling process using SHM-1.0 and the INTRA_BL framework. Here, when the base layer picture is used for reference, i.e. INTRA_BL mode is ON, multiple versions of the upsampled base layer pictures are made available for inter-layer prediction. The first up-sampled base layer corresponds to the fixed up-sampler of SHM-1.0, while a second up-sampled version is generated by combined upsampling and a separable bilateral filter.

The motivation of this proposal is to report an alternative complexity/performance operating point of our original SCE4.2.6 proposal [1].  The proposal in [1] uses a 2D bi-lateral filter that is applied after the upsampling operation is complete.  Here, we report a lower complexity and lower memory bandwidth configuration.  The rest of the proposal is organized as follows:  In Section 2, we provide background on the bi-lateral filter.  Section 3 describes our proposed filtering process that integrates the bi-lateral and upsampling filters.  Section 4 provides syntax and semantics.  Results appear in Section 5.

# Bilateral Filter

A bilateral filter is a non-linear filter [2].  It reduces the noise in the upsampled base layer while preserving edges. We assert that this is beneficial for spatial scalability, as it removes low amplitude and high frequency content that is normally associated with noise, for e.g. due to quantization, loop filtering, while preserving high amplitude and high frequency information corresponding to edges and structure.  This is achieved by replacing the intensity value at each pixel by a weighted average of intensity values from nearby pixels within a window. The weight is based on a Gaussian distribution and relative pixel intensities.

# For a pixel at location (x,y), the bilateral filtered output is then expressed as:

Ox,y=f(a,b,Ia,b,Ix,y)∗Ia,bf(a,b,Ia,b,Ix,y)	(1)

where the summation is performed on pixels within a supporting window, Ia,b denotes the input pixel value at location (a,b), and f(a,b, Ia,b, Ix,y ) is a function that maps the spatial distance and relative pixel intensity information at a location (a,b) to a weight value.

The design of the function f() provides the desirable non-linear and content adaptive properties.  Traditionally, the function consists of two parts – a spatial kernel and a gradient kernel – and is expressed as:

f(a,b,Ia,b,Ix,y)=S(a,b)∗e−(G(Ia,b,Ix,y)22∗σR2)	(2)

where S(a,b) denotes a two-dimensional Gaussian kernel with standard deviation σS, G(a,b,Ix,y) denotes the absolute difference between Ia,b, and Ix,y, and σR is a control parameter. In general, the larger difference between the two pixel values, the lower the weight value.  This preserves edges and avoids over smoothing.    We advocate using simple differences to calculate the gradient.

# Combined Bilateral Filter with Upsampler

We propose an approximation of the upsampling and bi-lateral processes with significantly fewer multiplies and additions when compared to the sequential application of an upsampler and traditional two-dimensional bi-lateral filter.  In addition, the approximation does not require additional line buffers.  As a first step in the approximation, we embed the bi-lateral filter into the upsampling framework.  This is achieved by considering a separable operation for both the interpolation and content adaptive components.  We note that in most applications, a separable bi-lateral filter does not perform well.  For example, it may perform poorly on diagonal structures.  However, for this application, we report that embedding the operation into an upsampling process mitigates this trait.

While a separable bi-lateral filter begins to reduce the number of adds and multiplies in the system, it does not address the need for additional line buffers.  To solve this, we remove all dependencies on neighboring pixels that are upsampled, using only integer-pel pixels in the content adaptive process. This allows the bi-lateral filtering to be done in place in the same pass as the upsampling of a pixel. Thus, our new filter is expressed as

Output(Ai,j)=f(Ai−1,j)∗Ai−1,j+Ai,j+f(Ai+1,j)∗Ai+1,jf(Ai−1,j)+1+f(Ai+1,j)

Output(bi,j)=f(Ai,j)∗Ai,j+bi,j+f(Ai+1,j)∗Ai+1,jf(Ai,j)+1+f(Ai+1,j)

where Ai,j is an integer-pel pixel and bi,j denotes a fractional pel pixel.  As a key novelty, the calculation of the integer pel pixels only uses information from other integer pel locations, and so does not require interpolation of neighboring values.  Similarly, the calculation of the fractional pel pixel values use only the interpolated pixel value at the current location.  All other pixel values are integer pel.  By restricting the calculation to use only integer pel pixel values (and a co-located fractional pel value), the need for additional line buffers is removed.  Moreover, the number of additions and multiplies are diminished, as the number of pixels corresponding to a window size in the enhancement layer is reduced.

A block diagram of the proposed filter is shown in Figure 1.  As can be seen in the figure, the bi-lateral operation is now a separable operation that is performed in-place with the interpolation.  Shaded blocks denote the non-linear and content adaptive components.

Fig. 1. Modified 1-D upsampling with BLT filtering.

# Syntax

The modified upsampling process is signalled at CU level together with INTRA_BL flag.  Example syntax tables are shown in the tables below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| if( getLayerId() > 0 ) { |  |
| bilateral_filter_present_flag | u(1) |
| } |  |
| num_extra_slice_header_bits | u(3) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| } |  |
| if( getLayerId() > 0 && bilateral_filter_present_flag) { |  |
| most_probable_upsampler_idx | u(1) |
| } |  |
| ………………… |  |
| byte_alignment( ) |  |
| } |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( getLayerId() > 0 ) { |  |
| cu_intra_bl_flag[ x0 ][ y0 ] | u(1) |
| if(cu_ intra_bl_flag[ x0 ][ y0 ] && bilateral_filter_present_flag ) { |  |
| cu_mapped_upsampler_idx[ x0 ][ y0 ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

bilateral_filter_present_flag specifies the presense of the cu_mapped_upsampler_idx syntax elment in the slice.  When the syntax element is not present in the bit-stream it shall be inferred to be zero.

cu_mapped_upsampler_idx[ x0 ][ y0 ] specify the mapped up-sampler index. The array indices  x0 , y0  specify the location ( x0 , y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The up-sampler index is obtained using:

cu_upsampler_idx[ x0 ][ y0 ] = cu_mapped_upsampler_idx[ x0 ][ y0 ]
if( cu_mapped_upsampler_idx[ x0 ][ y0 ] = = 0 ) {
	cu_upsampler_idx[ x0 ][ y0 ] = most_probable_upsampler_idx
} else if(cu_mapped_upsampler_idx[ x0 ][ y0 ] <= most_probable_upsampler_idx ) {
	cu_upsampler_idx[ x0 ][ y0 ]--
}

cu_upsampler_idx[ x0 ][ y0 ]  equals 0 corresponds to the fixed upsampler. cu_upsampler_idx[ x0 ][ y0 ]  equals 1 corresponds to the upsampler combined with bilateral filter.

# Result

We evaluated the proposed method in SHM-1.0.  Here, we report results using SCE4 and SHM common test condition [3] [4] relative to SHM-1.0 anchor.  For implementation, we realize the function f() with the lookup table [16, 15, 12, 9, 6, 4, 2, 1 0] and with the index to the table defined as min(gradient>>2, 8).  Here, “gradient” defines the difference between two pixel values, “>>” is a right shift operation, and min(x,8) returns the minimum value of x or 8.  The support of the bi-lateral filter is restricted to a 3x1 kernel, which allows further reduction of the complexity.  Specifically, we are able to replace the division in () with a reciprocal multiplication, where the reciprocal is stored in a LUT.

In our implementation, we modified upsampleBasePic to generate two upsampled base layers.  The first base layer corresponds to using the existing SHM upsampler.  The second base layer corresponds to the process proposed in this document.  Additionally, in the encoder, the upsampler is selected at a CU level in the function checkRDCostIntraBL based on R-D cost.

As can be seen from Table 1, the method provides improved performance at higher spatial scalability ratios.  Specifically, we observe an overall (EL+BL) bit-rate reduction of  -0.9%, -0.8% and  -1.0% for AI, RA and LD-P 2X test conditions.  This is equivalent to an EL bit-rate reduction of -1.1%, -1.1% and -1.3% for AI, RA and LD-P 2X, respectively.  Bit-rate reductions for 1.5xtest conditions are also observed and in the range of 0% to -.3% (EL+BL).

Table 1. Result of Proposed Adaptive Upsampling Method vs. SHM1.0 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.4% | -1.1% | -1.3% |  |  |  |
| Class B | -0.7% | -0.2% | -0.2% | -0.2% | 0.2% | 0.2% |
| Overall (Test vs Ref) | -0.9% | -0.4% | -0.5% | -0.2% | 0.2% | 0.2% |
| Overall (Test vs single layer) | 11.4% | 13.2% | 12.9% | 10.1% | 10.5% | 9.9% |
| EL only (Test vs Ref) | -1.1% | -0.6% | -0.7% | 0.8% | 1.3% | 1.3% |
| Enc Time[%] | 121.5% |  |  | 116.2% |  |  |
| Dec Time[%] | 151.0% |  |  | 147.4% |  |  |
| Enc Mem[%] | 102.7% |  |  | 102.1% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | 0.0% | -0.8% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.6% | 0.3% | 0.3% | 0.0% | 0.5% | 0.4% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.8% | 0.2% | 0.0% | 0.0% | 0.5% | 0.4% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 18.4% | 32.4% | 33.1% | 16.5% | 29.2% | 31.4% | 14.9% | 28.9% | 32.7% |
| EL only (Test vs Ref) | -1.1% | 0.0% | -0.2% | 1.2% | 1.7% | 1.6% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105.6% |  |  | 103.4% |  |  | 98.7% |  |  |
| Dec Time[%] | 178.0% |  |  | 177.8% |  |  | 97.3% |  |  |
| Enc Mem[%] | 101.0% |  |  | 101.0% |  |  | 100.7% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -0.6% | -1.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | -0.8% | -0.1% | -0.1% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -1.0% | -0.2% | -0.3% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 24.9% | 36.3% | 37.8% | 22.2% | 33.2% | 36.1% | 22.3% | 34.4% | 39.1% |
| EL only (Test vs Ref) | -1.3% | -0.5% | -0.6% | 1.0% | 1.3% | 1.4% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103.9% |  |  | 103.0% |  |  | 102.6% |  |  |
| Dec Time[%] | 174.0% |  |  | 174.6% |  |  | 103.2% |  |  |
| Enc Mem[%] | 101.0% |  |  | 101.2% |  |  | 101.0% |  |  |

# Complexity Assessment

We also report the complexity of the method.  As mentioned above, we propose a window size of 3x1 and the use of a reciprocal LUT for performing the division.  The operation then becomes:

Output(Ai,j)=(f(Ai−1,j)∗Ai−1,j+Ai,j+f(Ai+1,j)∗Ai+1,j)∗LUT[f(Ai−1,j)+f(Ai+1,j)]		(3)

Output(bi,j)=(f(Ai,j)∗Ai,j+bi,j+f(Ai+1,j)∗Ai+1,j)∗LUT[f(Ai,j)+f(Ai+1,j)]		(4)

Thus, the process needs 3 multiplication, and 5 additions (including 2 to calculate the gradient) to compute an output value.  Since the operation is separable and applied to both luma and chroma, the worst-case complexity is 9 multiplication and 15 addition for each sample.  To be clear, this is in addition to the existing complexity of the upsampler (37 multiplies and 32 additions per sample).

Dynamic range analysis shows that for an 8-bit input, the combined upsampling and bilateral filtering will result in a max 31-bit accumulator when using reciprocal look up, 15 bit intermediate output and 8 bit final output. All numbers are within the allowed dynamic ranges.

The proposed method does not use combined prediction mode.

The summary complexity analysis table below lists the average number of memory accesses, multiplication and add computed. For detail, please refer to the complexity analysis excel sheet.

# Conclusions

In this document, a non-linear and content adaptive bilateral filtering process is proposed.  Average bit-rate reductions (BL+EL) of -0.9%, -0.8% and -1.0% are observed for AI, RA and LDP 2X conditions relative to SHM-1.0 anchors for the S-HEVC test conditions respectively. The proposed tool is combined into  the upsampling process with very small computation increase, and no memory bandwidth change.

# References

- J. Zhao, K. Misra, A. Segall, “SCE4.2.6: Adaptive up-sampling of base layer picture using bilateral filters,” JCTVC-Mxxx, Geneva, CH, Apr. 2013

- C. Tomasi and R. Manduchi, “Bilateral filtering for gray and color images,” IEEE International Conference on Computer Vision, pp.839-846, 1998.

- J. Chen, A. Segall et al, “Description of HEVC Scalable Extensions Core Experiment 4 (SCE4): Inter-layer filtering,” JCTVC-L1104, Geneva, CH, Jan. 2013

- X. Li, J. Boyce et al, “Common SHM test conditions and software reference configurations,” JCTVC-L1109, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).