| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0217 |
| --- | --- |

| Title: | SCE2: Results of test 1.2 and test 1.3 on inter-layer texture prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Since inter-layer texture prediction (ILTP) is signaled at CU level in SHVC Test Model 1 (SHM 1), ILTP cannot be used for PU smaller than 2Nx2N. In SCE2 Test 1.2, a modified merge process is proposed for PU smaller than 2Nx2N to support ILTP for any PU size in Inter slice. To be specific, after checking the availability of the first four merge candidates (i.e. collocated BL, left spatial, upper spatial and upper-right spatial), if there is at least one unavailable candidate, ILTP mode is added in the merge candidate after them. The inter-layer texture prediction gets enabled for a PU when the derived merge_idx specifies ILTP mode. Experimental results with common test condition reportedly show that, when compared to the SHM-1.0 anchor, the proposed method results in luma BL+EL BD-rate gain of RA 2x: 0.2%, RA 1.5x: 0.2%, RA SNR: 0.1%, LD-P 2x: 0.4%, LD-P 1.5x: 0.4%, LD-P SNR: 0.3%, LD-B 2x: 0.3%, LD-B 1.5x: 0.3% and LD-B SNR: 0.2%.

SCE2 Test 1.3 is a combination of SCE2 Test 1.2 and skipped ILTP signaling method in SCE2 Test 1.1. Test 1.3 experimental results reportedly show that luma BL+EL BD-rate is RA 2x: 0.2%, RA 1.5x: 0.2%, RA SNR: 0.2%, LD-P 2x: 0.5%, LD-P 1.5x: 0.6%, LD-P SNR: 0.4%, LD-B 2x: 0.4%, LD-B 1.5x: 0.6% and LD-B SNR: 0.3%.

# Introduction

In this contribution, the modified merge process is proposed for smaller PU than 2Nx2N to support inter-layer texture prediction (ILTP). When PU size is not 2Nx2N, ILTP mode is added into the merge candidate list if a particular condition is met. By the modified merge process, ILTP can be enabled at PU level using merge_idx without an additional flag. In this contribution, this proposed method is also combined with the skipped ILTP signaling method in SCE2 test 1.1 [1].

# Proposed method

Test 1.2: Modified merge process for PU-level inter-layer texture prediction

The benefit of ILTP is not fully exploited in SHM 1 since ILTP can be enabled only for 2Nx2N prediction block. In some regions, the ILTP for non-square PU or NxN PU can provide additional coding gain especially in Inter slice. Therefore, it is proposed to modify the merge process in Inter slice to support ILTP for smaller PU than 2Nx2N. The proposed method is based on our previous work in [2].

The modified merge process checks the availability of the first four merge candidates, i.e. collocated BL, left spatial, upper spatial and upper-right spatial, and add each of them into the merge candidate list if available. Then, if there is at least one unavailable candidate, ILTP mode is added in the merge candidate. It is descried in Figures 1 and 2. For ILTP mode, refIdxLx is set to -1, predFlagLx is set to 0 for x = 0 and 1. When the derived refIdxLx for x = 0 and 1 by merge_idx are equal to -1, a PU is predicted from the collocated reference-layer reconstruction. Note that the modified merge process is applied to PU whose size is not 2Nx2N.

Figure 1. Modified merge candidate list derivation    Figure 2. Modified PU merge decoding process

Test 1.3: Combined with test 1.1, skipped ILTP signaling [1]

The proposed modified merge process is combined with test 1.1, the skipped ILTP signaling method [1]. It should be noted that both methods improves the ILTP signaling without any new syntax elements as shown by experiments.

# Experimental results

# Test 1.2: Modified merge process for PU-level inter-layer texture prediction

# In this test, the proposed modified merge process is compared with SHM-1.0 anchor under the common test conditions [3]. The proposed method resulted in luma BL+EL BD-rate gain of 0.2%, 0.2%, 0.1%, 0.4%, 0.4%, 0.3%, 0.3%, 0.3% and 0.2% for RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x, LD-P SNR, LD-B 2x, LD-B 1.5x and LD-B SNR, respectively. It is summarized in Table 2.

Table 2. BD-rate result of the modified merge process compared to SHM-1.0 anchor.

Test 1.3: Combined test with test 1.1, skipped ILTP signaling [1]

# In this test, the proposed modified merge process and the skipped ILTP signaling method in [1] are enabled and compared with SHM-1.0 anchor. This combined method resulted in luma BL+EL BD-rate gain of 0.2%, 0.2%, 0.2%, 0.5%, 0.6%, 0.4%, 0.4%, 0.6% and 0.3% for RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x, LD-P SNR, LD-B 2x, LD-B 1.5x and LD-B SNR, respectively. It is summarized in Table 3.

Table 3. BD-rate result of the combined method compared to SHM-1.0 anchor.

# Conclusion

In this contribution, the modified merge process is proposed for smaller PU than 2Nx2N to support the inter-layer texture prediction at PU level. And this method is combined with the skipped ILTP signaling method proposed in [1]. It turned out that both methods improved coding efficiency of Inter slice without any new syntax elements. The modified merge process improved luma BL+EL BD-rate up to 0.4%. When combined with the skipped ILTP signaling method, luma BL + EL BD-rate was improved up to 0.6%.

# Test Model text changes

Attached.

# Reference

- D.-K. Kwon and M. Budagavi, “SCE2: Results of test 1.1 on skipped inter-layer texture prediction (ILTP) block signaling in Inter slice,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0216, Apr. 2013.

- D.-K. Kwon, M. Budagavi and M. Zhou, “Non-TE2: Improved inter-layer texture prediction signaling,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0230, Jan. 2013.

- X. Li, J. Boyce, P. Onno and Y. Ye, “Common test condition and software reference configurations for the scalable test model,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1009, Jan. 2013

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).