| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L1007 |
| --- | --- |

| Title: | SHVC Test Model 1 (SHM 1) |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of SHVC Test Model Description |  |  |
| Author(s) or Contact(s): | Jianle Chen, Qualcomm Jill Boyce, Vidyo Yan Ye, InterDigital Miska M. Hannuksela, Nokia | Email: | cjianle@qti.qualcomm.com jill@vidyo.com Yan.Ye@interdigital.com miska.hannuksela@nokia.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document serves as a source of general tutorial information on SHVC and also provides a draft description of SHVC Test Model 1.

CONTENTS

Page

Abstract	i

1	Introduction	1

2	Description of SHVC Test Model	1

2.1	General overview of coding structure	1

2.2	Coding of the enhancement layer in reference index based SHVC	1

2.2.1	Inter layer texture prediction in reference index based SHVC	1

2.2.2	Inter layer motion prediction in reference index based SHVC	2

2.3	Coding of the enhancement layer in TextureRL based SHVC	2

2.3.1	Inter layer texture prediction in TextureRL based SHVC	2

2.3.2	Inter layer motion prediction in TextureRL based SHVC	2

2.4	Resampling process of picture samples	3

2.4.1	Downsampling process	3

2.4.2	Upsampling process	4

3	Software	5

3.1	Software repository	5

3.2	Build System	5

3.3	Software Structure	5

4	SHVC text specification draft	6

F.8.1	General decoding process	7

Annex G   Reference index based scalable extensions	8

G.1	Scope	8

G.2	Normative references	8

G.3	Definitions	8

G.4	Abbreviations	8

G.5	Conventions	8

G.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	8

G.7	Syntax and semantics	8

G.7.1	Method of specifying syntax in tabular form	8

G.7.2	Specification of syntax functions, categories, and descriptors	8

G.7.3	Syntax in tabular form	8

G.7.4	Semantics	11

G.8	Decoding process	13

G.8.1	General decoding process	13

G.8.2	NAL unit decoding process	19

G.8.3	Slice decoding process	19

G.8.4	Decoding process for coding units coded in intra prediction mode	20

G.8.5	Decoding process for coding units coded in inter prediction mode	20

G.8.6	Scaling, transformation and array construction process prior to deblocking filter process	20

G.8.7	In-loop filter process	20

G.9	Parsing process	20

G.10	Specification of bitstream subsets	21

G.11	(Void)	21

G.12	Byte stream format	21

G.13	Hypothetical reference decoder	21

G.14	SEI messages	21

G.15	Video usability information	21

Annex H   TextureRL based scalable extensions	22

H.1	Scope	22

H.2	Normative references	22

H.3	Definitions	22

H.4	Abbreviations	22

H.5	Conventions	22

H.6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	22

H.7	Syntax and semantics	22

H.7.1	Method of specifying syntax in tabular form	22

H.7.2	Specification of syntax functions, categories, and descriptors	22

H.7.3	Syntax in tabular form	22

H.7.4	Semantics	25

H.8	Decoding process	27

H.8.1	General decoding procss	27

H.8.2	(void)	27

H.8.3	(void)	27

H.8.4	Decoding process for coding units coded in intra prediction mode	27

H.8.5	Decoding process for coding units coded in inter prediction mode	27

H.8.6	Scaling, transformation and array construction process prior to deblocking filter process	34

H.8.7	In-loop filter process	35

H.8.8	Decoding process for coding units coded in textureRL prediction mode	35

H.9	Parsing process	37

H.9.2	CABAC parsing process for slice segment data	37

H.10	Specification of bitstream subsets	38

H.11	Profiles and levels	38

H.12	Byte stream format	38

H.13	Hypothetical reference decoder	38

H.14	SEI messages	38

H.15	Video usability information	38

# Introduction

The JCTVC established the first SHVC test model (SHM1) at its 12th meeting in Geneva from 14 – 23 January 2013. Two inter-layer texture prediction signalling mechanisms, namely the “reference index based” method and the “textureRL based” method, were tested. The two inter-layer texture signalling methods showed coding performance comparable to each other. It was decided to include both of them in SHM1. In addition, inter-layer motion prediction tools using the compressed base layer motion field were also adopted, respectively, into both SHVC solutions.

It was also decided that the SHM will support AVC base layer and only inter-layer texture prediction is used for this case.

# Description of SHVC Test Model

## General overview of coding structure

The design of HEVC naturally enables temporal scalability when a hierarchical temporal prediction structure is used. Hence the JCTVC currently concentrates on developing tools to support spatial and coarse grain SNR scalabilities in SHVC. A general block diagram of the SHVC encoder is depicted in Figure 2-1. Under multi-loop decoding framework, the pictures in different layers of the same time instance are coded into the bitstream in an ascending order of nuh_layer_id. The original input pictures are downsampled and coded into the base layer bitstream by using a conformant HEVC or AVC codec. For the enhancement layer coding, additional inter-layer prediction tools, which employ already coded data (including reconstructed picture samples and motion parameters) in reference layer for efficiently coding an enhancement layer, are integrated to SHVC codec. The newly integrated tools are described in the following sub-sections.

Figure 2-1 High-level block diagram of the SHVC encoder.

## Coding of the enhancement layer in reference index based SHVC

### Inter layer texture prediction in reference index based SHVC

In reference index based SHVC solution, the block level syntax and decoding process are not changed for supporting inter-layer texture prediction. Only the high-level syntax has been modified so that the reconstructed pictures (upsampled if necessary) from the reference layers having the same POC value as that of the current picture can be used as the reference pictures for coding the current enhancement layer picture. The inter-layer reference pictures as well as the temporal reference pictures are included in the reference picture lists. The signaled reference picture index is used to indicate whether the current Prediction Unit (PU) is predicted from a temporal reference picture or an inter-layer reference picture.

The reference lists in SHM1 are constructed as follows. The temporal references are first added into the reference lists (L0, L1) in the same manner as the reference list construction in HEVC. After that, the inter-layer reference is added at the end of the lists as a long term reference picture. The inter-layer reference is added to reference picture list L0 if the current enhancement-layer picture is coded as P-Slice, and is added to both reference picture lists L0 and L1 if the current enhancement-layer picture is coded as B-Slice.

### Inter layer motion prediction in reference index based SHVC

This tool is disabled when the reference layer is coded by an AVC codec.

In reference index based SHVC solution, inter-layer motion parameter prediction is performed by setting the inter-layer reference picture as the collocated reference picture for TMVP derivation. In the case of spatial scalability, motion field mapping process between two layers is performed;  no additional  block level decoding process modification to TMVP derivation is applied. The syntax  sps_inter_layer_mfm_enable_flag in the SPS extension signals the use of this tool.

In motion field mapping process, the motion field of the upsampled inter-layer reference picture is obtained based on the compressed motion field of the lower resolution reference layer picture. The motion parameters (including MV and reference index) and prediction mode for each 16×16 block of the upsampled inter-layer reference picture are derived from the corresponding motion parameters and prediction mode of the collocated block in the reference layer picture. The 16×16 block size is chosen to be compliant with HEVC TMVP derivation process where compressed motion field of reference picture is used.

## Coding of the enhancement layer in TextureRL based SHVC

### Inter layer texture prediction in TextureRL based SHVC

In TextureRL based SHVC solution, the inter-layer texture prediction is performed at the CU level and a new prediction mode, named as textureRL mode, is introduced. The collocated upsampled base layer block is used as the prediction for the enhancement layer CU coded in textureRL mode. For an input CU of the enhancement layer encoder, the best CU mode is determined among intra, inter and textureRL modes. Beside the RD selection procedure used in unmodified HEVC encoder, the RD cost of textureRL mode is calculated additionally for the enhancement layer CUs.A CU level flag (texture_rl_flag) is used to indicate usage for the current CU.

The residue of textureRL predicted CU is coded as follows. Transform process of a textureRL predicted CU is the same as an intra predicted CU, where discrete sine transform (DST) is applied to TU of luma component having 4×4 size and discrete cosine transform (DCT) is applied to the other type of TUs. Transformed coefficients coding of textureRL predicted CU is the same as that of inter predicted CU, where no_residue_flag is used to indicate whether the coefficients of the whole CU are skipped.

### Inter layer motion prediction in TextureRL based SHVC

This tool is disabled when the reference layer pictures are coded by an AVC codec.

In HEVC, two motion parameters prediction mechanisms, namely “merge mode” and “advanced motion vector prediction (AMVP) mode” are used for MV coding. In merge mode, the motion parameters (motion vector, reference picture index) of the current PU is inherited from the neighboring blocks without coding the motion vector difference. The HEVC encoder can select the best motion vector candidate from multiple candidates formed by spatially neighbouring PUs and temporally neighbouring PUs, and transmit the corresponding index indicating the chosen candidate.

As depicted in Figure 2-2, in addition to spatially and temporally neighboring PUs, the motion parameters of the collocated base-layer block is also used to form the merge candidate list. The base layer merge candidate is derived at a location colocated to the central position of the current PU and inserted as the first candidate in merge list. In the case of spatial scalability, the base-layer MV is scaled according to the spatial resolution ratio between the two layers. The pruning (duplicated candidates check) is performed for each spatially neighboring candidate with collocated base layer candidate. For the collocated base layer merge candidate and spatial merge candidate derivation, a maximum of four merge candidates are selected among candidates that are located in six different positions. The temporal merge candidate is derived in the same manner as done for HEVC merge list. When the number of candidates does not reach to maximum number of merge candidate (MaxNumMergeCand), the additional candidates, including combined bi-predictive candidates and zero merge candidates, are generated and added at the end of the merge list, which is the same as HEVC merge list construction. The allowed value of MaxNumMergeCand for SHVC is from 1 to 5, inclusively, which is the same as the one for HEVC. In the current SHVC common test condition, MaxNumMergeCand is set equal to five.

Figure 2-2 Merge candidate derivation for in SHVC.

## Resampling process of picture samples

### Downsampling process

This section introduces the downsampling process used to generate the base layer input video sequences from the enhancement layer input video sequences used in the SHVC common test conditions for 1.5x and 2x spatial scalability.

Figure 2-3 (a) and (b) show the relative sampling grids of the enhancement layer picture (white positions) and the base layer picture (blue positions) for 2x and 1.5x spatial scalabilities, respectively. Both the enhancement layer and base layer video sequences are assumed to be in YUV4:2:0 format.

(a)   2x ratio                                           						         (b) 1.5x ratio

Figure 2-3 Relative sampling grids between the enhancement layer picture and the base layer picture for 2x and 1.5x ratio. White squares: EL luma; white triangules: EL chroma; blue circles: BL luma; blue triangules: BL chroma.

12-tap, 2D separable downsampling filters are applied to the enhancement layer pictures. The downsampling filter coefficients for 1.5x downsampling ratio and for 2x downsampling ratio are shown in Table 2-1 and Table 2-2, respectively. The same downsampling filters are applied to luma and to chroma. For 1.5x ratio, 8-phase filter is used; 4 of the 8 phases are not used by the 1.5x downsampling process, therefore their filter coefficients are not shown in Table 2-1. For 2x ratio, 4-phase filter is used; 2 of the 4 phases are not used by the 2x downsampling process, therefore their filter coefficients are not shown in Table 2-2.  For each sample location (x, y) in the base layer picture, the following is used to select the filter phase:

ph=0if 2x ratio(12⋅x)%8if 1.5x ratio	(2-xx)

pv=(8⋅y+offset)%4if 2x ratio(12⋅y+offset)%8if 1.5x ratio, where offset =0if luma1if chroma	(2-xx)

The filters with horizontal phase ph and vertical phase pv from Table 2-1 or Table 2-2 are applied, first in the horizontal dimension and then in the vertical dimension.

Additional detailed information on the downsampling filters, including the downsampling source code, can be found in the MPEG document, "Downsampling filter for anchor generation for scalable extensions of HEVC," MPEG document no. m24499, 100th MPEG meeting, Geneva, Switzerland, Apr. 30-May 4, 2012. [Ed. (YY): this can be moved to a dedicated reference section if we decide to establish one.]

Table 2-1 1.5x downsampling filters

| phase i | Downsampling filter coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f15[i,0] | f15[i,1] | f15[i,2] | f15[i,3] | f15[i,4] | f15[i,5] | f15[i,6] | f15[i,7] | f15[i,8] | f15[i,9] | f15[i,10] | f15[i,11] |
| 0 | 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0 |
| 1 | -1 | 5 | -3 | -12 | 29 | 75 | 45 | -7 | -8 | 5 | 0 | 0 |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 4 | -1 | 3 | 2 | -13 | 8 | 65 | 65 | 8 | -13 | 2 | 3 | -1 |
| 5 | -1 | 2 | 3 | -12 | 2 | 59 | 70 | 14 | -13 | 1 | 4 | -1 |
| 6 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 7 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |

Table 2-2 2x downsampling filters

| phase i | Downsampling filter coefficients |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f20[i,0] | f20[i,1] | f20[i,2] | f20[i,3] | f20[i,4] | f20[i,5] | f20[i,6] | f20[i,7] | f20[i,8] | f20[i,9] | f20[i,10] | f20[i,11] |
| 0 | 2 | -3 | -9 | 6 | 39 | 58 | 39 | 6 | -9 | -3 | 2 | 0 |
| 1 | 1 | -1 | -8 | -1 | 31 | 57 | 47 | 13 | -7 | -5 | 1 | 0 |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |

### Upsampling process

This section introduces the upsampling process applied on the base layer reconstructed picture, in order to provide inter layer prediction for enhancement layer, when the base and enhancement layer pictures have different spatial resolutions.

Given the upsampling ratio N in both directions, the upsampling process is conceptually approximated by interpolating the base layer reconstructed picture to 16 times its size in both directions and decimating the 16x picture with the ratio M, where 16≈M×N. To perform 16x upsampling, 16-phase interpolation filters are used. Detailed filter coefficients are provided in Tables G-1 and G-2 for luma and for chroma, respectively. The current SHM only supports 1.5x and 2x spatial scalability, which means only a subset of the 16-phase interpolation filters are needed. The phase filters not used in either 2x or 1.5x upsampling process are not defined; instead, they are marked as “n/a” in Tables G-1 and G-2. The luma interpolation filtes are 8-tap, and the chroma interpolation filters are 4-tap. They are 2D separable filters.

In actual implementation, the 16x interpolation is not performed for every sample; instead, only those samples in the 16x picture that will be kept after decimation are interpolated. To generate a sample located as (x, y) in the enhancement layer picture, its corresponding position (x16, y16) in the virtual 16x picture before decimation is first found by using the method specified in G.7.1.3.1.5. Then, the sample value at (x16, y16) is interpolated by applying the appropriate phase filter to the support region from the base layer reconstructed picture. The process to determine the base layer support region and the appropriate phase filters are specified in G.7.1.3.1.3 and G.7.1.3.1.4 for luma and for chroma, respectively. Table 2-3 summarizes the phase filters used for 2x and 1.5x upsampling for luma and chroma in each of the horizontal and vertical directions.

Table 2-3 Phase filters used for 1.5x and 2x upsampling

| Ratio | Color Component & Direction | Applicable Phase |
| --- | --- | --- |
| 2x | Luma | 0, 8 |
|  | Chroma Horizontal | 0, 8 |
|  | Chroma Vertical | 6, 14 |
| 1.5x | Luma | 0, 5, 11 |
|  | Chroma Horizontal | 0, 5, 11 |
|  | Chroma Vertical | 4, 9, 15 |

# Software

## Software repository

The source code for the software is available in the following SVN repository.

https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/

For tool integration branch for a company can be obtained by contacting:

Seregin, Vadim  (vseregin@qti.qualcomm.com)

## Build System

The software can be built under linux using make. For Windows, solutions for different versions of Microsoft Visual Studio are provided.

## Software Structure

The SHVC Test Model Software has the same software structure to the currenet HM, which includes the following applications and libraries for encoding, decoding:

- Applications:

  - TAppEncoder, executable for bit stream generation

  - TAppDecoder, executable for reconstruction.

  - TAppCommon, common functions for configuration file parsing .

- Libraries:

  - TLibEncoder, encoding functionalities

  - TLibDecoder, decoding functionalities

  - TLibCommon, common functionalities

  - TLibVideoIO, video input/output functionalities

# SHVC text specification draft

The specifications made in this section are based on HEVC text specification draft 10 (JCTVC-L1003_v19) and SHVC working draft 1 (JCTVC-L1008) and MV-HEVC working draft 3 (JCT3V-C1004).

In text blocks copied from JCTVC-L1003_v19 changes are highlighted.

Modifications of HEVC specification:

In this Recommendation | International Standard Annexes A through H contain normative requirements and are an integral part of this Recommendation | International Standard.

Annex G specifies syntax, semantics, and decoding processes for reference index based scalable extension. The reader is referred to Annex G for the entire decoding process for reference index based scalable extension, which is specified there with references being made to clauses 1-9 and Annexes A-F.

Annex H specifies syntax, semantics, and decoding processes for TextureRL based scalable extension. The reader is referred to Annex H for the entire decoding process for TextureRL based scalable extension, which is specified there with references being made to clauses 1-9 and Annexes A-G.

[Ed. (All): This specification text is subject to be changed when a final version of HEVC v1 specification text (JCTVC-L1003) is available (expected 1 March 2013), when MV-HEVC specification text (JCT3V-C1004) has been updated to be based on JCTVC-L1003 (expected 22 March 2013) and when SHVC working draft text is available (expected 22 March 2013). For example, clause numbering and cross-references may have to be updated. An editors’ update of JCTVC-L1007 will be released after the MV-HEVC&SHVC text have been finalized.]

[Ed. (JL&JB): It is expected that the variables InterLayerRefIdxEnableFlag, InterLayerTextureRlEnableFlag and InterLayerMotionPredictionEnableFlag used in this version of  this specification will be replaced by syntax elements signaled in the high level syntax, in a location TBD]

Annex F of this specification is the same as the one in JCTVC-L1008 and JCT3V-C1004 except replacing subclause F8.1 with the following (changes are highlighted).

  - General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to the 3 sample arrays of the current picture with the NAL units with colour_plane_id equal to 0 being assigned to SL, the NAL units with colour_plane_id equal to 1 being assigned to SCb, and the NAL units with colour_plane_id equal to 2 being assigned to SCr.

NOTE – The variable ChromaArrayType is derived as 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures with chroma_format_idc being equal to 0.

–	The following ordered steps apply for the decoding of the current picture.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process specified in subclause Error: Reference source not found is invoked.

–	The following applies:

–	If InterLayerRefIdxEnableFlag is equal to 1 , the decoding process specified in G.7.1.1 is invoked.

–	Otherwise ( InterLayerTextureRlEnableFlag is equal to 1) the decoding process specified in H.7.1.1 is invoked

–	After all slices of the current picture have been decoded, the decoding process specified in subcaluse Error: Reference source not found is invoked.

- Annex G 

Reference index based scalable extensions

This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax, semantics and decoding processes for scalable coding extensions that use the syntax, semantics, and decoding process specified in clauses 2-9 for the slice segment data and all layers below it.

- Scope

Bitstreams and decoding processes conforming to this annex are completely specified in this annex with reference made to clauses 2-9 and Annexes A-F where occurrences of nuh_layer_id in clauses 2-9 and Annexes A-E is consistently replaced by nuh_layer_id.

- Normative references

The specifications in clause 2 apply.

- Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause 3. These definitions are either not present in clause 3 or replace definitions in clause 3.

- Abbreviations

The specifications in clause 4 apply.

- Conventions

The specifications in clause 5 apply.

- Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specifications in clause 6 apply.

- Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

  - Method of specifying syntax in tabular form

The specifications in subclause F.7.1 apply

  - Specification of syntax functions, categories, and descriptors

The specifications in subclause F.7.2 apply .

  - Syntax in tabular form

    - NAL unit syntax

The specifications of subclause F.7.3.1 and all its subclauses apply

    - Raw byte sequence payloads and RBSP trailing bits syntax

      - Video parameter set RBSP syntax

The specifications of subclause F.7.3.2.1 apply

      - Sequence parameter set RBSP

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_inter_layer_mfm_enable_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |
| } |  |

      - Picture parameter set RBSP syntax

The specifications of subclause F.7.3.2.3 apply.

      - Supplemental enhancement information RBSP syntax

The specifications of subclause F.7.3.2.4 apply.

      - Access unit delimiter RBSP syntax

The specifications of subclause F.7.3.2.5 apply.

      - End of sequence RBSP syntax

The specifications of subclause F.7.3.2.6 apply.

      - End of bitstream RBSP syntax

The specifications of subclause F.7.3.2.7 apply.

      - Filler data RBSP syntax

The specifications of subclause F.7.3.2.8 apply.

      - Slice segment layer RBSP syntax

The specifications of subclause F.7.3.2.9 apply.

      - RBSP slice segment trailing bits syntax

The specifications of subclause F.7.3.2.10 apply.

      - RBSP trailing bits syntax

The specifications of subclause F.7.3.2.11 apply.

      - Byte alignment syntax

The specifications of subclause F.7.3.2.12 apply.

    - Profile, tier and level syntax

The specifications of subclause F.7.3.3 apply.

    - Scaling list data syntax

The specifications of subclause F.7.3.4 apply.

    - Supplemental enhancement information message syntax

The specifications of subclause F.7.3.5 apply.

    - Slice segment header syntax

The specifications of subclause F.7.3.6 and all its subclauses apply.

    - Short-term reference picture set syntax

The specifications of subclause F.7.3.7 apply.

    - Slice segment data syntax

The specifications of subclause F.7.3.8 and all its subclauses apply.

  - Semantics

    - NAL unit semantics

The specifications of subclause F.7.4.1 and all its subclauses apply

    - Raw byte sequence payloads, trailing bits, and byte alignment semantics

      - Video parameter set RBSP semantics

The specifications of subclause F.7.4.2.1 apply.

      - Sequence parameter set RBSP semantics

The specifications of subclause F.7.4.2.2 apply with the following addition.

inter_view_mv_vert_constraint_flag equal to 1 specifies that vertical component of motion vectors used for inter layer prediction are constrained in the CVS. When inter_view_mv_vert_constraint_flag is equal to 1, the vertical component of the motion vectors used for inter-layer prediction shall be equal to or less than 56 in units of luma samples. When inter_view_mv_vert_constraint_flag is equal to 0, no constraint for of the vertical component of the motion vectors used for inter-layer prediction is signalled by this flag. When not present, the inter_view_mv_vert_constraint_flag is inferred to be equal to 0.

sps_inter_layer_mfm_enable_flag equal to 1 specifies the motion field mapping process is applied as part of the resampling process for reference pictures specified in subclause Error: Reference source not found. sps_inter_layer_mfm_enable_flag equal to 0 specifies that the motion field mapping processing is not applied. When not present, the value of sps_inter_layer_mfm_enable_flag is inferred to be equal to 0.  When avc_base_layer_flag is equal to 1, sps_inter_layer_mfm_enable_flag shall be equal to 0.

      - Picture parameter set RBSP semantics

The specifications of subclause F.7.4.2.3 apply.

      - Supplemental enhancement information RBSP semantics

The specifications of subclause F.7.4.2.4 apply.

      - Access unit delimiter RBSP semantics

The specifications of subclause F.7.4.2.5 apply.

      - End of sequence RBSP semantics

The specifications of subclause F.7.4.2.6 apply.

      - End of bitstream RBSP semantics

The specifications of subclause F.7.4.2.7 apply.

      - Filler data RBSP semantics

The specifications of subclause F.7.4.2.8 apply.

      - Slice segment layer RBSP semantics

The specifications of subclause F.7.4.2.9 apply.

      - RBSP slice segment trailing bits semantics

The specifications of subclause F.7.4.2.10 apply.

      - RBSP trailing bits semantics

The specifications of subclause F.7.4.2.11 apply.

      - Byte alignment semantics

The specifications of subclause F.7.4.2.12 apply.

    - Profile, tier and level semantics

The specifications of subclause F.7.4.3 apply.

    - Scaling list data semantics

The specifications of subclause F.7.4.4 apply.

    - Supplemental enhancement information message semantics

The specifications of subclause F.7.4.5 apply.

    - Slice segment header semantics

      - General slice segment header semantics

The specifications of subclause F.7.4.6.1 apply.

      - Reference picture list modification semantics

The specifications of subclause F.7.4.6.2 apply with the following modification.

The variable NumPocTotalCurr is derived as follows. [Ed. (YK): With the addition of the inter-layer stuff, this variable name gets confusing. Maybe it should be changed to "NumPicTotalCurr". Purely editorial - can be done later on, even in Version 1.]

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ StRpsIdx ]; i++)
	if(UsedByCurrPicS0[ StRpsIdx ][ i ] = = 1)
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ StRpsIdx ]; i++)	(G-xx)
	if(UsedByCurrPicS1[ StRpsIdx ][ i ] = = 1)
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] = = 1)
		NumPocTotalCurr++
NumPocTotalCurr += NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]

      - Weighted prediction parameters semantics

The specifications of subclause F.7.4.6.3 apply.

    - Short-term reference picture set semantics

The specifications of subclause F.7.4.7 apply.

    - Slice segment data semantics

The specifications of subclause F.7.4.8 and its all subclauses apply.

- Decoding process

  - General decoding process

The specifications of subclause F.8.1 apply.

    - Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The following steps specify the decoding processes using syntax elements in the slice segment layer and above:

–	Prior to decoding the first slice of the current picture, subclause G.7.1.2 is invoked.

–	At the beginning of the decoding process for each P or B slice, the marking process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- The following ordered steps specify the decoding processes using syntax elements in the coding tree unit layer and above.

–	The processes in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found are invoked for the decoding processes using syntax elements in the coding tree unit layer and above.

    - Marking process for inter-layer reference picture set

.[Ed. (YY): there is mixed use of terms such as resampled reference layer picture, resampled inter layer reference picture, and inter layer reference picture.]Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ currLayerId ] ]; i++ ) {
	rlPic = the picture with picture order count equal to PicOrderCnt and
		nuh_layer_id equal to RefLayerId[ LayerIdInVps[ currLayerId ][ i ] ]
	RefPicSetInterLayer[ i ] = rsPic as derived in the clause G.7.1.3 with rlPic given as input
	rsPic is marked as "used for long-term reference"
}

    - Resampling process for inter layer reference pictures

This process is invoked at the beginning of the decoding process for the first P or B slice of the current picture with nuh_layer_id not equal to 0.

Input to this process is

- A decoded reference layer picture rlPic.

Output of this process is

–	The resampled reference layer picture rsPic.

The variables nPicW and nPicH specify the width and height of the current picture in luma samples. The variable rsPicSample specifies the resampled sample values of rsPic of the luma and chroma components. The variable rsPicMotion specifies the resampled motion field of rsPic.

The variables nPicWRL and nPicHRL specify the width and height of the reference layer picture rlPic in luma samples. The variable rlPicSample specifies the sample values of rlPic of the luma and chroma components. The variable rlPicMotion specifies the motion field of rlPic.

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if nPicW is equal to nPicWRL and nPicH is equal to nPicHRL

  - rsPicSample is set to rlPicSample,

  - When sps_inter_layer_mfm_enable_flag is equal to 1, rsPicMotion is set to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.7.1.3.1 is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - When sps_inter_layer_mfm_enable_flag is equal to 1, the picture motion field resampling process as specified in subclause Error: Reference source not found is invoked with the compressed motion field of rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

      - Resampling process of picture sample values

[Ed. (YY): currently resampling process is defined for YUV4:2:0 only.]

Inputs to this process are:

–	a (nPicWRL)x(nPicHRL) array rlPicSampleL of luma samples

–	a (nPicWRL/2)x(nPicHRL/2) array rlPicSampleCb of chroma samples of the component Cb

–	a (nPicWRL/2)x(nPicHRL/2) array rlPicSampleCr of chroma samples of the component Cr

Outputs of this process are:

–	a (nPicW)x(nPicH) array rsPicSampleL of luma samples,

–	a (nPicW/2)x(nPicH/2) array rsPicSampleCb of chroma samples of the component Cb,

–	a (nPicW/2)x(nPicH/2) array rsPicSampleCr of chroma samples of the component Cr.

The array rsPicSampleL of the samples of the luma component is derived by invoking the luma sample resampling process specified in subclause G.7.1.3.1.1with the (nPicWRL)x(nPicHRL) array rlPicSampleL given as input.

The array rsPicSampleCb of the samples of the chroma component Cb is derived by invoking the chroma sample resampling process specified in subclause G.7.1.3.1.2 with the (nPicWRL/2)x(nPicHRL/2) array rlPicSampleCb and cIdx equal to 1 given as input.

The array rsPicSampleCr of the samples of the chroma component Cr is derived by invoking the chroma sample resampling process specified in subclause G.7.1.3.1.2 with the (nPicWRL/2)x(nPicHRL/2) array rlPicSampleCr and cIdx equal to 2 given as input.

        - Resampling process of luma sample values

Inputs to this process are:

–	a (nPicWRL)x(nPicHRL) array rlPicSampleL.

Outputs of this process are:

–	a (nPicW)x(nPicH) array rsPicSampleL  of luma samples.

The luma samples rsPicSampleL [ xP ][ yP ] with ( xP = 0.. nPicW – 1, yP = 0.. nPicH – 1) are derived by invoking the luma sample interpolation process specified in subclause G.7.1.3.1.3with rlPicSampleL and luma sample location ( xP, yP ) given as inputs and rsPicSampleL[ xP ][ yP ] as output.

        - Resampling process of chroma sample values

Inputs to this process are:

–	a (nPicWRLC)x(nPicHRLC) array rlPicSampleC,

–	a variable cIdx specifying the color component index.

Outputs of this process are:

–	a (nPicWC)x(nPicHC) array rsPicSampleC of chroma samples.

The chroma samples rsPicSampleC[ xPC ][ yPC ] with ( xPC = 0..nPicWC – 1, yPC = 0..nPicHC – 1) are derived by invoking the chroma sample interpolation process specified in subclause G.7.1.3.1.4 with rlPicSampleC, cIdx and chroma sample location ( xPC , xPC  ) given as inputs and rsPicSampleC[ xPC ][ yPC ] as output.

        - Luma sample interpolation process

Inputs to this process are

–	a (nPicWRL)x(nPicHRL) array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a interpolated luma sample value intLumaSampleL.

- Error: Reference source not found specifies the 8-tap filter coefficients fL[ p, x ] with p = 0..15 and x = 0..7 used for the luma resampling process.

Table G-1 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 4 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 7 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 10 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 11 | −1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 14 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 15 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample locations used in resampling as specified in subclause G.7.1.3.1.5 is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 )	(G-xx)
xPhase = ( xRef16 ) % 16	(G-xx)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 )	(G-xx)
yPhase = ( yRef16 ) % 16	(G-xx)

- The sample value tempArray[ n] with n = 0..7, is derived as follows.

yPosRL = Clip3( 0, nPicHRL – 1, yRef + n – 1 ) 	(G-xx)

tempArray[n] = fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef – 3), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef      ), yPosRL ] +
	fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 1), yPosRL ] +	(G-xx)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, nPicWRL – 1, xRef + 4), yPosRL ]

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(G-xx)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + 211) >> 12

intLumaSample = Clip3( 0, ( 1 << BitDepthY) – 1 , intLumaSample )	(G-xx)

        - Chroma sample interpolation process

Inputs to this process are:

–	a (nPicWRLC)x(nPicHRLC) array rlPicSampleC,

–	a chroma sample location ( xPC, yPC ) relative to the top-left chorma sample of the current picture.

Output of this process is a interpolated chroma sample value intChromaSample.

- Error: Reference source not found specifies the 4-tap filter coefficients fC[ p, x ] with p = 0..15 and x = 0..3 used for the chroma resampling process.

Table G-2 16-phase chroma resampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | n/a | n/a | n/a | n/a |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | n/a | n/a | n/a | n/a |
| 11 | −2 | 20 | 26 | −6 |
| 12 | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

The value of the interpolated chroma sample value intChromaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample locations used in resampling as specified in subclause G.7.1.3.1.5 is invoked with cIdx and chroma sample location ( xPC, yPC ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 )	(8-xx)
xPhase = ( xRef16 ) % 16	(8-xx)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 )	(8-xx)
yPhase = ( yRef16 ) % 16	(8-xx)

- The sample value tempArray[ n ] with n = 0 .. 3, is derived as follows.

yPosRL = Clip3( 0 , nPicHRLC – 1, yRef + n – 1 )	(8-xx)

tempArray[n] = fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, nPicWRLC – 1, xRef – 1), yPosRL ] +
	fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, nPicWRLC – 1, xRef      ), yPosRL ] +
	fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, nPicWRLC – 1, xRef + 1), yPosRL ] +	(8-xx)
	fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, nPicWRLC – 1, xRef + 2), yPosRL ]

- The interpolated chroma sample value intChromaSample is derived as follows.

intChromaSample = (fC[ yPhase, 0 ] * tempArray [ 0 ] +
	fC[ yPhase, 1 ] * tempArray [ 1 ] +
	fC[ yPhase, 2 ] * tempArray [ 2 ] +	(8-xx)
	fC[ yPhase, 3 ] * tempArray [ 3 ] + 211) >> 12

intChromaSample = Clip3( 0, ( 1 << BitDepthC) – 1 , intChromaSample )	(8-xx)

        - Derivation process for reference layer sample locations

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left picture sample of the color component of the current picture specified by cIdx,

Output of this process is a sample location ( xRef16, yRef16 ) specifying the collocated reference sample location in units of 1/16-th sample.

The variable xRef16 is derived as follow:

xRef16 = ( xP * nPicWRL * 16 + nPicW / 2) / nPicW					(G-xx)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = (yP * nPicHRL * 16 + nPicH / 2 ) / nPicH				(G-xx)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * nPicHRL * 16 + nPicH / 2 ) / nPicH – offset	(G-xx)

where the value of offset is derived as specified by the following:

if (nPicH is equal to nPicHRL)
			offset = 0
		otherwase if (nPicH is equal to 1.5 * nPicHRL)
			offset = 1
		otherwase if (nPicH is equal to 2.0 * nPicHRL)
			offset = 2

[Ed. (JB): The value of offset is taken from SVC extension of AVC, and need to be reconsidered.]

In bitstreams conforming to this version of this Specification, the values of nPicH shall be equal to the value of nPicHRL, 1.5 * nPicHRL or 2.0 * nPicHRL, and the values of nPicW shall be equal to the value of nPicWRL, 1.5 * nPicWRL or 2.0 * nPicWRL . [Ed. (JL),the place holder for this restriction need to be re-considered if it exists in the final spec]

      - Resampling process of  picture motion field

Input to this process is

- A variable rlPicMotion specifying the motion field of the picture rlPic.

Output of this process is

–	The variable rsPicMotion specifying the resampled motion field of the resampled picture.

The motion field of rlPic specified by rlPicMotion consists of:

- a (nPicWRL)x(nPicHRL) two-dimensional array predModeRL specifies the prediction modes of the reference layer picture rlPic,

- two (nPicWRL)x(nPicHRL) two-dimensional arrays refIdxLXRL specify the reference indices of the reference layer picture rlPic, with X = 0,1,

- two (nPicWRL)x(nPicHRL) two-dimensional arrays mvLXRL specify the luma motion vectors of the reference layer picture rlPic, with X = 0,1,

- two (nPicWRL)x(nPicHRL) two-dimensional arrays RefPicOrderCntLXRL specify the reference picture order counts of the reference layer picture rlPic, with X = 0, 1.

The resampled motion field specified by rsPicMotion consists of:

–	a (nPicW)x(nPicH) two-dimensional array predMode specifies the prediction modes of the resampled picture,

–	two (nPicW)x(nPicH) two-dimensional arrays refIdxLX specify the reference indexes of the resampled picture, with X = 0,1,

–	two (nPicW)x(nPicH)  two-dimensional arrays mvLX  specify the luam motion vectors of the resampled picture, with X = 0,1,

–	two (nPicW)x(nPicH) two-dimensional arrays RefPicOrderCntLX specify the reference picture order counts of the resampled picture, with X = 0, 1.

For each luma sample location xPb = 0...((nPicWRL+15)>>4)-1 and yPb = 0…((nPicHRL+15)>>4)-1,

–	The variables xP and yP are set to (xPb << 4) and (yPb << 4), respectively,

–	The variables predMode[xP][yP], refIdxLX[xP][yP], mvLX[xP][yP] and RefPicOrderCntLX[xP][yP], with X = 0,1, of the resampled picture are derived by invoking inter layer motion derivation process specified in subclause G.7.1.3.2.1 with the luma location ( xP, yP ), predModeRL, refIdxLXRL, mvLXRL and RefPicOrderCntLXRL, with X = 0,1, given as input.

        - Derivation process for inter layer motion

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a (nPicWRL)x(nPicHRL) two-dimensional array predModeRL,

- two (nPicWRL)x(nPicHRL) two-dimensional arrays refIdxL0RL and refIdxL1RL

- two (nPicWRL)x(nPicHRL) two-dimensional arrays mvL0RL and mvL1RL

- two (nPicWRL)x(nPicHRL) two-dimensional arrays RefPicOrderCntL0RL and RefPicOrderCntL1RL.

Outputs of this process are

- a derived prediction mode predMode[ xP ][ yP ],

- two derived motion vectors mvL0[ xP ][ yP ] and mvL1[ xP ][ yP ]

- two derived reference indices refIdxL0[ xP ][ yP ] and refIdxL1[ xP ][ yP ]

- two derived reference picture order counts RefPicOrderCntL0[ xP ][ yP ] and RefPicOrderCntL1[ xP ][ yP ]

The variables predMode[ xP ][ yP ], mvLX[ xP ][ yP ], refIdxLX[ xP ][ yP ] and refPicOrderCntLX[ xP ][ yP ] are derived as follows.

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xP + 8													(G-xx)

yPCtr = yP + 8													(G-xx)

- The collocated position (xRL, yRL) is derived as follows

xRL = ( xPCtr * nPicWRL + nPicW / 2 ) / nPicW							(G-xx)

yRL = ( yPCtr * nPicHRL + nPicH / 2 ) / nPicH							(G-xx)

xRL = ( xRL >> 4 ) << 4												(G-xx)

yRL = ( yRL >> 4 ) << 4												(G-xx)

- The reference layer motion vector is derived as follows

- If ( xRL >= nPicWRL ) or ( yRL >= nPicHRL ), predMode[ xP ][ yP ] is set to MODE_INTRA.

- Otherwise, predMode[ xP ][ yP ] is derived as follows

predMode[ xP ][ yP ] = predModeRL[ xRL ][ yRL ]							(G-xx)

- If predMode[ xP ][ yP ] is equal to MODE_INTER, for each X = 0, 1, the following applies

refIdxLX[ xP ][ yP ] = refIdxLXRL[ xRL ][ yRL ]							(G-xx)

RefPicOrderCntLX[ xP ][ yP ] = RefPicOrderCntLXRL[ xRL ][ yRL ]			(G-xx)

mvLX[ xP ][ yP ][0] = (mvLXRL[ xRL ][ yRL ][0] * nPicW + offset[0]) / nPicWRL	(G-xx)

mvLX[ xP ][ yP ][1] = (mvLXRL[ xRL ][ yRL ][1] * nPicH + offset[1]) / nPicHRL	(G-xx)

        - where

          - offset[ 0 ] is set to ( nPicWRL /2 – 1) if mvLXRL[ xRL ][ yRL ][ 0 ] > 0 , and ( 1 – nPicWRL / 2 ) otherwise;

          - offset[ 1 ] is set to ( nPicHRL / 2 – 1) if mvLXRL[ xRL ][ yRL ][ 1 ] > 0, and  ( 1 – nPicHRL / 2 ) otherwise.

- Otherwise, if predMode[ xP ][ yP ] is equal to MODE_INTRA

        - both components of mvL0[ xP ][ yP ] and mvL1[ xP ][ yP ] are set to 0, refIdxL0[ xP ][ yP ] and refIdxL1[ xP ][ yP ] are set to –1, RefPicOrderCntL0[ xP ][ yP ] and RefPicOrderCntL1[ xP ][ yP ] are set to –1

  - NAL unit decoding process

The specifications in subclause 8.2 apply.

  - Slice decoding process

    - Decoding process for picture order count

The specifications in subclause 8.3.1 apply.

    - Decoding process for reference picture set

The specifications in subclause 8.3.2 apply.

    - Decoding process for generating unavailable reference pictures

The specifications in subclause 8.3.3 apply.

    - Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause Error: Reference source not found. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(G-xx)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 
}

The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(G-xx)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
		RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(G-xx)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	for( i = NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1; i >= 0; rIdx++, i-- )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)	(G-xx)
	RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :RefPicListTemp1[ rIdx ]

  - Decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4 apply.

  - Decoding process for coding units coded in inter prediction mode

    - General decoding process for coding units coded in inter prediction mode

The specifications in subclause 8.5.1 apply.

    - Inter prediction process

The specifications in subclause 8.5.2 apply.

    - Decoding process for prediction units in inter prediction mode

The specifications in subclause 8.5.3 apply with the following additions.

It is a requirement of bitstream conformance that, for X being replaced by either 0 or 1, the variables mvLX[0] and mvLX[1] as an output of the subclause 8.5.3.1 shall be equal to 0 if the value of refIdxLX as an output of the subclause 8.5.3.1 corresponds to a resampled reference layer picture. That is, in any conformant bitstream, for X being replaced by either 0 or 1, upon invoking the decoding process in subclause 8.5.3.1, the values of the syntax elements merge_idx, mvp_lX_flag, ref_idx_lX, MvdLX, and mvd_l1_zero_flag shall always result in zero values for mvLX[0] and mvLX[1] when the value of refIdxLX of the reference picture list RefPicListX indicates a resampled reference layer picture. [Ed. (YY): look for another more precise and concise way to define “corresponds to a resampled reference layer picture”]

    - Decoding process for the residual signal of coding units coded in inter prediction mode

The specifications in subclause 8.5.4 apply with the following additions.

  - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 apply.

  - In-loop filter process

The specifications in subclause 8.7 apply.

- Parsing process

The specifications in clause 9 apply.

- Specification of bitstream subsets

The specifications in clause 10 apply.

- (Void)

- Byte stream format

The specifications in Annex F.12 apply.

- Hypothetical reference decoder

The specifications in Annex F.13 and its subclauses apply.

- SEI messages

The specifications in subclause F.14 and its subclauses apply

- Video usability information

The specifications in subclause F.15 apply

- Annex H 

TextureRL based scalable extensions

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax, semantics and decoding processes for scalable coding extensions.

- Scope

Bitstreams and decoders conforming to this annex are completely specified in this annex with reference made to clauses 2-9 and Annexes A-F where occurrences of nuh_layer_id in clauses 2-9 and Annexes A-E is consistently replaced by nuh_layer_id.

- Normative references

The specifications in clause 2 apply.

- Definitions

For the purpose of this annex, the following definitions apply in addition to the definitions in clause 3. These definitions are either not present in clause 3 or replace definitions in clause 3.

TBD

- Abbreviations

The specification in clause 4 apply.

- Conventions

The specification in clause 5 apply.

- Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

The specification in clause 6 apply.

- Syntax and semantics

This clause specifies syntax and semantics for coded video sequences that conform to one or more of the profiles specified in this annex.

  - Method of specifying syntax in tabular form

The specifications in subclause F.7.1 apply.

  - Specification of syntax functions, categories, and descriptors

The specifications in subclause F.7.2 apply.

  - Syntax in tabular form

    - NAL unit syntax

The specifications in subclause F.7.3.1 apply.

    - Raw byte sequence payloads and RBSP trailing bits syntax

The specifications in subclause F.7.3.2 apply.

    - Profile, tier and level syntax

TBD

    - Bit rate and picture rate information syntax

The specifications in subclause F.7.3.4 apply.

    - Operation point set syntax

The specifications in subclause F.7.3.5 apply.

    - Scaling list data syntax

The specifications in subclause F.7.3.6 apply

    - Supplemental enhancement information message syntax

The specifications in subclause F.7.3.7 apply

    - Slice segment header syntax

The specifications in subclause F.7.3.8 apply

    - Slice segment data syntax

      - General slice segment data syntax

The specifications in subclause 7.3.9.1 apply.

      - Coding tree unit syntax

The specifications in subclause 7.3.9.2 apply.

      - Sample adaptive offset syntax

The specifications in subclause 7.3.9.3 apply.

      - Coding quadtree syntax

The specifications in subclause 7.3.9.4 apply.

      - Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if(( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) ¦¦ PredMode[ x0 ][ y0 ] = = MODE_TEXTURE_RL) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

      - Prediction unit syntax

The specifications in subclause F.7.3.9.6 apply.

      - PCM sample syntax

The specifications in subclause F.7.3.9.7 apply.

      - Transform tree syntax

The specifications in subclause F.7.3.9.8 apply.

      - Motion vector difference syntax

The specifications in subclause F.7.3.9.9 apply.

      - Transform unit syntax

The specifications in subclause F.7.3.9.10 apply.

      - Residual coding syntax

The specifications in subclause F.7.3.9.11 apply.

  - Semantics

    - NAL unit semantics

The specifications in subclause F.7.4.1 apply.

    - Raw byte sequence payloads and RBSP trailing bits semantics

The specifications in subclause F.7.4.2 apply.

    - Profile, tier and level semantics

The specifications in subclause F.7.4.3 apply.

    - Bit rate and picture rate information semantics

The specifications in subclause F.7.4.4 apply.

    - Operation point set semantics

The specifications in subclause F.7.4.5 apply.

    - Scaling list data semantics

The specifications in subclause F.7.4.6 apply

    - Supplemental enhancement information message semantics

The specifications in subclause F.7.4.7 apply

    - Slice segment header semantics

The specifications in subclause F.7.4.8 apply

    - Slice segment data semantics

      - General slice segment data semantics

The specifications in subclause F.7.4.9.1 apply.

      - Coding tree unit semantics

The specifications in subclause F.7.4.9.2 apply.

      - Sample adaptive offset semantics

The specifications in subclause F.7.4.9.3 apply.

      - Coding quadtree semantics

The specifications in subclause F.7.4.9.4 apply.

      - Coding unit semantics

The specifications in subclause F.7.4.9.5 apply with following modifications and additions.

texture_rl_flag[ x0 ][ y0 ] specifies the prediction mode for the current coding unit. texture_rl_flag[ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived using reconstructed samples of the reference layer representation as specified in subclause H.7.8; texture_rl_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in either intra prediction or inter prediction mode.

When texture_rl_flag is equal to 1, the variable PredMode[ x ][ y ] and PartMode[ x ][ y ] are derived as follows.

for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS – 1,

PredMode[ x ][ y ] = MODE_TEXTURE_RL		(H-xx)

PartMode [ x ][ y ] = PART_2Nx2N		(H-xx)

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to 0.

      - Prediction unit semantics

The specifications in subclause 7.4.9.6 apply.

      - PCM sample semantics

The specifications in subclause 7.4.9.7 apply.

      - Transform tree semantics

The specifications in subclause 7.4.9.8 apply.

      - Motion vector difference semantics

The specifications in subclause 7.4.9.9 apply.

      - Transform unit semantics

The specifications in subclause 7.4.9.10 apply.

      - Residual coding semantics

The specifications in subclause 7.4.9.11 apply.

- Decoding process

  - General decoding procss

The specifications in subclause F.8.1 apply.

    - Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify decoding processes using syntax elements in the slice segment layer and above:

- The processes in subclauses H.7.4, H.7.5, H.7.6, H.7.7 and H.7.8 specify decoding processes using syntax elements in the coding tree unit layer and above.

  - (void)

[Ed.(MH): The (void) subclauses are merely to keep the subclause numbers similar to those in clause 8.]

(void)

  - (void)

(void)

  - Decoding process for coding units coded in intra prediction mode

The specifications in subclause 8.4 apply.

  - Decoding process for coding units coded in inter prediction mode

The specifications in subclause 8.5 and its subclauses apply with the following modifications.

    - Decoding process for prediction units in inter prediction mode

When InterLayerMotionPredictionEnableFlag equal to 1, the specifications in subclause 8.5.3 apply with following modifications and additions. When InterLayerMotionPredictionEnableFlag equal to 0, the specifications in subclause 8.5.3 apply.

      - Derivation process for motion vector components and reference indices

The specifications in subclause 8.5.3.1 apply with following modifications and additions.

        - Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The location ( xOrigP, yOrigP ) and the variables nOrigPbW and nOrigPbH are derived to store the values of ( xOrigP, yOrigP ) , nPbW and nPbH as follows.

( xOrigP, yOrigP ) is set equal to ( xP, yP )		(H-xx)

nOrigPbW = nPbW		(H-xx)

nOrigPbH = nPbH		(H-xx)

When log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, ( xP, yP ), nPbW, nPbH and partIdx are modified as follows.

( xP, yP ) = ( xC, yC )		(H-xx)

nPbW = nCS		(H-xx)

nPbH = nCS		(H-xx)

partIdx = 0		(H-xx)

NOTE – When log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from co-located reference layer prediction unit partitions in subclause H.7.5.1.1.10 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx and the picture of the reference layer representation RlPic as inputs and the output being the availability flags availableFlagRL, the reference indices refIdxLXRL, the prediction list utilization flags predFlagLXRL, and the motion vectors mvLXRL (with X being 0 or 1, respectively).

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause H.7.5.1.1.2 is invoked with the luma coding block location ( xC, yC ), the coding block size nCS, the luma prediction block location ( xP, yP ), the variable the width and the height of the luma prediction block nPbW and nPbH and the partition index partIdx as well as availableFlagRL, mvLXRL and refIdxLXRL as inputs and the output being the availability flags availableFlagTextureRL, availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, availableFlagB2, the reference indices refIdxLXTextureRL, refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, refIdxLXB2, the prediction list utilization flags predFlagTextureRL, predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, predFlagLXB2 and the motion vectors mvLXTextureRL, mvLXA0, mvLXA1, mvLXB0, mvLXB1, mvLXB2 (with X being 0 or 1, respectively).

- The reference indices for the temporal merging candidate, refIdxLXCol (with X being 0 or 1, respectively), are set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, and refIdxLXCol as the inputs and the output being the availability flags availableFlagLXCol and the temporal motion vectors mvLXCol (with X being 0 or 1, respectively). The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(H-xx)

predFlagLXCol = availableFlagLXCol		(H-xx)

- The merging candidate list, mergeCandList, is constructed as follows.

- RL, if availableFlagRL is equal to 1

- A1, if availableFlagA1 is equal to 1

- B1, if availableFlagB1 is equal to 1

- B0, if availableFlagB0 is equal to 1

- TextureRL, if availableFlagTextureRL is equal to 1

- A0, if availableFlagA0 is equal to 1

- B2, if availableFlagB2 is equal to 1

- Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xOrigP][ yOrigP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xOrigP ][ yOrigP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(H-xx)

mvLX[ 1 ] = mvLXN[ 1 ]	(H-xx)

refIdxLX = refIdxLXN	(H-xx)

predFlagLX = predFlagLXN	(H-xx)

- When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, the following applies.

refIdxL1 = −1		(H-xx)

predFlagL1 = 0		(H-xx)

        - Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a variables availableFlagRL, predFlagLXRL, mvLXRL and refIdxLXRL

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagTextureRL, availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, availableFlagB2 of the neighbouring prediction units,

- the reference indices refIdxLXTextureRL, refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, refIdxLXB2 of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXTextureRL, predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1, predFlagLXB2 of the neighbouring prediction units,

- the motion vectors mvLXTextureRL, mvLXA0, mvLXA1, mvLXB0, mvLXB1, mvLXB2 of the neighbouring prediction units.

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1, and mvLXA1 the following applies.

–	The luma location ( xA1, yA1 ) inside the neighbouring luma coding block is set equal to ( xP − 1,  yP + nPbH − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xA1, yA1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA1.

–	When one or more of the following conditions are true, availableA1 is set equal to FALSE

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xA1 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yA1 >> (log2_parallel_merge_level_minus2 + 2)).3

- PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and partIdx is equal to 1

- if PredFlagLX[ xA1 ][ yA1 ] is equal to 1 and MvLX[ xA1 ][ yA1 ] is equal to mvLXRL and RefIdxLX[ xA1 ][ yA1 ] is equal to refIdxLXRL for X being (0..1)

–	The variables availableFlagA1, refIdxLXA1, predFlagLXA1, and mvLXA1 are derived as follows.

- If availableA1 is equal to FALSE, availableFlagA1 is set equal to 0, both components mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0 (with X being 0 or 1, respectively).

- Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made.

mvLXA1 = MvLX[ xA1 ][ yA1 ]		(H-xx)
refIdxLXA1 = RefIdxLX[ xA1 ][ yA1 ]		(H-xx)
predFlagLXA1 = PredFlagLX[ xA1 ][ yA1 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXA1, mvLXA1 is equal to mvLXRL and refIdxLXA1 is equal to refIdxLXRL for X being (0..1) availableFlagA1 is set to 0.

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1, and mvLXB1 the following applies.

–	The luma location ( xB1, yB1 ) inside the neighbouring luma coding block is set equal to ( xP + nPbW − 1,  yP − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xB1, yB1 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB1.

–	When one or more of the following conditions are true, availableB1 is set equal to FALSE

- (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xB1 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yB1 >> (log2_parallel_merge_level_minus2 + 2)).

- PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and partIdx is equal to 1

–	The variables availableFlagB1, refIdxLXB1, predFlagLXB1, and mvLXB1 are derived as follows.

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0 (with X being 0 or 1, respectively).

    - availableB1 is equal to FALSE

    - availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xB1, yB1 ) have the same motion vectors and the same reference indices

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made.

mvLXB1 = MvLX[ xB1 ][ yB1 ]		(H-xx)
refIdxLXB1 = RefIdxLX[ xB1 ][ yB1 ]		(H-xx)
predFlagLXB1 = PredFlagLX[ xB1 ][ yB1 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB1, mvLXB1 is equal to mvLXRL and refIdxLXB1 is equal to refIdxLXRL for X being (0..1) availableFlagB1 is set to 0.

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0, and mvLXB0 the following applies.

–	The luma location ( xB0, yB0 ) inside the neighbouring luma coding block is set equal to ( xP + nPbW,  yP − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xB0, yB0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB0.

–	When (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xB0 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yB0 >> (log2_parallel_merge_level_minus2 + 2)), availableB0 is set equal to FALSE

–	The variables availableFlagB0, refIdxLXB0, predFlagLXB0, and mvLXB0 are derived as follows.

- If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0 (with X being 0 or 1, respectively).

    - availableB0 is equal to FALSE

    - availableB1 is equal to TRUE and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xB0, yB0 ) have the same motion vectors and the same reference indices

- Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made.

mvLXB0 = MvLX[ xB0 ][ yB0 ]		(H-xx)
refIdxLXB0 = RefIdxLX[ xB0 ][ yB0 ]		(H-xx)
predFlagLXB0 = PredFlagLX[ xB0 ][ yB0 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB0, mvLXB0 is equal to mvLXRL and refIdxLXB0 is equal to refIdxLXRL for X being (0..1) availableFlagB0 is set to 0.

For the derivation of availableFlagTextureBL, refIdxLXTextureBL, predFlagLXTextureBL, and mvLXTextureBL the following applies.

–	If one or more of the following conditionis are true, availableFalgTextureBL is set equal to 0, both components of mvLXTextureBL are set equal to 0, refIdxLXTextureBL is set equal to -1 and predFlagLXTextureBL is set equal to 0 (with X being 0 or 1, respectivel).

- nPbW, nPbH and nCS are the same

- All of availableFlagRL, availableFlagA1, availableFlagB1, and availableFlagB0 are equal to 1

–	Otherwise, availableFlagTextureBL is set equal to 1 and the following assignements are made.

mvLXTextureBL = (0, 0)		(H-xx)
refIdxLXTextureBL = -1		(H-xx)
predFlagLXTextureBL =  0		(H-xx)

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0, and mvLXA0 the following applies.

–	The luma location ( xA0, yA0 ) inside the neighbouring luma coding block is set equal to ( xP − 1,  yP + nPbH ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xA0, yA0 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableA0.

–	When (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xA0 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yA0 >> (log2_parallel_merge_level_minus2 + 2)), availableA0 is set equal to FALSE

–	The variables availableFlagA0, refIdxLXA0, predFlagLXA0, and mvLXA0 are derived as follows.

- If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0 (with X being 0 or 1, respectively).

    - availableA0 is equal to FALSE

    - availableA1 is equal to TRUE and the prediction units covering luma location ( xA1, yA1 ) and luma location ( xA0, yA0 ) have the same motion vectors and the same reference indices

    - availableFlagRL + availableFlagTextureBL + availableFlagA0 + availableFlagA1 + availableFlagB0 is equal to 4.

- Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made.

mvLXA0 = MvLX[ xA0 ][ yA0 ]		(H-xx)
refIdxLXA0 = RefIdxLX[ xA0 ][ yA0 ]		(H-xx)
predFlagLXA0 = PredFlagLX[ xA0 ][ yA0 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXA0, mvLXA0 is equal to mvLXRL and refIdxLXA0 is equal to refIdxLXRL for X being (0..1) availableFlagA0 is set to 0.

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2, and mvLXB2 the following applies.

–	The luma location ( xB2, yB2 ) inside the neighbouring luma coding block is set equal to ( xP − 1, yP − 1 ).

–	The availability derivation process for a prediction block as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the current luma coding block size nCbS set equal to nCS, the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH, the luma location ( xB2, yB2 ) and the partition index partIdx as inputs and the output is assigned to the prediction block availability flag availableB2.

–	When (xP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (xB2 >> (log2_parallel_merge_level_minus2 + 2)) and (yP >> (log2_parallel_merge_level_minus2 + 2)) is equal to (yB2 >> (log2_parallel_merge_level_minus2 + 2)), availableB2 is set equal to FALSE

–	The variables availableFlagB2, refIdxLXB2, predFlagLXB2, and mvLXB2 are derived as follows.

- If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0 (with X being 0 or 1, respectively).

    - availableB2 is equal to FALSE

    - availableA1 is equal to TRUE and prediction units covering luma location ( xA1, yA1 ) and luma location ( xB2, yB2 ) have the same motion vectors and the same reference indices

    - availableB1 is equal to TRUE and the prediction units covering luma location ( xB1, yB1 ) and luma location ( xB2, yB2 ) have the same motion vectors and the same reference indices

    - availableFlagRL + availableFlagTextureBL + availableFlagA0 + availableFlagA1 + availableFlagB0 is equal to 4.

- Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made.

mvLXB2 = MvLX[ xB2 ][ yB2 ]		(H-xx)
refIdxLXB2 = RefIdxLX[ xB2 ][ yB2 ]		(H-xx)
predFlagLXB2 = PredFlagLX[ xB2 ][ yB2 ]		(H-xx)

- if availableFlagRL is equal to 1, and predFlagLXRL is equal to predFlagLXB2, mvLXB2 is equal to mvLXRL and refIdxLXB2 is equal to refIdxLXRL for X being (0..1) availableFlagB2 is set to 0.

        - Derivation process for co-located reference layer merging candidates

Inputs to this process are

- a luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current luma coding block,

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a variable partIdx specifying the index of the current prediction unit within the current coding unit.

- a reference layer picture rlPic

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flags availableFlagRL of the co-located prediction units,

- the reference indices refIdxLXRL of the co-located prediction units,

- the prediction list utilization flags predFlagLXRL of the co-located prediction units,

- the motion vectors mvLXRL of the co-located prediction units.

For the derivation of availableFlagRL, refIdxLXRL, predFlagLXRL, and mvLXRL the following applies.

- The availability derivation process for a prediction block as specified in subclause H.7.5.1.1.11 is invoked with the luma location ( xP, yP ), the width and the height of the luma prediction block nPbW and nPbH and and the reference layer picure rlPic as inputs and the output is assigned to the prediction block availability flag availableFlagRL, refIdxLXRL, and mvLXRL. The variables availableFlagRL and predFlagLXRL (with X being 0 or 1, respectively) are derived as specified below.

availableFlagRL = availableFlagL0RL | | availableFlagL1	(H-xx)

predFlagLXRL = availableFlagLXRL	(H-xx)

The validity check is performed for derived refIdxLXRL from the co-located predicted unit of the reference layer and the following applies.

- If refIdxLXRL is greater than num_ref_idx_lX_active_minus1 the predFlagLXRL is set to 0 for X being (0..1).

        - Derivation process for inter-layer luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the luma prediction block, nPbW and nPbH,

- a reference layer picture rlPic

Outputs of this process are (with X being 0 or 1, respectively)

- the availability flag availableFlagLXRL,

- the reference index refIdxLXRL,

- the motion vector prediction mvLXRL.

The variables nPicW and nPicH specify the width and the height of the currently decoded picture in luma samples.

The variables nPicWRL and nPicHRL specify the width and the height of rlPic in luma samples,

The variable currPb specifies the current luma prediction block at luma location ( xP, yP ).

The variables mvLXRLand availableFlagLXRL with X being 0 or 1, are derived as follows.

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xP + nPbW / 2	(H-xx)

yPCtr = yP + nPbH / 2	(H-xx)

- The reference layer motion vector is derived as follows

- The colocated position derivation process as specified in subclause H.7.5.1.1.12 is invoked with  xPCtr, yPCtr, nPicW, nPicH, nPicWRL, and nPicHRL as inputs and the outputs being assigned to  xPCtrRL and yPCtrRL ;

- The variable blPb specifies the luma prediction block covering the modified location given by ( ( xPCtrRL >> 4 ) << 4, ( yPCtrRL >> 4 ) << 4 ) inside the reference layer picture specified by rlPic.

- The luma location ( xPRL, yPRL ) is set equal to the top-left sample of the collocated luma prediction block specified by blPb relative to the top-left luma sample of the reference layer picture specified by rlPic.

- The arrays predFlagLXRL[ x ][ y ], mvLXRL[ x ][ y ] and refIdxLXRL[ x ][ y ] are set equal to the corresponding arrays of the reference layer picture specified by rlPic, PredFlagLX[ x ][ y ], mvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for. The variables mvLXRL, refIdxLXRL and availableFlagLXRL are derived as follows

        - If colPb is coded in an intra prediction mode, both components of mvLXRL are set equal to 0, availableFlagLXRL is set equal to 0, and refIdxLXRL is set equal to -1;

          - Otherwise, If predFlagLXRL[ xPRL ][ yPRL ] is equal to 0, both components of mvLXRL are set equal to 0, and refIdxLXRL is set equal to -1, and availableFlagLXRL is set equal to 0.

          - Ohterwise, refIdxLXRL is set equal to  RefIdxLXRL [ xPRL ][ yPRL ], and availableFlagLXRL is set equal to 1. The two components of mvLXRL are derived as follows

mvLXRL [0] =  (mvLXRL[ xPRL ][ yPRL ][0]* nPicW+ offset[0])/ nPicWRL

mvLXRL [1] =  (mvLXRL[ xPRL ][ yPRL ][1]* nPicH+ offset[1]) / nPicHRL

          - where

            - offset[0] is set to (nPicWRL/2-1) if mvLXRL[ xPRL ][ yPRL ][0] >0, and (1- nPicWRL/2) otherwise;

            - offset[1] is set to (nPicHRL/2-1) if mvLXRL[ xPRL ][ yPRL ][1] >0, and  (1- nPicHRL/2) otherwise.

        - Derivation process for the collocated position of  the current picture in a reference layer picture

Inputs of this process are:

– a luma location ( xC, yC ) relative to the top-left luma sample of the current picture,

– variables specifying the width and the height of the current picture in luma samples, nPicW and nPicH,

– 	variables specifying the width and the height of a reference layer picture in luma samples, nPicWRL and nPicHRL,

Ourput of this process is:

– a luma location ( xRL, yRL ) relative to the top-left luma sample of the reference layer picture.

The variable nPicW and nPicH specify the width and the height of the current picture.

The variables xRL and yRL are derived as follows:

xRL = ( xC * nPicWRL + nPicW / 2) / nPicW						(H-xx)

yRL = ( yC * nPicHRL + nPicH / 2 ) / nPicH						(H-xx)

  - Scaling, transformation and array construction process prior to deblocking filter process

The specifications in subclause 8.6 and its subclauses apply with following modifications .

    - Transformation process for scaled transform coefficients

The specifications in subclause 8.6.4 and its subclauses apply with following modifications .

Inputs of this process are:

–	a luma location ( xT, yT ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nT specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	a (nT)x(nT) array d of scaled transform coefficients with elements d[ x ][ y ].

Output of this process is:

–	the (nT)x(nT) array r of residual samples with elements r[ x ][ y ].

Depending on CuPredMode[ xT ][ yT ], nT and cIdx, the variable trType is derived as follows.

–	If CuPredMode[ xT ][ yT ] is equal to MODE_INTRA or MODE_TEXTURE_RL, nT is equal to 4, and cIdx is equal to 0, trType is set equal to 1.

–	Otherwise, trType is set equal to 0.

The (nT)x(nT) array r of residual samples is derived as follows.

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 is transformed to e[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found for each column x = 0..nT − 1 with the size of the transform block nT, the list d[ x ][ y ] with y = 0..nT − 1 and the transform type variable trType as the inputs and the output is the list e[ x ][ y ] with y = 0..nT − 1.

- The intermediate sample values g[ x ][ y ] with  x = 0..nT − 1, y = 0..nT − 1  are derived by

g[ x ][ y ] = Clip3( −32768, 32767, ( e[ x ][ y ] + 64 ) >> 7 )	(H-xx)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 is transformed to r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found for each row y = 0..nT − 1 with the size of the transform block nT, the list g[ x ][ y ] with x = 0..nT − 1 and the transform type variable trType as the inputs and the output is the list r[ x ][ y ] with x = 0..nT − 1.

  - In-loop filter process

The specifications in subclause 8.7 apply.

  - Decoding process for coding units coded in textureRL prediction mode

    - General decoding process for coding units coded in textureRL prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

Let rlPic be the decoded picture with picture order count equal to PicOrderCnt and nuh_layer_id equal to RefLayerId[ LayerIdInVps[ currLayerId ][ 0 ] ].

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ) as input.

The variable nCSL is set equal to 1 << log2CbSize and the variable nCSC is set equal to ( 1 << log2CbSize ) >> 1.

Decoding process for coding units coded in textureRL prediction mode consists of following ordered steps:

- The textureRL prediction process as specified in subclause H.7.8.2 is invoked with the luma location ( xC, yC ), the luma coding block size log2CbSize, and the reference layer picure rlPic as the inputs and the outputs are three arrays predSamplesL, predSamplesCb, predSamplesCr.

- The decoding process for the residual signal of coding units coded in textureRL prediction mode specified in subclause G.7.5.4 is invoked with the luma location ( xC, yC ), luma coding block size log2CbSize as the inputs and the outputs are three arrays resSamplesL, resSamplesCb, resSamplesCr.

- The reconstructed samples of the current coding unit are derived as follows.

–	The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the luma coding block location ( xC, yC ), the variable nS set equal to nCSL, the variable cIdx set equal to 0, the (nCSL)x(nCSL) array predSamples set equal to predSamplesL, and the (nCSL)x(nCSL) array resSamples set equal to resSamplesL as the inputs.

–	The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the chroma coding block location ( xC/2, yC/2 ), the variable nS set equal to nCSC, the variable cIdx set equal to 1, the (nCSC)x(nCSC) array predSamples set equal to predSamplesCb, and the (nCSC)x(nCSC) array resSamples set equal to resSamplesCb as the inputs.

–	The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the chroma coding block location ( xC/2, yC/2 ), the variable nS set equal to nCSC, the variable cIdx set equal to 2, the (nCSC)x(nCSC) array predSamples set equal to predSamplesCr, and the (nCSC)x(nCSC) array resSamples set equal to resSamplesCr as the inputs.

    - TextureRL prediction process

This process is invoked when decoding coding unit whose PredMode[ xC ][ yC ] is equal to MODE_TEXTURE_RL.

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

–	a reference layer picture rlPic

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to 1 << log2CbSize and the variable nCSC is set equal to ( 1 << log2CbSize ) >> 1.

The variables nPicW and nPicH specify the width and height of the current picture in luma samples.

The variables nPicWRL and nPicHRL specify the width and height of the reference layer picture rlPic in luma samples. The variable rlPicSample specifies the sample values of rlPic of the luma and chroma components.

The array predSampleL of the prediction samples of luma component is derived by invoking the upsampled sample prediction process specified in subclause H.7.8.3 with the luma location ( xC, yC ), the width and the height of the current luma prediction block nCSL and the sample arrays rlPicSampleL given as input.

The array predSampleCb of the prediction samples of component Cb is derived by invoking the upsampled sample prediction process specified in subclause H.7.8.4 with the chroma location ( xC/2, yC/2 ), the width and the height of the current chroma prediction block nCSC, and the sample array rlPicSampleCb as well as cIdx equal to 1 given as input.

The array predSampleCr of the prediction samples of component Cr is derived by invoking the upsampled sample prediction process specified in subclause H.7.8.4 with the chroma location ( xC/2, yC/2 ), the width and the height of the current chroma prediction block nCSC, and the sample array rlPicSampleCr as well as cIdx equal to 2 given as input.

    - Resampling process of luma samples

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

–	a (nPicWRL)x(nPicHRL) array rlPicSampleL,

Output of this process is a (nPbW)x(nPbH) array predSamplesL of luma prediction samples.

The luma prediction sample predSamplesL[ x ][ y ] with ( x = 0 .. nPbW – 1, y = 0 .. nPbH – 1 ) are derived by invoking the luma sample interpolation process specified in subclause G.7.1.3.1.3 with rlPicSampleL and luma sample location ( xC + x,  yC + y ) given as inputs and predSamplesL[ x ][ y ] as output.

    - Resampling process of chroma samples

Inputs to this process are:

–	a chroma location ( xCC, yCC ) specifying the top-left sample of the current chorma coding block relative to the top left chroma sample of the current picture,

–	variables nPbWC and nPbHC specifying the width and the height of the current chroma prediction block,

–	a (nPicWRLC)x(nPicHRLC) array rlPicSampleC,

–	a variable cIdx specifying the color component index.

Outputs of this process are:

–	a (nPbWC)x(nPbHC) array predSamplesC of chroma prediction samples.

The chroma samples predSamplesC[ xC ][ yC ] with ( xC = 0..nPbWC – 1, yC = 0..nPbHC – 1 ) are derived by invoking the chroma sample interpolation process specified in subclause G.7.1.3.1.4 with rlPicSampleC, cIdx and chroma sample location (xCC + xC , yCC + yC ) given as inputs and predSamplesC[ xC ][ yC ] as output.

- Parsing process

The specifications in clause 9 and its subclauses apply with following modifications and additions.

  - CABAC parsing process for slice segment data

The specifications in subclause 9.2 and its subclauses apply with following modifications and additions.

    - Initialization process

The specifications in subclause 9.2.1 and its subclauses apply with following modifications and additions.

      - Initialization process for context variables

The specifications in subclause 9.2.1.1 apply with following modifications and additions.

–	Error: Reference source not found is appended to the end of Table 9-4.

–	Error: Reference source not found is appended to the end of this subclause.

Table H-1 Association of ctxIdx and syntax elements for each initializationType in the initialization process

|  | Syntax element | ctxIdxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit() | texture_rl_flag | Table | 0..2 | 3..5 | 6..8 |

Table H-2 Values of variable initValue for texture_rl_flag ctxIdx

| Initialization variable | texture_rl_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 197 | 197 | 185 | 197 | 197 | 185 | 185 | 185 | 201 |

    - Binarization process

The specifications in subclause 9.2.2 apply with following modifications and additions.

–	Error: Reference source not found is appended to the end of Table 9-32.

Table H-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| texture_rl_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 4 |

    - Decoding process flow

The specifications in subclause 9.2.3 apply with following modifications and additions.

      - Derivation process for ctxIdx

The specifications in subclause 9.2.3.1 apply with following modifications and additions.

–	Error: Reference source not found is appended to the end of Table 9-37.

Table H-4 Assignment of ctxIdxInc to syntax elements with context coded bins

| Syntax element | binIdx |
| --- | --- |
|  | 0 |
| texture_rl_flag | 0,1,2 (subclause H.9.3.3.1.1) |

        - .xIdxrtlsing process for slice data Derivation process for ctxIdx

The specifications in subclause 9.2.3.1.1 apply with following modifications and additions.

–	Error: Reference source not found is appended to the end of Table 9-38.

Table H-5 Specification of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| texture_rl_flag | texture_rl_flag [ xL ][ yL ] | texture_rl_flag [ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |

- Specification of bitstream subsets

The specifications in clause F.10 apply.

- Profiles and levels

TBD

- Byte stream format

The specifications in Annex B apply.

- Hypothetical reference decoder

TBD

- SEI messages

The specifications in subclause F.14 and its subclauses apply.

- Video usability information

The specifications in subclause F.15 apply.