| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0218 |
| --- | --- |

| Title: | Inter-layer slice header syntax elements prediction in SHVC and MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In many scalable and multi-view video applications, it is reported that all layers are coded using the same GOP structure and that it is common for all layer pictures in the same access unit (AU) to have the same temporal reference pictures and reference picture lists. Moreover, when the pictures at the same temporal position are coded using the identical reference pictures, their weighted prediction parameters (WPPs) are reported to be likely identical to each other.  Based on this observation, it is proposed to signal in slice header whether reference picture lists and WPP are predicted from those of scalable inter-layer reference picture or inter-view reference picture.

# Introduction

Based on the observation that all layer pictures in the same AU are coded using the same reference picture lists in most of scalable coding application, the inter-layer prediction scheme of reference picture list and WPP, which require relatively large number of bits when signaled, has been proposed in [1] for SHVC. Since the same observation also applies to multi-view coding application, this scheme is generalized in this contribution for scalable (SHVC) + multi-view (MV-HEVC) coding applications.

# Proposed method

Two sets of flags are proposed for inter-layer reference picture list prediction and inter-layer WPP prediction, respectively, as shown in Table 1.

Table 1. Proposed syntax elements for non-base layer slice header (SH)

| Slice_segment_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| if ( interDLayerPredFlag ¦¦ interVLayerFlag ) |  |
| inter_layer_rp_prediction_flag | u(1) |
| if ( inter_layer_rp_prediction_flag && interVLayerFlag && interDLayerPredFlag ) |  |
| rp_prediction_from_dlayer_flag | u(1) |
| if ( !inter_layer_rp_prediction_flag ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( I = 0; I < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( I < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ I ] | u(v) |
| } else { |  |
| poc_lsb_lt[ I ] | u(v) |
| used_by_curr_pic_lt_flag[ I ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ I ] | u(1) |
| if( delta_poc_msb_present_flag[ I ] ) |  |
| delta_poc_msb_cycle_lt[ I ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if ( !inter_layer_rp_prediction_flag ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) { |  |
| if ( interDLayerPredFlag ¦¦ interVLayerFlag ) |  |
| inter_layer_wpp_prediction_flag | u(1) |
| if ( inter_layer_rp_prediction_flag && interVLayerFlag && interDLayerPredFlag ) |  |
| wpp_prediction_from_dlayer_flag |  |
| if ( !inter_layer_wp_prediction_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| …… |  |
| } |  |
| …… |  |
| } |  |

2.1 Inter-layer reference picture list prediction

inter_layer_rp_prediction_flag equal to 1 specifies that reference picture lists are inferred from either scalable inter-layer reference picture or inter-view reference picture, and syntax elements related with reference picture set and reference picture list modification are not signaled. inter_layer_rp_prediction_flag equal to 0 specifies that syntax elements related with reference picture lists are explicitly signaled in a slice header. If not signaled, inter_layer_rp_prediction_flag is inferred to be 0.

inter_layer_rp_prediction_flag is signaled only when either interDLayerPredFlag or interVLayerPredFlag, which specifies that inter-view prediction or scalable inter-layer prediction is enabled, is equal to 1. interDLayerPredFlag or interVLayerPredFlag can be signaled by several means such as scalability masks and explicit flags, etc.

rp_prediction_from_dlayer_flag equal to 1 specifies that reference picture lists are inferred from a scalable inter-layer reference picture. rp_prediction_from_dlayer_flag equal to 0 specifies that reference picture list is inferred from an inter-view reference picture with the closest viewId to the current viewId in the same scalable layer. If not signaled, rp_prediction_from_dlayer_flag is inferred to be 1 if inter_layer_rp_prediction_flag and interDLayerPredFlag are equal to 1. Otherwise, rp_prediction_from_dlayer_flag is inferred to be 0.

2.2 Inter-layer WPP prediction

inter_layer_wpp_prediction_flag equal to 1 specifies that WPP table is inferred from either scalable inter-layer reference picture or inter-view reference picture, and WPP table is not signaled. inter_layer_wpp_prediction_flag equal to 0 specifies that WPP table is explicitly signaled in a slice header. If not signaled, inter_layer_wpp_prediction_flag is inferred to be 0.

inter_layer_wpp_prediction_flag is signaled only when either interDLayerPredFlag or interVLayerPredFlag, which specifies that inter-view prediction or scalable inter-layer prediction is enabled, is equal to 1. interDLayerPredFlag or interVLayerPredFlag can be signaled by several means such as scalability masks and explicit flags, etc.

wpp_prediction_from_dlayer_flag equal to 1 specifies that WPP table is inferred from a scalable inter-layer reference picture. wpp_prediction_from_dlayer_flag equal to 0 specifies that WPP table is inferred from an inter-view reference picture with the closest viewId to the current viewId in the same scalable layer. If not signaled, wpp_prediction_from_dlayer_flag is inferred to be 1 if inter_layer_wpp_prediction_flag and interDLayerPredFlag are equal to 1. Otherwise, wpp_prediction_from_dlayer_flag is inferred to be 0

# Conclusion

In this contribution, the inter-layer prediction scheme of reference picture list and WPP was proposed for scalable (SHVC) + multi-view (MV-HEVC) coding applications. The proposed syntax elements will be useful in reducing overhead signaling especially when multiple layers are coded into the same GOP structure and layer pictures in the same AU have the same temporal reference pictures, which is common in many applications.

# Reference

- D.-K. Kwon, M. Budagavi and M. Zhou, “Inter-layer slice header syntax element prediction in SHVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0231, Jan. 2013.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).