| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0219 |
| --- | --- |

| Title: | Reference-layer cropping offsets signaling in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-467-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The current SHVC design does not allow to select reference layer (RL) region to be re-sampled for inter-layer prediction. In this contribution, it is proposed to signal scaled reference-layer cropping offset and reference-layer cropping offsets in SPS so that any RL region to be resampled for inter-layer prediction and mapped to any EL region based on these offsets.

# Introduction

It has been proposed in [1] to use the output RL picture for inter-layer prediction instead of the decoded RL picture, along with offsets between corners of scaled reference layer and enhancement layer (i.e. scaled reference layer offsets), which are signaled in SPS as in SVC. In this contribution, it is proposed to signal reference-layer cropping offsets in EL’s SPS as well as scaled reference layer offsets, instead of using the output RL picture directly for inter-layer prediction.

# Proposed method

Along with the scaled reference layer offsets, it is proposed to signal reference-layer cropping offsets in EL’s SPS as shown in Table 1. First, scaled_ref_layer_offset_flag is proposed to singal whether scaled reference layer offsets in [1] is proposed. Second, it is proposed to signal ref_layer_offset_flag and four reference layer offsets (ref_layer_left_offset, ref_layer_right_offset, ref_layer_top_offset and ref_layer_bottom_offset) when ref_layer_offset_flag is set equal to 1.

Table 1. Proposed sequence parameter set RBSP

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ...... |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| scaled_ref_layer_offset_flag | u(1) |
| if (scaled_ref_layer_offset_flag) |  |
| { |  |
| scaled_ref_layer_left_offset | ue(v) |
| scaled_ref_layer_right_offset | ue(v) |
| scaled_ref_layer_top_offset | ue(v) |
| scaled_ref_layer_bottom_offset | ue(v) |
| } |  |
| ref_layer_offset_flag | u(1) |
| if ( ref_layer_offset_flag ) |  |
| { |  |
| ref_layer_left_offset | ue(v) |
| ref_layer_right_offset | ue(v) |
| ref_layer_top_offset | ue(v) |
| ref_layer_bottom_offset | ue(v) |
| } |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |
| } |  |

scaled_ref_layer_offset_flag equal to 1 indicates that offsets between the luma samples of a resampled layer picture used for inter-layer prediction and the luma sample of the current layer picture follow next in the SPS. scaled_ref_layer_offset_flag equal to 0 indicates that these offset are not present.

scaled_ref_layer_left_offset specifies the horizontal offset between the upper-left luma sample of a resampled layer picture used for inter-layer prediction and the upper-left luma sample of the current layer picture in units of two luma samples. When scaled_ref_layer_left_offset is not present, it is inferred to be equal to 0.

scaled_ref_layer_right_offset specifies the horizontal offset between the bottom-right luma sample of a resampled layer picture used for inter-layer prediction and the bottom-right luma sample of the current layer picture in units of two luma samples. When scaled_ref_layer_right_offset is not present, it is inferred to be equal to 0.

scaled_ref_layer_top_offset specifies the vertical offset between the upper-left luma sample of a resampled layer picture used for inter-layer prediction and the upper-left luma sample of the current layer picture in units of two luma samples. When scaled_ref_layer_top_offset is not present, it is inferred to be equal to 0.

scaled_ref_layer_bottom_offset specifies the vertical offset between the bottom-right luma sample of a resampled layer picture used for inter-layer prediction and the bottom-right luma sample of the current layer picture in units of two luma samples. When scaled_ref_layer_bottom_offset is not present, it is inferred to be equal to 0.

The variables ScaledRefLayerLeftOffset, ScaledRefLayerRightOffset, ScaledRefLayerTopOffset, ScaledRefLayerBottomOffset, nPicW, nPicH are derived by

ScaledRefLayerLeftOffset   = 2 * scaled_ref_layer_left_offset		              	        (G-xx)

ScaledRefLayerRightOffset  = 2 * scaled_ref_layer_right_offset 	                              (G-xx)

ScaledRefLayerTopOffset   = 2 * scaled_ref_layer_top_offset                                 (G-xx)

ScaledRefLayerBottomOffset  = 2 * scaled_ref_layer_bottom_offset 	                      (G-xx)

nPicW = pic_width_in_luma_samples − ScaledRefLayerLeftOffse − ScaledRefLayerRightOffset      (G-xx)

nPicH = pic_height_in_luma_samples − ScaledRefLayerTopOffset − ScaledRefLayerBottomOffset    (G-xx)

ref_layer_offset_flag equal to 1 indicates that offsets between the luma samples of a reference layer picture to be resampled for inter-layer prediction and the luma sample of the decoded reference layer picture follow next in the SPS. ref_layer_offset_flag  equal to 0 indicates that these offsets are not present

ref_layer_left_offset specifies the horizontal offset between the upper-left luma sample of a reference layer picture to be resampled for inter-layer prediction and the upper-left luma sample of the reference layer picture in units of two luma samples. When ref_layer_left_offset is not present, it is inferred to be equal to 0.

ref_layer_right_offset specifies the horizontal offset between the bottom-right luma sample of a reference layer picture to be resampled for inter-layer prediction and the bottom-right luma sample of the reference layer picture in units of two luma samples. When ref_layer_right_offset is not present, it is inferred to be equal to 0.

ref_layer_top_offset specifies the vertical offset between the upper-left luma sample of a reference layer picture to be resampled for for inter-layer prediction and the upper-left luma sample of the reference layer picture in units of two luma samples. When ref_layer_top_offset is not present, it is inferred to be equal to 0.

ref_layer_bottom_offset specifies the vertical offset between the bottom-right luma sample of a reference layer picture to be resampled for inter-layer prediction and the bottom-right luma sample of the reference layer picture in units of two luma samples. When ref_layer_bottom_offset is not present, it is inferred to be equal to 0.

The variables RefLayerLeftOffset, RefLayerRightOffset, RefLayerTopOffset, RefLayerBottomOffset, nPicWRL, nPicHRL are derived by

RefLayerLeftOffset   = 2 * ref_layer_left_offset						(G-xx)

RefLayerRightOffset  = 2 * ref_layer_right_offset           				       (G-xx)

RefLayerTopOffset   = 2 * ref_layer_top_offset 			                   	(G-xx)

RefLayerBottomOffset  = 2 * ref_layer_bottom_offset 		     	                      (G-xx)

nPicW= ref_pic_width_in_luma_samples − RefLayerLeftOffset – RefLayerRightOffset      	(G-xx)

nPicH = ref_pic_height_in_luma_samples − RefLayerTopOffset − RefLayerBottomOffset   	       (G-xx)

One benefit of the reference layer offsets is that we can fully exploit the inter-layer prediction even when the output BL picture is smaller than the actual decoded BL picture. Another benefit is that ROI scalability can be realized without constraints. By signaling different values of reference layer offsets, any region of BL picture can be scaled to enhancement layer. Two examples are described in Figure 1 and Figure 2. In Figure 1, any ROI region in base layer is scaled and mapped to a whole EL picture by using non-zero reference layer offsets with scaled reference layer offsets being equal to 0. In Figure 2, any ROI region in base layer is scaled and mapped to any area of EL by using non-zero reference layer offsets and non-zero scaled reference layer offsets.

Figure 1. ROI scalability with the proposed reference layer offsets

Figure 2. ROI scalability with both reference layer offsets and scaled ref layer offsets

# Conclusion

In this contribution, it is proposed to signal reference layer cropping offset and scaled reference layer cropping offset in EL’s SPS so that the encoder can choose any RL picture region to be resampled for inter-layer prediction for ROI scalability.

# Test Model text changes

H.7.3.2.2 Sequence parameter set RBSP

See Section 1.

H.7.4.2.2 Sequence parameter set RBSP semantics

See Section 1.

G.8.1.3.1.5 Derivation process for reference layer sample locations

The variable xRef16 is derived as follow:

xRef16 = ( (xP – SclaedRefLayerLeftOffset) * nPicWRL * 16 + nPicW / 2) / nPicW

+ RefLayerLeftOffset  										(G-xx)

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = ((yP – ScaledRefLayerTopOffset) * nPicHRL * 16 + nPicH / 2 ) / nPicH

+ RefLayerTopOffset						 				(G-xx)

–	Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( (yP – ScaledRefLayerTopOffset) * nPicHRL * 16 + nPicH / 2 ) / nPicH

+ RefLayerTopOffset – offset								(G-xx)

where the value of offset is derived as specified by the following:

H.8.5.3.1.12 Derivation process for the collocated position of the current picture in a reference layer picture

Inputs of this process are:

– a luma location ( xC, yC ) relative to the top-left luma sample of the current picture,

– variables specifying the width and the height of the current picture in luma samples, nPicW and nPicH,

– 	variables specifying the width and the height of a reference layer picture in luma samples, nPicWRL and nPicHRL,

Ourput of this process is:

– a luma location ( xRL, yRL ) relative to the top-left luma sample of the reference layer picture.

The variable nPicW and nPicH specify the width and the height of the current picture.

The variables xRL and yRL are derived as follows:

xRL = ( (xC – ScaledRefLayerLeftOffset)* nPicWRL + nPicW / 2) / nPicW

+ RefLayerLeftOffset						                                        (H-xx)

yRL = ( (yC – ScaledRefLayerTopOffset) * nPicHRL + nPicH / 2 ) / nPicH

+ RefLayerTopOffset		                                                    (H-xx)

# Reference

- J. Boyce, D. Hong and W. Jang, “Legacy base layer codec support in HEVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0178, Jan. 2013.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).