| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0220 |
| --- | --- |

| Title: | Non-SCE3: Uni-directional combined prediction (UCP) in Inter slice |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-479-1797 d-kwon@ti.com +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The combined prediction (CP) tool calculates the prediction in the enhancement layer (EL) by averaging EL inter prediction and collocated up-sampled base-layer (BL) prediction. It reportedly provides additional coding gains for EL at the cost of increased memory bandwidth and computations. CP has increased computations since it combines up to three samples (one each from List 0, List 1, and up-sampled BL pictures) where as bi-prediction combines up to two samples (one each from List 0 and List 1 pictures) per EL pixel. Also, the worst-case memory bandwidth for a CP PU is reported to be larger than that of bi-predicted HEVC PU. In this contribution, uni-directional CP (UCP) is proposed to reportedly keep the worst-case memory bandwidth and computations unchanged for a PU. UCP enables combined prediction for only uni-predicted PUs larger than or equal to 8x8 and disables it for bi-predicted PUs thereby eliminating the additional computations and memory bandwidth required in CP. When compared to the SHM-1.0 IntraBL anchor, experimental results with common test condition reportedly show that the proposed UCP tool results in luma BL+EL BD-rate gain of 0.2%, 0.3%, 0.3% for RA 2x, RA 1.5x, RA SNR, and 1.1%, 1.3%, 2.5% for LD-P 2x, LD-P 1.5x, LD-P SNR, and 0.2%, 0.2%, 0.4% for LD-B 2x, LD-B 1.5x, LD-B SNR.

In this contribution, the UCP tool is also combined with PU-level IntraBL approach and compared with SHM-1.0 RefIdx anchor for RA and LD-B cases. Experiment results reportedly show that the UCP+PU-level IntraBL combination provides 1.0% better average Luma BD-rate for RA and LD-B cases when compared to RefIdx approach at roughly the same worst-case memory bandwidth and computations.

# Introduction

Combined prediction (CP) has been proposed for inter-coded block in enhancement layer in [1] and [2]. A combined prediction flag is signaled at CU level and, when it is enabled, new predictions for all PUs are generated by averaging inter prediction and collocated up-sampled base-layer (BL) prediction. Combined prediction increases memory bandwidth and computations for additional coding gain. Due to additional bandwidth to fetch collocated BL prediction, the worst-case memory bandwidth for a PU becomes larger than that of bi-predicted PU.

In this contribution, it is proposed to bypass CP for bi-predicted PUs and any PUs smaller than 8x8 so that the worst-case memory bandwidth remains the same as in HEVC. The proposed method is implemented top of [1]. While CP is applied only to luma component in [1], it is applied to all color components in the proposed method. However, the proposed method can be applied directly to [1] as well.

It should be noted that the proposed uni-directional CP has the same worst-case memory bandwidth as RefIdx approach, in which CP is enabled implicitly in B slice. Therefore, IntraBL approach with the proposed uni-directional CP and RefIdx approach can be fairly compared in terms of memory bandwidth. In addition, the proposed method is also combined with [3], which has PU-level inter-layer texture prediction. By doing so, since inter-layer texture prediction is done at PU level in RefIdx approach, the comparison between two different approaches becomes fairer.

# Proposed method

In the proposed method, combined prediction for inter-coded block is signaled at CU level as shown in Table 1. In addition, motion compensation process is modified slightly.

For each PU, let’s denote List0 and List1 inter prediction as PEL,0 and PEL,1, respectively, and denote collocated up-sampled BL prediction as PBL. When combined_pred_flag for a CU is set to 1, for all PUs in the CU, final prediction PEL is generated as follows:

1) If a PU is uni-predicted and larger than or equal to 8x8, i.e. has either PEL,0 or PEL,1,

PEL = (PEL,0 + PBL + 1) / 2 or PEL = (PEL,1 + PBL + 1) / 2.

2) If a PU is bi-predicted, i.e. has both PEL,0 and PEL,1,

PEL is derived from PEL,0 and PEL,1 without any change i.e. PEL = (PEL,0 + PEL,1 + 1) / 2

The above process is applied to all color components.

Table 1. Proposed coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if( nuh_layer_id > 0) |  |
| combined_pred_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| … |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| if( nuh_layer_id > 0 && (PartMode = = PART_2Nx2N ¦¦ nCbS > 8) ) |  |
| combined_pred_flag[ x0 ][ y0 ] |  |
| } |  |
| } |  |
| … |  |
| } |  |
| } |  |

# Experimental results

# 1) Uni-directional CP vs. SHM-1.0 IntraBL anchor

# The proposed uni-directional CP is compared with SHM-1.0 IntraBL anchor under the common test condition [4]. The proposed method resulted in luma BL+EL BD-rate gain of 0.2%, 0.3%, 0.3% for RA 2x, RA 1.5x, RA SNR, and 1.1%, 1.3%, 2.5% for LD-P 2x, LD-P 1.5x, LD-P SNR, and 0.2%, 0.2%, 0.4% for LD-B 2x, LD-B 1.5x, LD-B SNR. It is summarized in Table 2. Note that the uni-directional CP provides the same gain as the original (bi-directional) CP for LD-P case. Coding gains for RA and LD-B cases show improvement by uni-directional combined prediction at the same worst-case memory bandwidth as the anchor.

Table 2. BD-rate result of the uni-directional CP compared to SHM-1.0 IntraBL anchor.

Table 3 shows the complexity analysis results of the uni-directional CP method. For RA and LD-B cases, actual memory bandwidth increased by only 1 ~ 2 % when compared to SHM-1.0 IntraBL anchor.

All the BD-rate results and complexity analysis by the uni-directional CP and original (bi-directional) CP can be found in the attached excel spreadsheets.

Table 3. Complexity analysis of the uni-directional CP compared to SHM-1.0 IntraBL anchor.

# 2) Uni-directional CP + SCE2 [3] vs. SHM-1.0 RefIdx anchor

In this test, PU-level inter-layer texture prediction in [3] is enabled along with the proposed uni-directional CP and its performance is compared with SHM-1.0 RefIdx anchor. Since combined prediction and PU-level inter-layer texture prediction are enabled in SHM-1.0 RefIdx anchor, this test for RA and LD-B case will disclose performance difference between IntraBL approach and RefIdx approach at the same worst-case memory bandwidth and inter-layer texture prediction.

Table 4 summarizes the BD-rate results for RA and LD-B when compared to SHM-1.0 RefIdx anchor. LD-P results are excluded in this table since UCP ends up becoming a bi-predictive approach for LDP where as RefIdx uses uni-prediction for LDP. On average, UCP+PU-level IntraBL [3] provides 1.0 % more luma BD-rate.

Table 4. BD-rate result of (uni-directional CP + [3]) compare to SHM-1.0 RefIdx anchor.

Finally, Table 5 shows the complexity analysis results when compared to SHM-1.0 RefIdx PU-based upsampling anchor. Memory bandwidth and complexity of them are almost the same for RA and LD-B cases.

Table 5. Complexity analysis of the uni-directional CP compared to SHM-1.0 RefIdx PU-based up-sampling anchor

# Conclusion

In this contribution, uni-directional combined prediction is proposed by disabling combined prediction for bi-predicted PUs and PUs smaller than 8x8. When compared to SHM-1.0 IntraBL anchor, it was shown that this method improves luma BD-rate by 0.2% ~ 0.4% for RA and LD-B cases at the same worst-case memory bandwidth as the anchor. When uni-directional combined prediction and PU-level inter-layer texture prediction are used, it was shown by experiments that average luma BD-rate for RA and LD-B cases improves by 1.0 % compared to SHM-1.0 RefIdx anchor at roughly the same worst-case memory bandwidth and inter-layer texture prediction.

# Reference

- V. Seregin, J. Chen and M. Karczewicz, “TE3: Combined inter mode (test 4.6.2.2),” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0206, Jan. 2013.

- T.-D Chuang, Y.-W. Huang, P. Lai, S. Liu and S. Lei, “TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0072, Jan. 2013

- D.-K. Kwon, M. Budagavi and M. Zhou, “Non-TE2: Improved inter-layer texture prediction signaling,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0230, Jan. 2013.

- X. Li, J. Boyce, P. Onno and Y. Ye, “Common test condition and software reference configurations for the scalable test model,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1009, Jan. 2013

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).