| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0222_r1 |
| --- | --- |

| Title: | Non-SCE3.4: Simplified Generalized Combined Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA Tzu-Der Chuang, Yuwen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | polin.lai@mediatek.com shan.liu@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents technical descriptions and test results of simplification methods applied on top of SCE3.4 Generalized Combined Prediction (GCP) in JCTVC-M0221. In the first test set, GCP size limitations are imposed such that the minimum PU width and height for GCP uni-prediction is 8, and for GCP bi-prediction is 16. Furthermore, chroma components do not perform GCP (luma-only GCP). In the second test set, besides the GCP size limitations and luma-only GCP, the motion compensation interpolation filters in luma-only GCP is changed to bilinear filters. The test results are as below.

Test set 1: GCP size limitations, luma-only GCP

EL+BL BD-rates of 2X / 1.5X / SNR:

3 GCP modes: RA -1.6% / -2.2% / -1.7%, LDP -3.6% / -4.8% / -4.2%, LDB -3.2% / -4.4% / -3.1%,

2 GCP modes: RA -1.3% / -1.9% / -1.6%, LDP -3.5% / -4.6% / -4.3%, LDB -2.8% / -3.8% / -2.9%. The average encoding runtime are 126.4% and 118.7% for 3 and 2 GCP modes respectively, decoding runtime is about 104%. The worst-case complexity (GCP on the minimum block-size) is about 120%~160% as compared to SHM1.0, in terms of memory bandwidth.

Test set 2: GCP size limitations, luma-only GCP, GCP with bilinear MC

EL+BL BD-rates of 2X / 1.5X / SNR:

3 GCP modes: RA -1.4% / -1.7% / -1.7%, LDP -3.8% / -4.1% / -4.9%, LDB -2.5% / -3.0% / -2.7%. The average encoding runtime are 126.6%, decoding runtime is about 102%. The worst-case complexity, in terms of computation and memory bandwidth, is about 112% and 80% as compared to SHM1.0.

# Introduction

In JCTVC-M0221 [1], techniques and test results of SCE3.4 Generalized Combined Prediction were presented. In GCP, the Inter-CU predictor EL_pred is modified as the following (unidirectional case):

EL_pred = (1-a) EL_ref + a BL_col + b (BL_col – BL_ref)

The following three GCP modes are utilized in SCE3.4. Each Inter-CU can select one of the GCP modes or simply using normal Inter mode (GCP mode index = 0). The corresponding {a,b} weightings and the mode indices are summarized as below:

Table 1 GCP modes utilized in SCE 3.4

| GCP mode idx | {a,b} | Inter-CU predictor | Note |
| --- | --- | --- | --- |
| 0 | {0,0} | EL_pred = EL_ref | Normal Inter |
| 1 | {0, 0.5} | EL_pred = EL_ref + 0.5 (BL_col – BL_ref) |  |
| 2 | {0.5, 0.5} | EL_pred = 0.5 EL_ref + 0.5 BL_col + 0.5 (BL_col – BL_ref) | Inter-DIFF half |
| 3 | {0, 1} | EL_pred = EL_ref + (BL_col – BL_ref) | Inter-DIFF |

The upsampling and motion compensation operations are analyzed below using the methodology presented in the BoG report of SHVC complexity evaluation JCTVC-L0440 [2].

- GCP mode 1: {a,b} = {0,0.5}

EL_pred = EL_ref + 0.5 (BL_col – BL_ref)

= 0.5 BL_col + (EL_ref - 0.5 BL_ref)

Complexity operations:

EL_pred = 0.5 UP1 {BL_col} + MC1 [(EL_ref - 0.5 UP2 {BL_ref}), EL_motion]

- GCP mode 2: {a,b} = {0.5,0.5}

EL_pred = 0.5 EL_ref + 0.5  BL_col + 0.5  (BL_col – BL_ref)

= BL_col  + 0.5 (EL_ref - BL_ref)

Complexity operations:

EL_pred = UP1 {BL_col} +0.5 MC1 [(EL_ref - UP2 {BL_ref}), EL_motion]

- GCP mode 3: {a,b} = {0,1}

EL_pred = EL_ref + (BL_col – BL_ref)

= BL_col  + (EL_ref - BL_ref)

Complexity operations:

EL_pred = UP1 {BL_col} + MC1 [(EL_ref - UP2 {BL_ref}), EL_motion]

Note that in the above GCP operations, motion compensations for EL_ref and BL_ref are combined. To ensure intermediate results follow dynamic range constraints, clipping process is conducted. The input range for interpolation filter is [0,255] for GCP mode 1 and is [-128,127] for GCP mode 2 and 3 [3][4].

The operations of GCP can be summarized as Fig. 2.

Fig. 1 GCP operations of upsampling and motion compensation

# Simplification of GCP

It was reported that the worst-case complexity of GCP is quite high due to additional upsampling process and additional memory bandwidth required for BL_col and BL_ref [1]. In this contribution, the following three simplification methods are utilized to reduce the complexity of GCP.

- GCP size limitations: Since the worst-case complexity occurs when GCP is applied on the smallest allowed PU sizes, the first simplification is to constrain the minimum PU size for which GCP can be applied. In this contribution, the GCP size limitations are imposed as the following:

  - For GCP uni-pred: Both PU width and PU height have to be larger than or equal to 8.

  - For GCP bi-pred: Both PU width and PU height have to be larger than or equal to 16.

- Luma-only GCP: When an Inter-CU utilizes GCP, the chroma components only perform normal Inter prediction. This way no additional upsampling and memory bandwidth are needed for the corresponding BL chroma component.

- Bilinear MC interpolation filter: For the combined motion compensation in GCP, as depicted in Fig. 1, bilinear MC interpolation filters are used for the luma-only GCP. Since in this contribution chroma does not utilize GCP (just normal Inter), 4-tap chroma interpolation filter as in SHM1.0 is used.

In this contribution, two test sets were provided. In the first set, methods a) and b) are used. In test set 2, a), b), and c) are all utilized to reduce complexity.

# Complexity Assessment

As illustrated in Fig. 1, the following additional complexity components are required:

- Additional memory bandwidth for BL_col.

- Additional upsampling computation for BL_col.

- Subtraction operation for (EL_ref - 0.5 UP2 {BL_ref}), which is the input for MC interpolation.

For BL_ref, UP and MC filtering processes are cascaded. Thus for a PU size W×H in EL resolution, MC interpolation needs (W+tap_itpF-1) × (H+tap_itpF-1) pixels. Here in the second test set where the Luma interpolation filters are bilinear filters, the value tap_itpF is simply 2.

- Bandwidth: {ceil [(W+2-1)/scale] + tap_upF-1 } × {ceil [(H+2-1)/ scale] + tap_upF-1 }.

- Upsampling computation needed for: (W+2-1) × (H+2-1) pixels.

Since in this contribution chroma does not utilize GCP (just normal Inter), 4-tap interpolation filter as in SHM1.0 is used. The additional bilinear filters require 2-tap × 4-phase (luma) lookup table.

# Test results

## Test set 1, a) GCP size limitations + b) No GCP process for chroma

The worst case complexity when applying simplification a) and b) are as below:

### 3 GCP modes

In this test, 3 GCP modes described in Section 1 are used. Qualcomm has cross-checked this test.

- BD-rate results:

### 2 GCP modes (supplementary, requested by JCTVC-L0440 [2])

In this test, only 2 GCP modes are tested, GCP mode 3 not used. XXX has cross-checked this test.

- BD-rate results:

## Test set 2, a) GCP size limitations + b) No GCP process for chroma + 
c) Bilinear MC interpolation filter for GCP

- Worst-case complexity, when applying simplification a) b) and c) are as below:

### 3 GCP modes

- BD-rate results, 3 GCP modes (LG has cross-checked this test):

# Proposed syntax and semantics

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| if( LayerID > 0 ) |  |
| gcp_mode_idx | ae(v) |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| if( LayerID > 0 ) |  |
| gcp_mode_idx | ae(v) |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

gcp_mode_idx specifies mode of generalized combined prediction (GCP) for a INTER coding unit. The value of gcp_mode_idx shall be in the range of 0 to 2, inclusive. When not present, gcp_mode_idx is inferred to be equal to 0.

# Conclusions

This contribution presents technical descriptions and test results of simplification methods applied on top of SCE3.4 Generalized Combined Prediction (GCP) in JCTVC-M0221.

- Test set 1: GCP size limitations, luma-only GCP

EL+BL BD-rates of 2X / 1.5X / SNR:

3 GCP modes: RA -1.6% / -2.2% / -1.7%, LDP -3.6% / -4.8% / -4.2%, LDB -3.2% / -4.4% / -3.1%,

2 GCP modes: RA -1.3% / -1.9% / -1.6%, LDP -3.5% / -4.6% / -4.3%, LDB -2.8% / -3.8% / -2.9%. The average encoding runtime are 127.4% and 118.7% for 3 and 2 GCP modes respectively, decoding runtime is about 104%. The worst-case complexity (GCP on the minimum block-size) is about 120%~160% as compared to SHM1.0, in terms of memory bandwidth.

- Test set 2: GCP size limitations, luma-only GCP, GCP with bilinear MC

EL+BL BD-rates of 2X / 1.5X / SNR:

3 GCP modes: RA -1.5% / -1.7% / -1.7%, LDP -3.8% / -4.1% / -4.9%, LDB -2.5% / -3.0% / -2.7%. The average encoding runtime are 126.6%, decoding runtime is about 102%. The worst-case complexity (GCP on the minimum block-size) is about 112% and 80% as compared to SHM1.0, in terms of computation and memory bandwidth.

# References

- “SCE3.4: Generalized Combined Prediction”, JCTCV-M0221, Incheon, KR, Apr. 2013

- “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTCV-L0440, Geneva, CH, Jan. 2013

- “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTCV-L0078, Geneva, CH, Jan. 2013

- “TE3-4.4.1: Inter prediction based on difference picture”, JCTCV-L0136, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).