| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0223_r1 |
| --- | --- |

| Title: | Non-SCE4.1: Fixed Inter-layer Filter for SNR Scalability with Only One Non-unity Coefficient |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ximin Zhang, PoLin Lai, Shan Liu, and Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Email: | polin.lai@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a fixed inter-layer filter for SNR scalability, in which there is only one non-unity coefficients. The filter footprint is 3×3 square, with coefficient 8 applied to the to-be-filtered pixel and unity coefficients (=1) applied to its eight immediate neighbors. In SHM1.0 IntraBL framework, the filtering process is turned on/off on a CU-by-CU basis using CU-level filter control flag.

The test results are, for EL+BL luma BD-rates, RA SNR -0.8%, LDP SNR -2.1%, and LDB SNR -0.8%. The average encoding and decoding runtime are 102% and 102%. The proposed method has no impact on the worst-case complexity. The average complexity only increases add-operations to 106%.

# Introduction

Fixed inter-layer filters for SNR scalability were proposed in the 12th JCTVC meeting in Geneva [1][2]. In JCTVC-0087[1], a 7-tap 2D separable filter is used, which contains non-dyadic coefficients. In JCTVC-0290 [2], a 2D non-separable filter with 5×5 diamond-shaped footprint (13 pixels) is used, in which all the coefficients are dyadic. In this contribution, a filter with 3×3 square footprint (9 pixels) is proposed, in which the center coefficient is 8 and all the other coefficients are just 1.

# Proposed filter with only one non-unity coefficient

The proposed filter with 3×3 square footprint is depicted as below, which is denote as 3×3-{8,1,1} filter:

As can be seen, the “3×3-{8,1,1} filter” contains only one non-unity coefficient: the center coefficient 8 that is applied to the to-be-filtered pixel.

# Complexity assessment

With the proposed inter-layer filter, additional complexity components in IntraBL CU are:

- Only one multiplication is required to filter each pixel, since all the other coefficients are unity.

- 9 additions are required to filter each pixel. This includes adding the offset for rounding purposes.

- Bandwidth: For a CU, 3×3 filter requires one additional line on each side of the CU boundary.

# Test results

Intel has cross-checked this test (JCTVC-M0371).

- BD-rate results and average complexity:

- Worst-case complexity:

The proposed inter-layer filter is only applied to IntraBL CU. According to the complexity assessment Excel sheet provided in JCTVC-L0440 [3], the proposed inter-layer filter does not affect the worst-case complexity, as the worst-case remains to be the Inter bi-directional prediction.

# Proposed syntax and semantics

The syntax changes are provided below using red text with yellow highlight.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) { |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| SNR_filter_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| ... |  |
| ... |  |
| ... |  |

The initialization process, binarization process, the derivation process of the context of SNR_filter_flag is the same as those processes for the context.

# Conclusions

A fixed inter-layer filter, with 3×3 square footprint and only one non-unity coefficient (8), is presented for SNR scalability in this contribution. In SHM1.0 IntraBL framework, the filtering process is turned on/off on a CU-by-CU basis using CU-level filter control flag. The test results demonstrate luma BD-rates of RA SNR -0.8%, LDP SNR -2.1%, and LDB SNR -0.8%. The average encoding and decoding runtime are 102% and 102%. The average complexity only increases add-operations to 106%.

# References

- “On integer samples filtering during up-sampling”, JCTCV-L0087, Geneva, CH, Jan. 2013

- “Non-TE B4: Inter-layer fixed refining filter”,  JCTCV-L0290, Geneva, CH, Jan. 2013

- “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTCV-L0440, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).