| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0224_r1 |
| --- | --- |

| Title: | Non-SCE4.2: Inter-layer Fixed Directional Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, and Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Email: | polin.lai@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents inter-layer filtering method using fixed directional filters. The fixed directional filters use only 5 pixels within the 3×3 neighborhood of the to-be-filtered pixel. To determine which fixed directional filter to be applied to a given pixel, the local orientation is computed by computing and comparing local gradients. In SHM1.0 IntraBL framework, the filtering process is turned on/off on a CU-by-CU basis using CU-level filter control flag. The test results are as follows:

EL+BL luma BD-rates for 2X, AI / RA /LDP / LDB: -0.5% / -0.5% / -0.7% / -0.5%

EL+BL luma BD-rates for 1.5X, AI / RA /LDP / LDB: -0.1% / 0.0% / -0.4% / -0.1%

EL+BL luma BD-rates for SNR scalability, RA /LDP / LDB: -0.8% / -1.9% / -0.9%

The average encoding and decoding runtime are 104% and 106%. The proposed method has no impact on the worst-case complexity. Per-pixel complexity for the filtering process is analysis in this document,

# Introduction

Inter-layer filtering using bilateral-like approaches were proposed in the 12th JCTVC meeting in Geneva [1][2], in which the filter coefficients are determined on-the-fly for each pixel. In JCTVC-0195[1], the filter footprint is 3×3, in which the filter coefficients are determined by quantizing the intensity difference between the to-be-filtered pixel and its neighboring pixels. Several multiplications, plus one division, are needed for filtering one pixel, since the coefficients are not pre-determined. In JCTVC-0252 [2], the complexity is even much higher as the coefficients are determined by taking the pixel intensity difference into a Gaussian function.

In this contribution, fixed directional filters for inter-layer filtering are proposed. The fixed directional filters are predetermined, with dyadic coefficients applied to only5 pixels within the 3×3 neighborhood of the to-be-filtered pixel. To select a filter from the fixed directional filters, local orientation is computed by computing and comparing local gradients.

# Proposed fixed directional filters

To achieve directional filtering capabilities and avoid filtering across edges, the filtering patterns are designed to contain different weight magnitude in different directions. Four directional filtering patterns used in this proposal are depicted in Fig. 1 below:

Fig.1 Directional filtering patterns

In Fig. 1, c is the coefficient (filter weight) applied to the current to-be-filtered pixel, while a and b are the coefficients applied to the corresponding neighboring pixels as the depicted locations. For example with weights {a,b,c} = {1,2,10}, the left-most filter applies larger weight 2 to the pixels on left and right sides of the to-be-filtered pixel, as compared to smaller weight 1 to the pixels above and below. Similarly, the second filter applies larger weight 2 to the pixels above and below the to-be-filtered pixel, as compared to smaller weight 1 to the pixels on left and right sides. Since these directional filters are fixed, they can be hard-coded in the implementation.

To properly apply the filters, the filter selection should be conducted adaptively according to the signal local characteristics. For example, to better preserve edges, the directional filtering pattern is selected to apply larger coefficients to the pixels along the edges (or align with the local orientation). For example, Fig. 2 below illustrates examples using the filter patterns in Fig. 1, in which the larger filter coefficients are applied along with the edges (along with the local orientation). The solid lines without arrow heads indicate edges direction, and the lines with arrow heads on both ends indicated directions with larger filter coefficients (coefficient b).

Fig.2 Examples of filtering pattern selection according to 
edge directions (or according to local orientation)

# Selecting the fixed directional filters

The fixed directional filtering patterns are selected on a pixel-by-pixel basis, thus the filter determining process at encoder is low-latency.

To determine the local edge / orientation, such that the corresponding filter pattern can be selected, computing and comparing local directional features is conducted, which is summarized below:

- For a given pixel P(x,y), compute gradients GH and GV in using neighboring pixels:

GH (x,y) = GraH * P(x,y)

GV (x,y) = GraV * P(x,y)

The gradient operator for GraH and GraV in the contribution is the Prewitt operator, which applies the following weights to the neighboring pixels:

- Compare the strength and orientation of GH and GV to determine the directional class of the current pixels. The comparison method is as follows:

Fig. 3 Determine the direction of local orientation

For pixels in Class 1, their vertical gradients GV are much stronger than their horizontal gradients GH, i.e., the local characteristic is smoother in horizontal direction. Thus the fixed directional filter with larger coefficients on left and right pixels will be used.

Similarly for pixels in Class 2, the local characteristic is smoother in vertical direction. Thus the fixed directional filter with larger coefficients on above and below pixels will be used. The illustration in Fig. 3 demonstrates the Class distribution according to the strength and orientation of GH and GV.

- Besides the filter selection according to local orientations, another attribute for selecting filter is the strength of local variations. The rationale is that, for pixels exhibiting stronger local variations, conservative filtering patterns is used; while aggressive filtering patterns only applied to pixels in smoother areas (smaller local variations). Thus, the complete filter selection method in this contribution is depicted in Fig. 4.

With Th1 < Th2, only pixels with their gradient-sums smaller than Th1 use aggressive filter patterns, which have center coefficients 2. For pixels with gradient-sum within [Th1, Th2], conservative filtering pattern with center weight 10 are used. Finally for pixels with large gradient-sum > Th2 (stronger local variations), no filtering process is applied.

In this contribution, Th1 and Th2 are 32 and 96 for spatial scalability, and 32 and max (no Th2) for SNR scalability. The comparison in Fig. 4 is implemented using quantization.

# Complexity assessment

With the proposed inter-layer filter, additional complexity components in IntraBL CU are:

Filtering:

- 3 multiplications are required to filter each pixel.

- 5 additions are required to filter each pixel. This includes adding the offset for rounding purposes.

- Bandwidth: For a CU, 3×3 filter requires one additional line on each side of the CU boundary.

Determine orientation:

- 8 additions to compute gradients GH and GV. 1 addition to compute gradient sum.

- 2 Comparisons between GH and GV in Fig. 3

- The comparison in Fig. 4 is implemented using quantization.

# Test results

LG has cross-checked this test in JCTVC-M0423.

- BD-rate results:

- Worst-case complexity:

The proposed inter-layer filter is only applied to IntraBL CU. According to the complexity assessment Excel sheet provided in JCTVC-L0440 [3], the proposed inter-layer filter does not affect the worst-case complexity, as the worst-case remains to be the Inter bi-directional prediction.

# Proposed syntax and semantics

The syntax changes are provided below using red text with yellow highlight.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) { |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if(texture_rl_flag[ x0 ][ y0 ]) |  |
| Fixed_directional_filter_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| ... |  |
| ... |  |
| ... |  |

The initialization process, binarization process, the derivation process of the context of Fixed_directional_filter_flag is the same as those processes for the context texture_rl_flag[ x0 ][ y0 ].

# Conclusions

Fixed directional filters are proposed for inter-layer filtering. The fixed directional filters use only 5 pixels within the 3×3 neighborhood of the to-be-filtered pixel. To select a fixed directional filter to be applied to a given pixel, the local orientation is computed by computing and comparing local gradients. In SHM1.0 IntraBL framework, the filtering process is turned on/off on a CU-by-CU basis using CU-level filter control flag. The test results are as follows:

- EL+BL luma BD-rates for 2X, AI / RA /LDP / LDB: -0.5% / -0.5% / -0.7% / -0.5%

- EL+BL luma BD-rates for 1.5X, AI / RA /LDP / LDB: -0.1% / 0.0% / -0.4% / -0.1%

- EL+BL luma BD-rates for SNR scalability, RA /LDP / LDB: -0.8% / -1.9% / -0.9%

The average encoding and decoding runtime are 104% and 106%. Per-pixel complexity for the filtering process is analysis in this document,

# References

- “Non-TE4: Switchable De-ringing Filter for Inter-layer Prediction”, JCTCV-L0195, Geneva, CH, Jan. 2013

- “Non-TE4: Adaptive up-sampling of base layer picture using bilateral filters”,  JCTCV-L0252, Geneva, CH, Jan. 2013

- “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTCV-L0440, Geneva, CH, Jan. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).